| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0196-v1 |
| --- | --- |

| Title: | AHG12: Range-Exact signaling for adaptive luma clipping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll, Jung-Kyung Lee | Tel: Email: | duizcoll@ofinno.com, jlee@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

In ECM-14, an adaptive clipping is applied to the luma reconstructed samples after most filtering, prediction, and reconstruction stages. The encoder achieves lower and upper luma values from the original picture or the MCTF pre-filtered picture, and these clipping bounds are signaled to the decoder in the picture header. In order to reduce the signaling overhead, these bounds are computed as the difference between the original picture’s lower and upper luma value and a predetermined reference range. The reference range for the non-intra slice is set to the max/min luma values of the collocated picture, and for the intra-slice, it is set to the narrow video range for the luma component.

This contribution proposes a new Range-Exact flag indicating where the original picture luma bounds match the predetermined reference range; if so, the max/min delta parameters are not signaled. In addition, it is proposed that the reference range be set according to the vui_full_range_flag in the VUI if it is present. Tests performance results of proposed methods, implemented on top of ECM-14.0, are as follows {for Y, U, V, EncT, DecT}:

AI: Overall {-0.xx%, -0.xx%, -0.xx%, 1xx.x%, 1xx.x%}.

RA: Overall {-0.xx%, -0.xx%, -0.xx%, 1xx.x%, 1xx.x%}.

LDB: Overall {-0.xx%, -0.xx%, -0.xx%, 1xx.x%, 1xx.x%}.

# Introduction

In ECM-14, with the aim to achieve the highest fidelity of the reconstructed picture, an adaptive clipping [1] is applied to the luma reconstructed samples, keeping the reconstructed dynamic range in the same dynamic range as the original picture. The clipping is applied after most filtering, prediction, and reconstruction operations. The encoder calculates the lower and upper luma values from the original picture or the MCTF pre-filtered picture if it is applied. The encoder signals the clipping range to the decoder as max/min delta values between the original picture luma bounds and a predetermined reference range, which can be derived on the decoder side.

The min/max luma delta values (targetMin, targetMax) are signaled in the picture header. The reference range for the non-Intra slice types is set to the max/min luma values of the collocated picture, and it is set to the luma narrow video range for the luma component [2] for the Intra slice:

targetMin=16*1≪bitdepth-8

targetMax=(235*1≪(bitdepth-8))

# Proposal

In ECM-14, the reference range used to derive the decoder's max/min delta values is fixed to the luma narrow video range, independently of the accurate video range of the video sequence. Adjusting the reference range to the real sequence range allows achieve lower max/min delta values, reducing the signaling overhead.

For I slices, this contribution proposes to adjust the reference range according to the vui_full_range_flag in the VUI. If vui_full_range_flag  is absent or signaled and set to 0, the encoder and decoder infer the reference range as a luma narrow video range. Otherwise, if the vui_full_range_flag  is signaled and set to 1, the reference range is adjusted to the full video range:

targetMin=0

targetMax=( ( 1 << BitDepthY ) - 1 )

It can be observed that the luma video range of a picture usually matches the respective sequence video range. For instance, the luma range of all pictures in classes C and D and some of the class F match the narrow video range. Moreover, some sequences in classes F and B have all pictures that match the full video range.

This contribution proposes a new Range-Exact flag to indicate where the original picture's lower and upper luma values match the predetermined reference range according to the slice type. If the picture Range-Exact flag is set to 1, the max/min delta parameters are not signaled; otherwise, the luma delta values are signaled as regular luma adaptive clipping.

# Experimental Results

The proposed methods are tested on top of ECM-14.0 software [3] under JVET ECM common test conditions [4] under AI, RA, and LDB configurations. The tables below summarize the coding performance for the proposed method, and full results are available as separate Excel spreadsheets.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | 0.00% | -0.03% | -0.01% | 98.3% | 96.1% | 98.8% | 100.0% |
| Class C | 0.00% | 0.00% | 0.00% | 97.9% | 97.7% | 99.6% | 100.0% |
| Class E | 0.00% | 0.00% | 0.00% | 100.0% | 95.1% | 99.8% | 100.0% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 101.4% | 100.8% | 100.1% | 100.0% |
| Class F |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0.02% | -0.11% | -0.02% | 99.0% | 98.7% | 100.1% | 100.0% |
| Class C | -0.01% | -0.03% | 0.18% | 100.6% | 100.0% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.01% | -0.23% | -0.08% | 100.6% | 100.3% | 99.9% | 100.0% |
| Class F |  |  |  |  |  |  |  |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0.03% | 0.52% | 0.07% | 99.9% | 99.5% | 99.6% | 100.1% |
| Class E | -0.15% | -0.07% | -0.25% | 100.3% | 100.8% | 98.7% | 100.2% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.04% | 0.10% | 0.34% | 102.2% | 101.6% | 100.0% | 100.0% |
| Class F |  |  |  |  |  |  |  |

# References

- Kai Cui, Zhi Zhang, Han Huang, Vadim Seregin, Marta Karczewicz,  "EE2-4.1: Adaptive clipping with signalled lower and upper bounds," JVET-AG0145, January 2024.

- Jonatan Samuelsson-Allendes,  "AHG12: Fixes for Adaptive Clipping and Sign Prediction," JVET-AI0096, July 2024.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-14.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AI2017, 35nd Meeting, Sapporo, July 2024.

# Patent rights declaration

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).