| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0198-v1 |
| --- | --- |

| Title: | AHG12: on residual coding in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, M. Balcilar, C. Salmon-Legagneur, F. Galpin | Tel: Email: | fabrice.leleannec@interdigital.com, muhammet.balcilar@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes several modifications of residual coding for ECM. First aspect concerns the signaling of last significant coefficient position in a transform block. In case the prefix indicates the maximum allowed value for considered transform block, then a secondary prefix flag, signaled after the regular last X/Y position prefix, indicates that suffix takes maximum allowed value. Second aspect concerns the signaling of significant coefficient flag and gtX coefficient flags of the regular residual transform coding. It is observed that there is a systematic error between true probability and predicted one of these flags. The proposal removes this systematic error.

The proposed signaling aspects reportedly provide the following BD-rate performances over ECM-14.0, with no impact on encoder/decoder runtime.

- RA (Y / Cb / Cr):  -0,07% / 0,01% / 0,00%

- AI (Y / Cb / Cr) : -0,04% / -0,04% / 0,00%

# Introduction

In ECM, the following observations are made.

First observation deals with the signaling of last significant coefficient position in a transform block. It includes the following syntax elements:

- last_sig_coeff_{x,y}_prefix indicates the interval of X/Y value in which the X/Y position of last significant coefficients in includes. It is truntcated rice binarized and each bin is context-based arithmetic coded.

- last_sig_coeff_{x,y}_suffix indicates the X/Y value in indicated interval taken by the X/Y position of last significant position. It is fixed-length binarized and each bins is EP coded.

It is observed that the above signaling is sub-optimal for a number of cases.  In particular, it is frequent that for large block sizes, the last significant X or Y position takes the maximum allowed value for the considered transform block sizes.

Second observation concerns the signaling of coefficient significant flag and gtX flags coding in ECM. It is observed that when CABAC’s probability prediction for these bits are very high, the true probability is higher than the predicted one. That observation is also valid for lower probability prediction. Thus, when CABAC’s probability prediction for these bits are very low, the true probability is lower than the predicted one. This mismatch between predicted probability and true probability is the reason of wasting some bits in the compressed bitstream.

# Technical proposal

## Modified lastXY signaling

First aspect consists in modifying the last XY signaling of ECM, which is illustrated on Table 1. The modified last XY signaling is shown by Table 2. It consists in the following.

- Prefix is signaled the same way as in ECM-14.0.

- In case the transform block size is lower than some value (e.g. 32) or prefix value is lower than some value (e.g. the maximum allowed value), same suffix coding as in ECM-14.0 is achieved.

- Otherwise, a secondary prefix flag is signaled to constraint the signaling of the suffix part. For example, it may indicate that suffix is equal to maximum allowed value, given the TB size and possible zeroed out transform coefficients.

- If secondary prefix flag is signaled and is equal to false, then suffix is signaled, otherwise it is coded conditionally to the information given by the secondary prefix.

Table 1: binarization of prefix and suffix of last significant position syntax elements in ECM

Table 2: proposed signaling for last significant coefficient X/Y position

## Modified signaling of coefficient significance and gtX flags

According to our observations, when the 15-bit CABAC probability prediction of the significant and gtX flags of regular residual transform are very high (e.g. above to 32720), the true probability is higher that the probability that CABAC’s arithmetic engine is able to perform. That is also valid for lower probability.

Thus, when the 15-bit CABAC probability prediction of these flags are very low (e.g. below 48), the true probability is lower that the probability that CABAC’s arithmetic engine is able to compute. Since there is a limitation in CABAC arithmetic engine to perform such low or high probability, we assume the corrected probability is 1 when 15-bit predicted probability is higher than some certain threshold (e.g. 32720) and the corrected probability is 0 when 15-bit predicted probability is lower than some certain threshold (e.g. 48).

This assumption leads us to not signal these flags at all and assumed it is 1 or 0 but keep updating CABAC context model. Even though the true probabilities are very closed to 1 and 0 on these certain cases, they are not always exactly 1 and 0.

Thus, it is possible to have some cases where our assumption is not true. To prevent these cases, we modify encoding search and invalidate the candidate of solutions that includes bits where our assumption is not true.

In second approach, the mentioned mismatch of the probabilities can be solved by increasing the CABAC’s arithmetic engine range resolution in order to encode any bin with a probability that is higher than current maximum probability or the probability that is lower than the current minimum probability. In this way, there is no need to change the encoding search algorithm to invalidate some solutions as in the first approach.

We experienced both approaches give equivalent result. We show the first approach’s results in next section.

# Experimental results

The proposed aspects were tested over ECM-14.0, according to ECM common test conditions [1]. Obtained performances over ECM-14.0 are shown by Table 3.

Table 3: performances of proposed modified signaling for last XY, significant and gtX flags over ECM-14.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,05% | -0,03% | -0,04% | 100,7% | 98,7% | 99,0% | 102,1% |
| Class A2 | -0,05% | -0,12% | -0,04% | 100,2% | 100,6% | 100,2% | 103,3% |
| Class B | -0,03% | -0,11% | 0,00% | 98,8% | 100,6% | 105,3% | 100,1% |
| Class C | -0,03% | -0,10% | -0,05% | 99,8% | 99,6% | 102,6% | 100,2% |
| Class E | -0,05% | 0,22% | 0,14% | 99,1% | 98,7% | 102,2% | 100,1% |
| Overall | -0,04% | -0,04% | 0,00% | 99,6% | 99,7% | 102,3% | 101,0% |
| Class D | -0,02% | 0,04% | -0,09% | 99,5% | 100,9% | 99,8% | 100,1% |
| Class F | -0,07% | -0,04% | -0,14% | 99,3% | 100,4% | 100,7% | 100,2% |
| Class TGM | -0,03% | -0,01% | -0,06% | 100,6% | 100,2% | 101,9% | 100,3% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,07% | -0,02% | -0,22% | 100,9% | 102,1% | 100,1% | 100,5% |
| Class A2 | -0,01% | -0,07% | -0,12% | 100,5% | 100,3% | 99,9% | 99,8% |
| Class B | -0,15% | 0,00% | -0,03% | 99,7% | 99,1% | 99,9% | 100,1% |
| Class C | -0,02% | 0,10% | 0,28% | 98,8% | 97,3% | 100,4% | 100,2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,07% | 0,01% | 0,00% | 99,9% | 99,5% | 100,1% | 100,2% |
| Class D | -0,04% | -0,04% | 0,01% | 99,1% | 98,2% | 99,9% | 100,0% |
| Class F | 0,01% | -0,09% | -0,11% | 98,8% | 98,5% | 100,0% | 100,0% |
| Class TGM | -0,07% | 0,06% | 0,03% | 98,5% | 98,5% | 100,5% | 99,8% |

Note: running times are not highly accurate. 100% is expected given the nature of proposed modifications.

# Conclusion

This contribution proposed a modified signaling for last significant coefficient X/Y, significant coefficient flag and gtX coefficient flags in ECM. It reportedly brings -0.07% coding gain in RA and -0.04% coding gain in AI, with no impact on encoder or decoder runtime. It is proposed to study the proposed method in next round of JVET EE2.

# Cited References

- JVET-AF2017, “Common test conditions and evaluation procedures for enhanced compression tool testing”, M. Karczewicz, Y. Ye, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 32nd Meeting, Hannover, DE, October 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).