| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0199-v0 |
| --- | --- |

| Title: | AhG11: Post-facto dimension-wise decomposition of multiplier for content-adaptive loop-filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhuowei Xu, Jacek Konieczny, | Tel: Email: | {zhuowei.xu, jacek.k }@tcl.com |
| Source: | TCL |  |  |

- Abstract

It is proposed to apply dimension-wise decomposition to multipliers, for a purpose of content adaptation of Neural Network-based Loop Filter (NNLF).

It is reported that the average coding gains of this approach, in the RA configuration are 0.XX% (Y), 0.XX % (Cb) and 0.XX % (Cr) with respect to NNVC-9.1 with LOP.3 and NNIntra tool ON.

Introduction

A content-adaptive approach for LOP.3 was proposed in JVET-AI0111 [1]. It is achieved by inserting multipliers after convolutional layers and then overfitting the multipliers without modifying base parameters. Specifically, a convolutional layer can be expressed as:

|  | σW*x+b⋅m | Eq. (1) |
| --- | --- | --- |

where W is base weights of convolutional kernels, * is a convolution operator, x is an input, b is a bias, σ is an activation function. m∈Rcm is a one-dimensional multiplier with a length of cm, and it is learned in the overfitting stage.

The generated weight-updates are coded with the MPEG-NNR standard [2, 3] and signalled within a NNFU APS.

Proposed decomposed multiplier

The proposed decomposed multiplier is achieved by applying a dimension-wise decomposition on the one-dimensional multiplier, as illustrated in Fig 1. Specifically, for a selected multiplier m, it is obtained by overfitting two low-rank matrices as:

|  | m=flatten(Wm1×Wm2⊤) | Eq. (2) |
| --- | --- | --- |

where × is a product operation, and Wm1∈Rd1×r, Wm2∈Rd2×r are dimension-wise decomposed components, d1 and d2 are sizes of decomposition and d1×d2=cm, r is the rank of decomposition and flatten⋅ is a tensor flattening operation to align the result with the original shape of m:

|  | Rd1×d2→ Rcm | Eq. (3) |
| --- | --- | --- |

Fig 1. Decomposed multipliers on LOP3 filter [4].

Size of decomposition

For a specific multiplier layer, each possible size of decomposition that can be implemented for the corresponding length of this multiplier is tested. A size of the decomposition that 1) meets a given PSNR error threshold of output reconstructed samples, and 2) has a minimum number of weight elements, is used for this multiplier layer. When the decomposition of the multiplier layer cannot meet the given PSNR error threshold, no decomposition is applied. Therefore, the sizes of decomposition for each multiplier layer are adaptively selected.

Implementation of low-rank decomposition

The trained multipliers introduced in JVET-AI0111[1] are being further decomposed. Specifically, for each multiplier, all sizes of decomposition that are possible for the given length of the multiplier are tested. For each size of the decomposition, the trained multiplier is reshaped to a shape corresponding to the selected decomposition, where each dimension of the reshaped multiplier corresponds to one size of the selected decomposition. Next, a matrix rank decomposition (with a rank of r) is applied on the reshaped multiplier to obtain a corresponding dimension-wise decomposed multiplier with the number of components equal to number of dimensions of the reshaped multiplier.

The number and choice of specific multiplier layers to be overfitted are the same as in JVET-AI0111[1].

Optional fine-tuning

For selected multiplier layers, additional fine-tuning can be performed to improve overfitting accuracy of the decomposed multiplier components.

Overfitting

Table 2 shows the details of NN information during the finetuning stage.

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Overfitting Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU NVIDIA A10-24GB |
|  | Framework: | PyTorch 1.12.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 90 epochs |
|  | Batch size: | 32 |
|  | Loss function: | Weighted-MSE |
|  | Training time (for 1 model – 1 segment): | A1 & A2: B: C: D: F: |
|  | Training data information: | JVET CTC RA mandatory sequences |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144×144 |
|  | Learning rate: | 1e-3 |
|  | Learning rate update strategy | Constant learning rate |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Up-sample 72x72 UV signal to 144x144 blocks Normalize to 0~1 |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

Signaling

Same signaling approaches are used as JVET-AI0111 [1].

Inference

Table 3 describes the loop-filter information at inference stage. It is important to note that the decomposed multiplier is applied to the corresponding base weights before inference. Therefore, the neural network structure is the same as the original LOP.3, and the decoding time should not increase.

Table 3. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters | 51248 |
|  | Parameter Precision (Bits) | Int16 |
|  | Memory Parameter (MB) | 0.05 |
|  | MAC/Pixel (Kilo) | 17.0 |
| Optional |  |  |
|  | Total Conv. Layers | 106 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 128x128 / 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Experimental results

Table 4 shows the coding performance of the proposed approach (with no fine-tuning) in the RA configuration over NNVC-9.1 with LOP.3 and NNIntra ON.

Table 4. Coding performance over NNVC-9.1 with LOP.3 and NNIntra ON

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | 0.05% | -0.35% | -0.39% |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |
| Class H |  |  |  |  |  |  |  |  |  |

Table 5 shows the coding performance of the proposed approach (with additional fine-tuning) in the RA configuration over NNVC-9.1 with LOP.3 and NNIntra ON.

Table 5. Coding performance over NNVC-9.1 with LOP.3 and NNIntra ON

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | 0.05% | -0.82% | -0.75% |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |
| Class H |  |  |  |  |  |  |  |  |  |

Recommendation

It is recommended to investigate it into EE1.

References

- M. Santamaria, R. Yang, F. Cricri, M. M. Hannuksela, D. Bugdayci Sansli, A. Hallapuro, J. Lainema, H. Zhang. AhG11: Content-adaptive LOP3 filter. JVET-AI0111. July 2024.

- ISO/IEC FDIS 15938-17. Information technology – Multimedia content description interface – Part 17: Compression of neural networks for multimedia content description and analysis.

- Test Model of Incremental Compression of Neural Networks for Multimedia Content Description and Analysis (INCTM), N0179. February 2022.

- D. Liu, J. Ström, M. Damghanian, P. Wennersten (Ericsson), D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), T. Shao, P. Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya. EE1-1.2: Joint LOP model with inputs transformed. JVET-AH0080. April 2024.

- Patent rights declaration(s)

TCL may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).