| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0204-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0161 (EE2-3.3: OBMC extension with intra prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jung-Kyung Lee, Damian Ruiz Coll | Tel: Email: | jlee@ofinno.com duizcoll@ofinno.com, |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution reports the EE2 test 3.3c/d results for crosschecking. EE2 Test 3.3c and Test 3.3d explore the top and left boundary pixels adjacent to intra block are blended with intra prediction subblock generated using the intra prediction mode derived by applying DIMD on the neighbouring reconstructed samples in addition to the existing OBMC process.

# Test description

In JVET-AJ0161 [1], an overlapped block motion compensation (OBMC) extension with intra prediction is proposed. Building upon test 3.3a, which involves OBMC with intra prediction, test 3.3c introduces using the intra prediction mode of a neighboring block to generate intra subblocks instead of the using DIMD-derived mode. In test 3.3d, further neighboring blocks are examined for available motion information to enhance inter subblock generation, expanding upon the approach in test 3.3a.

# Experimental results

The software provided for the proponent has been simulated in accordance with the EE2 description [2] and under JVET ECM common test conditions [3] in both RA and LDB configurations. The partial results for Test 3.3c and Test 3.3d of the cross-check are detailed in the accompanying spreadsheet and are further summarized in the subsequent tables. It is affirmed that the proponent's reported results of coding gain in JVET- AJ0161 [1] align with the cross-check outcomes presented in this report for RA/ LDB configuration except for 3.3c Class A1/A2 in RA. Note that the test runtimes provided in this report may be inaccurate due to cluster noise.

Test 3.3c:

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.23% | -0.22% | 102.2% | 104.4% | 100.3% | 99.8% |
| Class A2 | -0.02% | 0.05% | -0.01% | 102.7% | 103.2% | 100.1% | 100.0% |
| Class B | -0.05% | -0.11% | -0.02% | 100.9% | 102.9% | 100.1% | 100.3% |
| Class C | -0.05% | -0.03% | 0.08% | 100.9% | 102.0% | 100.0% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.08% | -0.03% | 101.5% | 103.0% | 100.1% | 100.1% |
| Class D | -0.06% | -0.16% | -0.03% | 100.5% | 102.1% | 100.2% | 100.2% |
| Class F | 0.00% | 0.02% | -0.03% | 100.9% | 103.1% | 100.1% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.04% | -0.03% | 104.0% | 103.6% | 100.0% | 100.0% |
| Class C | 0.05% | 0.19% | -0.02% | 102.1% | 104.3% | 100.0% | 100.3% |
| Class E | -0.11% | -0.05% | -0.62% | 101.3% | 104.5% | 100.1% | 100.4% |
| Overall | -0.02% | 0.07% | -0.17% | 102.7% | 104.1% | 100.0% | 100.2% |
| Class D | -0.02% | 0.81% | 0.16% | 101.0% | 102.7% | 99.9% | 100.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Test 3.3d:

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.09% | -0.14% | 102.3% | 104.2% | 100.2% | 99.8% |
| Class A2 | 0.00% | 0.06% | -0.06% | 102.3% | 103.2% | 99.8% | 100.4% |
| Class B | -0.04% | -0.05% | -0.13% | 101.6% | 102.6% | 100.1% | 100.2% |
| Class C | -0.05% | 0.01% | 0.18% | 101.7% | 102.1% | 100.0% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.02% | -0.04% | 101.9% | 102.9% | 100.0% | 100.1% |
| Class D | -0.01% | -0.09% | -0.18% | 101.0% | 101.7% | 100.1% | 100.2% |
| Class F | 0.06% | 0.12% | 0.23% | 101.8% | 103.4% | 100.2% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.01% | 0.00% | 0.10% | 104.0% | 103.7% | 100.0% | 100.3% |
| Class C | 0.05% | -0.04% | 0.10% | 102.8% | 105.6% | 100.0% | 100.2% |
| Class E | -0.03% | 0.46% | -0.97% | 103.1% | 104.9% | 100.2% | 100.5% |
| Overall | 0.01% | 0.10% | -0.17% | 103.4% | 104.6% | 100.1% | 100.3% |
| Class D | 0.05% | -0.06% | 0.01% | 101.9% | 102.2% | 99.8% | 100.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# Conclusion

The cross-checker has confirmed that the “OBMC extension with intra prediction” proposed in Test 3.3c/d [1] is implemented on top of ECM-14.0, and the software implementation matches the technical description of the algorithms in JVET- AJ0161 [1]. The reported EE2 Test-3.3c and Test-3.3d results, as documented in JVET- AJ0161 [1], correlate with the simulation outcomes derived from this cross-checking process. The cross-checker considers that EE2-3.3a achieves a good trade-off between coding gain and complexity, recommending adopting it in ECM-15.0.

# References

- Donghyun Kim, Jinho Lee, Jongho Kim, Woong Lim, Sung-Chang Lim, Jin Soo Choi, “EE2-3.3: OBMC extension with intra prediction” JVET-AJ0161, Nov. 2024.

- V. Seregin et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2),” JVET-AI2024, July 2024.

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AI2017, 35nd Meeting, Sapporo, July 2024.

# Patent rights declaration(s)

Ofinno LLC does not have any current or pending patent rights relating to the technology described in this contribution.