| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0207-v1 |
| --- | --- |

| Title: | AHG9/AHG16: Performance results and suggestions on generative face video SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bolin Chen, Yan Ye, Jie Chen, Ru-Ling Liao (Alibaba) Shanzhi Yin, Zihan Zhang, Shiqi Wang (CityU) Kaifa Yang, Yue Li, Yiling Xu (SJTU) Ye-Kui Wang (Bytedance) Shiv Gehlot, Guan-Ming Su, Peng Yin, Gary J. Sullivan, Sean McCarthy (Dolby) Han Boon Teo (Panasonic) | Email: | chenbolin.chenboli@alibaba-inc.com shanzhyin3-c@my.cityu.edu.hk sekiroyyy@sjtu.edu.cn yekui.wang@bytedance.com sean.mccarthy@dolby.com hanboon.teo@sg.panasonic.com |
| Source: | Alibaba Group, CityUHK, SJTU, Bytedance Inc., Dolby Laboratories and Panasonic |  |  |

- Abstract

This contribution reports the performance results of generative face video compression (GFVC) using GFV SEI messages specified in the draft 5 of VSEI TuC (JVET-AI2032) and the latest GFVC models included in AhG16 common software tools (JVET-AI0048). It is reported that the changes of GFV SEI messages adopted in the last meeting can effectively reduce the signalling overhead of GFV parameters, thus facilitating most of GFVC solutions to achieve coding performance benefits over VVC under GFVC common test conditions. Based on such performance results as well as the editorial status of the text, the proponents consider the GFV and GFVE SEI messages to be sufficiently mature and suggest to adopt GFV and GFVE SEI messages into the next draft of VSEI version 4 WD.

Introduction

At the previous JVET meetings, the GFVC common software [1][2]  and the test conditions [3] were established as part of AHG16 activities, and the GFV SEI message was included into the draft 3 of TuC for future extensions of VSEI[4]. In addition, JVET-AH0239 [5] implemented the GFV SEI message on top of the VTM software and conducted the experiments using the GFV SEI message to signal facial parameters.

At the 35th JVET meeting, JVET-AI0048 [6] improved the GFVC reconstruction quality in head-and-shoulder scenarios by introducing diverse training data into GFVC model training and the corresponding retrained GFVC models were adopted in AHG16 GFVC software. More importantly, there were a number of GFV SEI changes integrated into the draft 5 of TuC [7] for normative descriptions and reduced coding bits, including:

- JVET-AI0156 GFV SEI bug fixes and editorial updates

- JVET-AI0184 GFV SEI value ranges for ue(v)-coded syntax elements and editorial updates

- JVET-AI0189 GFV updates on signaling and derivation of translator NN

- JVET-AI0190 GFV SEI updates on persistence and disallowing pictures from being both a base picture and a fusion picture

- JVET-AI0191 GFV SEI updates on GFV facial parameters signaling

- JVET-AI0192 GFV SEI on output order of GFV output pictures

- JVET-AI0193 GFV SEI editorial suggestions

- JVET-AI0194 On chroma key fusion for GFV SEI message

- JVET-AI0195 Change coding of key point coordinates and decimal part of matrix elements from u(v) to ue(v), and change the key point coordinates that are input to the generative model from the actual coordinates to be the normalized coordinates

To better illustrate the effectiveness of these GFV SEI changes in coding bits reduction, overall tests were conducted with the current GFVC software tools [6] under GFVC common test conditions. It is reported that the optimized changes of GFV SEI messages can effectively reduce coding bits for GFV parameter signalling, thus facilitating most of GFVC solutions to achieve coding performance benefits over VVC at 256x256 resolution.

Experimental Results

Testing Conditions & Anchors

The experiments of GFVC via SEI message approach are conducted under GFVC test condition [3] in this contribution. The experiment flowchart is shown in Figure 1. The detailed settings are as follows:

- For anchor, VVC (VTM-22.2) is used and the test is conducted under low-delay B (LDB) configuration with quantization parameters (QP) setting to 37, 42, 47 and 52.

- In addition, we also provide an AE version test for GFVC, i.e., the base picture is compressed as an I picture using the VTM reference software 22.2, and other pictures are further represented with a series of facial parameters and compressed via a context-adaptive arithmetic coder. The QPs of the base picture are set to 22, 32, 42 and 52. It should be mentioned that all GFVC models in this test are included in AhG16 software tools [6].

- For test, the base picture is coded as an I picture using the VTM-22.2 with QP value of 22, 32, 42 and 52. The other pictures are coded with GFVC method and the feature parameters are signalled via GFV SEI message. It should be mentioned that all experimental tests use two different implementations in GFV SEI messages, i.e., one is JVET-AI0195 [8] and another is JVET-AI2032 [7] as follows:

- JVET-AI0195: changes coding of key point coordinates and decimal part of matrix elements from u(v) to ue(v), but predicts GFV parameters with the base picture.

- JVET-AI2032: updates on GFV facial parameters signaling and predicts GFV parameters with the previous picture. It should be mentioned that this version reflects the capability of the GFV SEI message in the latest VSEI TuC.

Figure 1 – Illustration of encoding/decoding processes for GFV SEI message.

Rate-Distortion Performance

Figure 2 shows the rate distortion performance (i.e., Rate-DISTS and Rate-LPIPS) of different GFVC algorithms with the GFV SEI as in output document JVET-AI2032, the GFV SEI as in JVET-AI0195, and GFV with arithmetic coding. VVC anchor performance is also plotted.

As shown, the arithmetic coding version of GFV is the most efficient, because it only compresses these facial parameters with arithmetic coding, and does not compress other information essential for practical applications. In contrast, the GFV SEI message provides suitable bitstream structures for practical applications. With the overhead reduction mechanisms adopted at the last meeting, the coding bits for GFV parameter signalling is significantly reduced compared with JVET-AI0195 (a preliminary implementation of GFV SEI), and the current GFV SEI can have competitive RD performance over the VVC anchor for most of the GFVC algorithms for most of the rate points. In JVET-AJ0209 [9], it can be seen that GFV SEI provides more compelling performance for 512x512 resolution.

- DAC Results

- FOMM Results

- CFTE Results

- FV2V Results

- HDAC Results

- HDAC Extension Result

Figure 2 – Rate distortion performance of VVC and AhG16 software with different GFVC algorithms measured DISTS and LPIPS in terms of average results for Class A and Class B at the 256px resolution.

BD-Rate Savings

Table 1 shows average bit-rate savings of 256x256 face video sequences for different GFVC algorithms compared with VVC. It can be seen that this optimized GFV SEI messages in JVET-AI2032 can effectively reduce coding bits for GFV parameter signalling compared with JVET-AI0195 for all GFVC algorithms. As such, this GFV SEI message in JVET-AI2032 can achieve competitive bit-rate savings over the VVC anchor for most of GFVC algorithms except for FOMM.

Table 1 - Average bit-rate savings of 256px face video sequences for AHG16 software algorithms with different compression approaches compared with the traditional VVC codec evaluated in the RGB 444 and YUV 420 domain

- DAC

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| AE | -67.4% | -68.1% | -22.7% | -51.8% | -14.5% | -46.4% |
| SEI (JVET-AI0195) | 9.7% | 6.5% | 31.9% | 42.7% | 18.6% | 41.6% |
| SEI (JVET-AI2032) | -26.2% | -28.3% | 10.5% | 2.9% | 5.3% | 2.9% |

- FOMM

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| AE | -32.8% | -32.7% | 11.3% | -2.9% | 9.8% | #VALUE! |
| SEI (JVET-AI0195) | 82.0% | 83.7% | 24.4% | 93.9% | 9.3% | 91.4% |
| SEI (JVET-AI2032) | 11.5% | 14.3% | 14.9% | 38.0% | 5.0% | 36.8% |

- CFTE

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| AE | -58.0% | -58.5% | -6.4% | -36.1% | -4.9% | -32.5% |
| SEI (JVET-AI0195) | 28.4% | 30.5% | 30.0% | 54.2% | 16.9% | 57.3% |
| SEI (JVET-AI2032) | -13.1% | -12.2% | 21.2% | 17.5% | 7.5% | 16.7% |

- FV2V

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| AE | -47.5% | -45.4% | -0.5% | 49.4% | -0.2% | -14.8% |
| SEI (JVET-AI0195) | 24.8% | 28.8% | 25.3% | 68.5% | 6.1% | #VALUE! |
| SEI (JVET-AI2032) | -12.5% | -10.2% | 21.3% | 17.9% | 16.7% | 22.0% |

- HDAC

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| AE | -27.2% | -22.5% | 117.6% | 57.8% | 115.2% | 69.2% |
| SEI (JVET-AI0195) | -2.93% | 5.24% | 156.56% | 122.74% | 172.17% | 131.43% |
| SEI (JVET-AI2032) | -14.5% | -8.1% | 133.4% | 93.7% | 148.3% | 101.3% |

- HDAC_Extension

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| AE | -19.0% | -21.0% | 57.2% | 13.5% | 56.0% | 26.0% |
| SEI (JVET-AI0195) | 15.9% | 12.9% | 105.6% | #VALUE! | 100.8% | 81.8% |
| SEI (JVET-AI2032) | -0.6% | -2.9% | 95.3% | 46.7% | #VALUE! | 54.0% |

Suggestion to move GFV and GFVE SEI messages to VSEI WD

The generative face video (GFV) SEI message was firstly proposed at the 29th JVET meeting in January 2023 [10] and adopted in the VSEI TuC draft in January 2024. During the past two years, various aspects of GFVC, including the compression performance benefits and the interoperability issue among different face representation methods, as well as the syntax and semantics of GFV SEI message, have been extensively studied in the JVET and in AhG16. In April 2024 as a straightforward extension to GFVC to reach a wider range of bitrate and quality, the generative face video enhancement (GFVE) SEI message was proposed and also included in VSEI TuC draft.

As the output of the AhG16 activities, test conditions [3] for evaluating the compression performance of GFVC were defined, and the software tools were developed, made available on git [12] and constantly improved and updated[1][2]. The GFV SEI message software was also implemented on top of VTM and made available on VTM TuC branch [13]. The experimental results reported in this contribution clearly show the coding performance benefits of the SEI message solution of GFVC over VVC for various face representation methods. In JVET-AJ0209, even higher coding performance gain was observed when the tests were conducted on the higher-resolution sequences with multi-resolution models proposed in this meeting.

Given the clear technical evidence of superior compression performance and the sufficient studies of syntax and semantics of GFV/GFVE SEI messages, the proponents consider these two SEI messages to be sufficiently mature and therefore suggest to adopt them into the next draft of VSEI version 4 WD. The proponents also suggest that the GFV and GFVE SEI messages fulfill the requirements laid out in m64987 and VCEG-BT04 “On VVC-assisted ultra-low-rate generative face video coding,” which led to the establishment of AHG16 on GFVC.

References

- Chen B, Ye Y, et al. AHG 16: Updated Common Software Tools for Generative Face Video Compression, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AH0114, April 2024.

- Chen B, Chen J, et al. AHG16: Proposed Common Software Tools and Testing Conditions for Generative Face Video Compression, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0042, January 2024.

- McCarthy S, Chen B. Test conditions and evaluation procedures for generative face video coding, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG2035, January 2024.

- McCarthy S, Chen J et al, Technologies under consideration for future extensions of VSEI (version 3), The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AG2032, January 2024.

- Chen B, Chen J, et al. “AHG9/AHG16: Software Implementation of Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AH0239, April 2024.

- Chen B, Ye Y, et al. “AhG16: Improving GFVC Performance with Diverse Training Data,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AI0048, July 2024.

- McCarthy S, Chen J et al, Technologies under consideration for future extensions of VSEI (version 5), The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AI2032, July 2024.

- Chen B, Chen J, et al. “AhG9/16: Updates on GFVC common software tools and GFV SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AI0195, July 2024.

- B. Chen, Y. Ye, et al. “A AHG9/AHG16: Performance results of generative face video SEI message on high-resolution sequences,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AJ0209, Nov. 2024.

- Chen B, Chen J, et al., “AHG9: Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AC0088, January 2023.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-gfvc/gfvc_v1

- https://vcgit.hhi.fraunhofer.de/jvet-tuc/VVCSoftware_VTM

Patent rights declaration(s)

Alibaba Group, City University of Hong Kong, Shanghai Jiao Tong University, Bytedance Inc., Dolby Laboratories and Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).