| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0208-v2 |
| --- | --- |

| Title: | [AHG11] Towards Incorporating End-To-End Learning-Based Image Coding into Traditional Codec |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Fabian Brand, Timofey Solovyev, Elena Alshina | Tel: Email: | fabian.brand@huawei.com solovyev.timofey@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies, Co. |  |  |

_____________________________

# Abstract

In this contribution, we present a method to combine end-to-end trained image compression methods with long-proven methods from VTM. The method comprises using an end-to-end coded image as reference image for a modified I-Frame. The encoder can then decide whether and how to use the AI-reference frame. A major advantage of this method is that data transfer from a GPU/NPU is only necessary on full frame level, thus solving a major problem in realizing neural-network-based components in hardware while using specialized hardware. This contribution aims at providing a proof-of-concept for this method and shows one possible way, E2E learning-based coding can be incorporated into traditional coding. With this hybrid method, we obtain rate-savings of -7.2% in Class E and -4.97% in Class C. For class C encoding run time overhead is 5% on CPU and 0.05% if GPU is used for E2E learning-based picture coding. This complexity-performance trade off is making this a promising new direction to explore, while keeping the main functionality of hybrid coders.

# Introduction

End-to-end (E2E) learning-based coding has several advantages over block-based coding. By using powerful statistical models, more diverse content can be modeled. On the other hand, content, like, most prominently, planar regions and straight edges, which makes up a large portion of image and video data can be very efficiently modeled with VVC-like codec. It stands to reason that a hybrid solution, which uses the proven method within known video coders as well as the powerful modeling from learning-based solutions can lead to the synergy. AHG11 made good progress in integrating learning-based methods on top of VTM by replacing/enhancing/augmenting individual components of VTM with learning-based methods. One problem lies in the upscaling of these methods. GPUs/NPUs are commonly found also in mobile devices, which are very well capable of running neural networks efficiently with high parallelism, however, transferring the data to the specialized hardware on block-level is inefficient, and hinders the deployment of deeply-integrated learning-based tools on specialized hardware.

Instead of augmenting the core of VTM with learning-based function, the method described in this contribution aims at using an E2E coded frame as reference frame to a modified VTM I-Frame. This way a data exchange between major coding engine and GPU/NPU happens only through decoded pictures buffer. Adding additional reference frames to image coding is a widely used technique, e.g. in scalable coding, multi-view-coding or DRFG (as studied in EE.1) [1]. Furthermore, this method of combining traditional and learnable coding leaves the core functionality of VTM intact and mainly unchanged.

In this contribution, we describe a method where the VTM encoder can decide for each I-Frame whether the standard VTM frame is used or a frame coded with the I-Frame coder of the DCVC-FM [2] framework. In details implementation is as follows. Encoding of the frame using E2E-lernable solution takes fraction of percent of VTM frame encoding. The E2E-coded frame is included into VTM via a reference frame. In the first version of this method we restricted the encoder to only skip modes, and to use it for the entire frame when this mode is selected. In principle it is also possible to only use the E2E-coded reference frame in parts of the image and to only transmit these regions (since region-based encoding is supported by some learnable image codec, such as JPEG AI).

# Method description

Our method uses the intra frame coder of the DCVC-FM [2] video coder. Most state-of-the-art publicly available learning-based image coders focus on optimizing on perceptual quality instead of PSNR. Since JVET mostly focuses on PSNR performance, we chose the DCVC-FM intra coder as an example. To demonstrate this method, we also used the pre-trained weights provided by the authors and only slightly modified the coder to have a similar luma-chroma compression performance trade-off as VTM. DCVC-FM Intra has a complexity of 524 kMAC/px and a model size 26.59 million parameters, for both the encoder and decoder. This is too high complexity to be practical. Note, however, that the presented method is agnostic to the E2E-coder used for external reference picture encoding, as well as agnostic to the traditional coder it is combined with (VTM, NNVC, ECM).

As mentioned in the introduction, we use the decoded frame as reference frame to a modified I-Frame and let the encoder decide whether this reference frame is used. In the case encoder decides not to refer to the external reference frame associated overhead is not transmitted. If encoder decides to refer to the external reference picture then all bits required for whole external reference picture reconstruction are included into the of the final bitstream. In the current version, this decision is made on frame-level, however, a block-based solution is under implementation.

# Results

We tested our methods according to the NNVC CTC (full length sequences for QP 22-42). Since this method mainly effects intra frames, we primarily tested the AI configuration. Additionally, we show with RA tests, that inserting an E2E coded frame in the prediction chain does not show any disadvantages. The current version is implemented on top of VTM 23.4, which serves as anchor. Since the harmonization with the LMCS tool is not completed, we turned this tool off in both our test and the anchor. Results are show in the Table 1.

Table 1 BD-rate relatively to VTM anchor under AhG11 test conditions.

| All Intra (AI) |  |  |  |
| --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | -0.39% | -6.35% | -2.83% |
| Class C | -4.97% | -10.57% | -8.79% |
| Class E | -7.22% | -13.16% | -20.53% |
| Overall |  |  |  |
| Class D | -6.38% | -15.05% | -12.37% |

| Random Access (RA) |  |  |  |
| --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C | -0.73% | -4.63% | -3.18% |
| Class E |  |  |  |
| Overall |  |  |  |
| Class D | -0.93% | -6.51% | -5.90% |

The results show good gains both in All Intra configuration and also in Random Access, clearly showing that the presence of an AI-coded reference frame does not impair the subsequent inter prediction process of VTM.

Simulations have been performed on non-homogeneous cluster. So, run time is not reported.

In the following we give accurately measured on a same stable machine encoding runtimes on single I frame for VTM, and DCVC-FM. For fairer comparison, we give DCVC-FM runtime both on CPU and GPU for low and high image quality. The results were obtained on a single threaded CPU (AMD Ryzen 9 7950X with 5200 MHz) and, if applicable on an RTX 4090 GPU. Accurate run-time measurement is shown in the Table 2.

Table 2 Accurate encoding run-time measurement.

|  | Runtime for high quality (QP 22 / QI 63) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM |  | DCVC-FM (GPU) |  |  |  | DCVC-FM (CPU) |  |  |  |
|  | Encoder | Decoder | Encoder |  | Decoder |  | Encoder |  | Decoder |  |
|  |  |  | Absolute | Relative | Absolute | Relative | Absolute | Relative | Absolute | Relative |
| Class B | 304 | 0.161 | 0.11 | 0.0004 | 0.11 | 0.68 | 31 | 0.10 | 24.88 | 155 |
| Class C | 83 | 0.054 | 0.04 | 0.0005 | 0.02 | 0.37 | 4.89 | 0.06 | 3.96 | 73 |
| Class D | 22 | 0.014 | 0.03 | 0.0014 | 0.008 | 0.57 | 0.89 | 0.04 | 0.67 | 48 |

|  | Runtime for low Quality (QP 42 / QI 0) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM |  | DCVC-FM (GPU) |  |  |  | DCVC-FM (CPU) |  |  |  |
|  | Encoder | Decoder | Encoder |  | Decoder |  | Encoder |  | Decoder |  |
|  |  |  | Absolute | Relative | Absolute | Relative | Absolute | Relative | Absolute | Relative |
| Class B | 39.39 | 0.064 | 0.11 | 0.0028 | 0.1 | 1.56 | 30.63 | 0.78 | 24.14 | 377 |
| Class C | 23.72 | 0.018 | 0.04 | 0.0017 | 0.02 | 1.11 | 4.95 | 0.21 | 3.22 | 179 |
| Class D | 6.98 | 0.006 | 0.03 | 0.0043 | 0.007 | 1.17 | 0.9 | 0.13 | 0.66 | 110 |

The results show that even when run on CPU, DCVC-FM shows a low runtime compared to VTM, which is independent of the image quality. Note that for the current encoder implementation both VTM and DCVC-FM have to be run. The encoding runtime overhead relatively to the anchor is essentially DCVC-FM run time, which is for class C (for example) is 6% for high quality and 21% for low quality (if external reference frame is coded on single threaded CPU) and 0.05% and 0.17%, respectively (if GPU is used). Furthermore, if the decoder has a GPU available, it is possible to run the DCVC-FM decoder in a similar time as VTM (keep in mind that in this case the load of VTM drops to a minimum, since only large blocks without motion compensation and residual coding are used). Note that this method is specifically designed to be friendly to the use of specialized hardware for learnable codec, memory transfer between major coding engine and learnable coded happens only through decoded picture buffer.

# Conclusion

In this contribution we presented one possibility how the large progress in E2E-trained image and video compression can be leveraged also in traditional coding methods. It is shown that 2-5% gain can be achieved with encoder run-time overhead of just 5% (even without GPU). The method is specifically designed to minimize the data-transfer between the CPU and potentially used specialized hardware, which constitutes a major bottleneck in the application of dual-architecture image and video coding. Currently the method entails frame-level switching between learned and traditional coding. However, in principal also switching in a finer granularity is possible with the approach, which can increase the flexibility and there can potentially increase the coding efficiency. Authors find complexity performance trade-off attractive, plan to further improve implementation and inform AhG11 about the progress.

# Patent rights declaration(s)

Huawei may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

| [1] | W. Bao, N. Fu, X. Chen, J. Jia, Z. Chen, Z. Liu, X. Xu and S. Liu, "EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement (JVET-AH0107)," in 34th JVET Meeting, Rennes, 2024. |
| --- | --- |
| [2] | J. Li, B. Li and Y. Lu, "Neural Video Compression with Feature Modulation," in Conference on Computer Vision and Pattern Recognition (CVPR), Seattle, 2024. |