| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0209-v1 |
| --- | --- |

| Title: | AHG9/AHG16: Performance results of generative face video SEI message on high-resolution sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bolin Chen, Yan Ye, Ru-Ling Liao, Jie Chen (Alibaba) Shanzhi Yin, Shiqi Wang (CityU) | Email: | chenbolin.chenboli@alibaba-inc.com |
| Source: | Alibaba and City University of Hong Kong |  |  |

- Abstract

At the 35 th meeting, JVET-AI0047 proposes high-resolution testing data with suitable copyright license for GFVC experimentation. In this contribution, we provide higher-resolution performance results according to multi-resolution GFVC models proposed in JVET-AJ0052 at this meeting. Experimental results show that all multi-resolution GFVC models with GFV SEI messages solutions can achieve promising RD performance for higher resolution testing data. Besides, with the increase of face video resolution from 256×256 to 512×512, the SEI-based GFVC approaches can achieve coding performance benefits for a wider range of bit rates. As such, it is suggested to add these higher-resolution test data to the AHG16 test condition document (JVET-AG2035) for further GFVC investigations.

Introduction

At the 34th JVET meeting, JVET-AH0113 [7] proposed to extend GFVC to code higher resolution face video sequences, but this attempt was faced with some problems such as lack of high-resolution training and testing data for face content as well as unoptimized algorithm design for robust signal reconstruction. At the 35th JVET meeting, JVET-AI0047 [6] proposed higher-resolution test data (512×512) with suitable copyright license for GFVC experimentation, and provided initial experimental results with one of GFVC models (i.e., CFTE) instead of with all GFVC models. Such results cannot demonstrate comprehensive GFVC evaluations for higher-resolution testing data.

To better advance GFVC in higher resolution, in this contribution, we further update higher-resolution performance results according to multi-resolution GFVC models proposed in JVET-AJ0052 [8] at this meeting. It can be seen that all GFVC approaches with GFV SEI messages can provide more compelling performance for 512x512 resolution.

Higher-Resolution GFVC Test Conditions

Test Dataset

For the testing dataset, JVET-AI0047 [6] proposes to use 33 talking face video sequences selected and pre-processed from the CFVQA dataset [2]. A snapshot of the video set is shown in Figure 1.

- The copyright of CFVQA dataset [2] is pasted below and is therefore suitable for use by JVET. The CFVQA dataset is sourced from the Pexels website and we select 15 sequences with face-centric content as class A and 18 sequences with head-and-shoulder content as Class B.

“All photos and videos on Pexels can be downloaded and used for free (Please read the license terms here. https://www.pexels.com//license/).”

“What is allowed?

We tried to keep the Pexels License as simple as possible. Keep reading to find out more.

- All photos and videos on Pexels are free to use.

- Attribution is not required. Giving credit to the photographer or Pexels is not necessary but always appreciated.

- You can modify the photos and videos from Pexels. Be creative and edit them as you like.”

- These selected sequences span a considerable diversity of face video content, in terms of age, gender, ethnicity, appearance, expression, head position, head motion, camera motion, background, etc.

- Each of these test sequences is cropped from the original sequence with the resolution of 512×512. The color format is RGB 444. The Class A sequences are 10-second long (i.e., 250 frames at 25 fps), and the Class B sequences are 5-second long (i.e.,125 frames at 25 fps).

- It should be mentioned that the proposed Class B test sequences are the same content as the Class B in GFVC test conditions specified in JVET-AG2035 [1] except for the resolution. In fact, the Class B sequences in JVET-AG2035 were downsampled from the proposed 512x512 sequences.

- The overall test dataset is upload to JVET ftp and available at testing_sequence.

  - Class A: Face-centric content

(b)  Class B: Head-and-shoulder content

Figure 1 – Proposed test set of 33 face sequences.

Test Conditions

Except for the higher-resolution test sequences, we follow other test conditions and software reference configurations for GFVC testing specified in JVET-AG2035 [1]. Test settings for VVC and for GFVC are as follows:

- For VVC, VTM 22.2 is used and the test is conducted under low-delay B (LDB) configuration with quantization parameters (QP) setting to 37, 42, 47 and 52.

- For GFVC via GFV SEI messages, its base picture is compressed as an I picture using the VTM reference software 22.2, and other pictures are further represented with a series of facial parameters and signalled via GFV SEI message [9]. The QPs of the base picture are set to 22, 32, 42 and 52.

Experimental Results

Figure 2 shows the rate distortion performance (i.e., Rate-DISTS and Rate-LPIPS) of GFVC with GFV SEI messages compared to that of VVC on two group of higher-resolution testing sequences. It can be seen that the SEI-based GFVC solutions can achieve coding performance benefits for a wider range of bit rates over the VVC codec at the video resolution of 512×512. This is because the overhead of sending facial parameters via SEI messages becomes smaller relative to the VVC-coded base pictures as the video resolution increases.

(a) Overall – DISTS                                               (b) Overall – LPIPS

(c) Class A – DISTS                                               (d) Class A – LPIPS

(e) Class B – DISTS                                               (f) Class B – LPIPS

Figure 2 - Rate distortion performance of VVC and GFVC with GFV SEI messages measured by two perceptual quality metrics DISTS and LPIPS in terms of overall dataset, Class A and Class B.

As shown in Table 1, we provide the average bit-rate savings of different GFVC algorithms with GFV SEI messages compared with the traditional VVC codec at 512×512 testing resolution. It can be seen that SEI-based GFVC approaches show performance benefits in terms of Rate-DISTS and Rate-LPIPS. Specifically, the DAC model can achieve the highest average bit-rate savings with 47.36% of Rate-DISTS and 41.79% of Rate-LPIPS.

Table 1 - Average bit-rate savings of 33 face video sequences for different GFVC algorithms with GFV SEI messages compared with the traditional VVC codec evaluated in the RGB 444 and YUV 420 domain

- CFTE

| Sequences |  | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| 512×512 | Class A | -42.89% | -22.28% | 0.00% | 1.50% | 0.00% | -1.20% |
|  | Class B | -30.48% | -14.53% | 7.98% | 11.65% | 0.00% | 8.81% |
|  | Overall | -36.68% | -18.40% | 3.99% | 6.57% | 0.00% | 3.81% |

- FOMM

| Sequences |  | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| 512×512 | Class A | -21.14% | -11.52% | 2.95% | 5.05% | 0.00% | 5.52% |
|  | Class B | -9.29% | -2.31% | 28.61% | 31.16% | 70.64% | 33.04% |
|  | Overall | -15.22% | -6.92% | 15.78% | 18.10% | 35.32% | 19.28% |

- FV2V

| Sequences |  | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| 512×512 | Class A | -47.10% | -28.40% | 0.00% | -2.30% | 0.00% | -1.01% |
|  | Class B | -34.19% | -23.84% | 5.46% | 8.94% | 78.49% | 12.98% |
|  | Overall | -40.65% | -26.12% | 2.73% | 3.32% | 39.25% | 5.98% |

- DAC

| Sequences |  | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| 512×512 | Class A | -53.08% | -46.12% | -1.61% | -5.82% | -1.41% | -2.55% |
|  | Class B | -41.64% | -37.46% | 34.05% | -3.63% | 0.00% | 3.42% |
|  | Overall | -47.36% | -41.79% | 16.22% | -4.72% | -0.71% | 0.43% |

Table 2 provide the average bit-rate savings comparisons of ClassB with two different testing resolutions of  256×256 and 512×512. It should be mentioned that the proposed higher-resolution Class B test sequences are the same content as the Class B in GFVC test conditions specified in JVET-AG2035 [1] except for the resolution. It can be also seen that with the increase of face video resolution to 512×512, the SEI-based GFVC approaches can achieve promising coding performance benefits.

Table 2 - Average bit-rate savings of ClassB video sequences for different GFVC algorithms with GFV SEI messages compared with the traditional VVC codec evaluated in the RGB 444 and YUV 420 domain at two different testing resolutions

- CFTE

| Sequences |  | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| 256×256 | Class B | -4.60% | -3.23% | 19.61% | 25.46% | 12.92% | 21.86% |
| 512×512 | Class B | -30.48% | -14.53% | 7.98% | 11.65% | 0.00% | 8.81% |

- FOMM

| Sequences |  | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| 256×256 | Class B | 19.93% | 22.83% | 29.78% | 35.21% | 10.03% | 35.85% |
| 512×512 | Class B | -9.29% | -2.31% | 28.61% | 31.16% | 70.64% | 33.04% |

- FV2V

| Sequences |  | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| 256×256 | Class B | -3.29% | -0.03% | 18.59% | 24.80% | 28.89% | 31.49% |
| 512×512 | Class B | -34.19% | -23.84% | 5.46% | 8.94% | 78.49% | 12.98% |

- DAC

| Sequences |  | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| 256×256 | Class B | -18.38% | -19.65% | 11.61% | 9.66% | 5.88% | 7.66% |
| 512×512 | Class B | -41.64% | -37.46% | 34.05% | -3.63% | 0.00% | 3.42% |

Conclusion

This contribution further updates higher-resolution performance results according to multi-resolution GFVC models proposed in JVET-AJ0052 at the 36th meeting. Experimental results show that all multi-resolution GFVC models with GFV SEI messages can achieve promising RD performance for higher-resolution testing data. It is suggested to add these higher-resolution test data to the AHG16 test condition for further GFVC investigations.

References

- McCarthy S, Chen B. Test conditions and evaluation procedures for generative face video coding, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG2035, January 2024.

- Li Y, Chen B, Chen B, et al. Perceptual Quality Assessment of Face Video Compression: A Benchmark and An Effective Method[J]. IEEE Transactions on Multimedia, 2024.

- Chen B, Ye Y, et al. AHG 16: Updated Common Software Tools for Generative Face Video Compression, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AH0114, April 2024.

- Zhu H, Wu W, Zhu W, et al. CelebV-HQ: A large-scale video facial attributes dataset[C]//European conference on computer vision. Cham: Springer Nature Switzerland, 2022: 650-667.

- McCarthy S, Chen J et al, Technologies under consideration for future extensions of VSEI (version 3), JVET-AG2032, online meeting, January 2024.

- Chen B, Yin S, et al. AHG 16: Higher-resolution Test Sequences and Test Results for Generative Face Video Compression, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AI0047, July 2024.

- Zou R, Chen B, et al. AHG 16: Lightweight CFTE with Multi-Resolution support, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AH0113, April 2024.

- Yin S, Chen B, et al. AHG 16: Multi-resolution Models for Generative Face Video Compression, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AJ0052, November 2024.

- McCarthy S, Chen J et al, Technologies under consideration for future extensions of VSEI (version 5), The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AI2032, July 2024.

Patent rights declaration(s)

Alibaba and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).