| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0210-v2 |
| --- | --- |

| Title: | AhG12: On In-Loop Filtering in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy, Yun Li, Marta Karczewicz | Tel: Email: | dmytror@qti.qualcomm.com yli30@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Unified Filter Architecture for In-Loop filtering of NNVC was defined in JVET-AD0380 for High-performance Operation Point (HOP, 460kMAC/Pixel) and further extended to Low Operation Point (LOP, 17kMAC/pixel) in JVET-AE0281 and Very Low Operation Point (VLOP, 5kMAC/pixel) in JVET-AH0051.

In this contribution, NN-based In-LOP Filter derived from NNVC VLOP (JVET-AH0051) was integrated into ECM framework and its operations were harmonized with existing ECM-ALF. NN model with complexity of 4.2kMAC/pixel and 40KB of side information was trained on DIV2K, BVI and TVD datasets following NNVC guidelines and tested with NNVC inference interface implemented in ECM13 and 14.

Proposed method was tested with ECM CTC, and reportedly provides the following BD-rate PSNR {Y, Cb, C} changes against the respective anchor:

Test 1:

- RA:  {-1.2%, -0.0% and -0.1%} and 101.0% of encoder and 238% decoder run time.

- AI:  {-0.7%, 0.0% and 0.0%} and 101.0% of encoder and 236% decoder run time.

Test 2:

- RA*:  {-0.7%, -0.1% and -0.0%} and ~100% of encoder and 151% decoder run time.

AI:  {-0.5%, 0.0% and 0.0%} and 101% of encoder and 175% decoder run time.

* - Average results for Test2 RA are provided over B,C classes, Class A RA are incomplete.

It is proposed to study this method in EE2.

- Introduction

Unified Filter Architecture for In-Loop Filtering of NNVC as well as inference process and training procedure was originally defined in JVET-AD0380 for High-performance Operation Point (HOP, 460kMAC/Pixel) and further extended to Low Operation Point (LOP, 17kMAC/pixel) in JVET-AE0281and Very Low Operation Point (VLOP, 5kMAC/pixel) in JVET-AH0051.

Simplified block diagram of the NN ILF for LOP and VLOP, which may be considered as relevant for integration into ECM, are shown in Figure 1. This method takes reconstructed pixels (Luma and chroma), prior to the deblocking, prediction pixels, coding context information (e.g. QP, BS, coding modes), extact input features and jointly process these features by Luma and Chroma backbones (sequences of convolution layers) independently.

The output of NN-ILF is combined with output of the deblocking filter, with the weights and the offsets derived at the encoder side and signalled in the bitstream. Resulting pixels are further processed by VVC SAO and ALF.

Figure 1. Simplified Unified Filter Atrchitecture of NNVC (LOP, VLOP).

- Proposed method

In this contribution, a modifed NN-based In-LOP Filter, derived from NNVC VLOP (JVET-AH0051)  was integrated into ECM and its operations are harmonized with existing ALF. The VLOP filter architecture was modified to remove existing in NNVC luma dependency on chroma and retrained on DIV2K, BVI and TVD dataset following the NNVC training guidelines. Resulting filter has complexity of 4.2kMAC/pixel and 40KB of the side information.

Simplified block diagram of the proposed method is shown in Figure 2. NN filter context includes base and slice QPs, coding modes and filter parameters, similarly to NNVC.

Figure 2. Proposed NN filter for ECM.

Utilization of the proposed NN-filter is harmonized with ECM ALF to benefit from existing RDO and signaling. Simplified block diagram of proposed ECM ALF is shown in Figure 3. NN-filter operates on input reconstructed pixels in parallel to existing Fixed, Residual and Gaussian filters and provides additional coding gain on top of these filters.

Figure 2. Integration of the proposed NN filter into ECM ALF.

- Simulation results

NN model was trained following NNVC guidelines for VLOP and tested in ECM13 and ECM14 with int16 NNVC SADL interface. Resulting model has complexity of 4.2kMAC/pixel and around 40KB parameters, quantized with 8-11 bits precission.

Two complexity configurations have been tested on ECM13 and 14, with complexity regulated by encoder through complexity-aware RDO.

Table 1 demonstrate performance of the NN filter unconstrained with complexity measurement, whereas

Table 1. BD-rate performance of the Test 1, comparing to ECM 13.

|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.88% | -0.02% | -0.02% | 100.7% | 266.9% | -1.60% | 0.00% | -0.20% | 100.9% | 234.0% |
| Class A2 | -0.54% | -0.03% | -0.03% | 98.6% | 212.9% | -1.53% | -0.06% | -0.14% | 101.0% | 195.8% |
| Class B | -0.54% | 0.00% | 0.00% | 101.2% | 232.2% | -1.08% | 0.03% | 0.09% | 99.7% | 236.6% |
| Class C | -0.44% | 0.01% | 0.01% | 100.8% | 255.7% | -0.80% | -0.05% | -0.16% | 102.1% | 275.2% |
| Class E | -1.07% | 0.06% | 0.06% | 100.9% | 226.5% |  |  |  |  |  |
| Overall | -0.66% | 0.01% | 0.01% | 100.5% | 238.3% | -1.20% | -0.02% | -0.08% | 100.8% | 236.6% |
| Class D | -0.63% | 0.08% | 0.09% | 104.1% | 224.5% | -0.85% | 0.28% | 0.12% | 101.6% | 229.6% |

Table 2 shows performance of the NN filter with complexity-ware RDO, restricting use of NN filter at slice/block level.

Note, average results for RA are extrapolated for Classes A1/A2 with anchor replacing missing QP points. Additionally, encoder run-time estimates for RA may be inaccurate due to use of heterogeneous CPU grid.

Table 2. BD-rate performance of the Test 2 (complexity-aware RDO enabled), comparing to ECM 14.

|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | -0.01% | -0.01% | 98.6% | 154.3% | -0.62% | -0.23% | -0.01% |  |  |
| Class A2 |  | -0.45% |  | -0.01% |  | -0.01% |  | 92.8% |  | 167.8% |  | -0.57% |  | -0.03% |  | -0.15% |  |  |  |  |  |
| Class B |  | -0.41% |  | -0.01% |  | 0.00% |  | 108.2% |  | 172.3% |  | -0.74% |  | -0.10% |  | -0.08% |  | 102.3% |  | 154.7% |  |
| Class C |  | -0.38% |  | 0.02% |  | 0.02% |  | 101.6% |  | 172.3% |  | -0.58% |  | -0.03% |  | 0.07% |  | 102.4% |  | 147.1% |  |
| Class E |  | -1.01% |  | 0.06% |  | 0.06% |  | 99.5% |  | 219.3% |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -0.51% |  | 0.01% |  | 0.01% |  | 101.0% |  | 175.3% |  | -0.64% |  | -0.10% |  | -0.04% |  | 102.3% |  | 151.3% |  |
| Class D |  | -0.57% |  | 0.08% |  | 0.08% |  | 101.2% |  | 203.9% |  | -0.70% |  | -0.03% |  | 0.12% |  | 106.1% |  | 160.8% |  |

- On Complexity-Performance tradeoff

Proposed method (test 1) provides 1.2% BD-rate Luma gain in RA with no increase in Encoder run-time, and noticeable increase of the decoder run-time (+130%). In the second test, complexity-aware RDO capable to disable application of NN filter at the slice/block level reduces decoder run-time increase to +50%, whereas still providing around 0.7% luma coding gain. This complexity-performance tradeoff may be found interesting at the exploration stage of ECM development, since it does not increase run-time of the encoder (main bottleneck of the ECM development) and multiple methods to reduce decoder run-time may be made available.

NN filter proposed for ECM in this contribution is based on VLOP architecture of JVET-AH051, which has been studied and cross-checked through EE1 joint training of NNVC. The complexity level of 5kMac/Pixel. In this proposal, complexity of the NN filter is further reduced to 4.2kMAC/pixels by restricting its operations to luma pixels only. Being combined with relatively low amount of NN parameters, 40KB of side information, this level of complexity is considered to be acceptable for exporlations and EE study in NNVC and ECM.

Proposed NN filter is expected to benefit from GPU accerators. For testing in ECM CTC, no such support was used and it was simulated on CPU grid with inference implemented through SADL inference library which is a generic C++ implementation of matrix operations and is not customly optimized for proposed method. SADL implements SIMD support for certain NN layers, while lack degree of optimizations featured by some coding tools of ECM.

Proposed filter was integrated into ECM in addition to existing filters in ECM ALF (2 fixed filters, Gaussian, Residual filtering) as well as others in-loop filters (BIF, deblocking SAO). While implementation of complexity-aware RDO shows a good degree of complexity scalability by disabling NN filter, no such operations have been performed over other filters. It is argued, that better performance-complexity tradeoff could be achieved during the EE2 study.

# Conclusion

In this contribution, NN-based In-LOP Filter derived from NNVC VLOP (JVET-AH0051) was integrated into ECM framework and its operations were harmonized with existing ECM-ALF. NN model with complexity of 4.2kMAC/pixel and 40KB of side information was trained following NNVC guidelines and tested with NNVC inference interface implemented in ECM13 and 14. Proposed method was tested with ECM CTC, and reportedly provides around 1.2% luma bd-rate gain in RA configuration.

It is proposed to study this method in EE2.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).