| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01-08 November 2024 | Document: JVET-AJ0213 |
| --- | --- |

| Title: | On Multiview Main 10 Profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S. Paluri D. Podborski A. Tourapis | Tel: Email: | {s_paluri,dpodborski, atourapis} @apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

This contribution requests the re-addition of the Multiview Main 10 profile in the HEVC specification. It is asserted that several services and encoders have already released a considerable number of multiview content labeled with the intended “Multiview Main 10” profile signaling, and that the removal of this profile has made such bitstreams not compliant to any defined profile in the HEVC standard. Re-encoding or re-labeling all these bitstreams may not be feasible or can be quite costly. Furthermore, some encoders or systems that also claim compatibility with the intended Multiview Main 10 profile might not be readily updateable.

# Introduction

The latest version of the HEVC specification introduces several new multiview capable profiles, intended among others for the delivery of stereoscopic video content. Until the 34th JVET meeting the list included two different 10 bit 4:2:0 capable profiles [1]. The first profile, which was tentatively named as the Multiview Main 10 profile, matched the capabilities of the Multiview Main profile while also adding support for 10 bits. However, the second profile, later named as the Multiview Extended 10 profile, further allowed the combination, in the same multi-layered bitstream, of layers with different format capabilities (e.g. some layers being in a 4:2:0 chroma format and others in a 4:0:0/monochrome format). This functionality was not supported by the Multiview Main profile. Different profile indicators (profile_idc values) were used to identify the two different profiles. An equivalent profile to the Multiview Extended 10 profile for 8 bit content was also being considered.

However, at that meeting, the group decided to only include the Multiview Extended 10 profile in the final specification, since that was considered as a super set, which covered all the functionalities desired by the other profile. It was felt that this decision would help limit fragmentation and better encourage the adoption of this new profile. Since the group also agreed on the inclusion of the Multiview Extended profile, and to avoid any possible confusion with the functionalities and limitations of the Multiview Main profile, the profile_idc indication of the Multiview Extended 10 profile was set to the value of 12.

Although this change was well intended, unfortunately it happened very close to the finalization of the specification, while also it was maybe not so well publicized. Several implementors of multiview applications/encoders assumed that the Multiview Main 10 profile was going to be included in the final specification and have been generating, for quite some time, bitstreams labeled as Multiview Main 10 conforming bitstreams. Similarly, some services also only claim Multiview Main 10 compatibility. Re-encoding or re-labeling such bitstreams can be quite difficult and in many cases expensive, while not all such services might be easily updateable to support Multiview Extended 10 bitstreams. Similar observations are made in [2], section 4.3.1, item 8f.

It is therefore requested that the Multiview Main 10 profile is reintroduced into the HEVC specification, with the originally intended capabilities and indications, as presented in JVET-AH0046 [1].

# Summary of changes

# TBD – see text in [1].

# References

- Y.-K. Wang, H. Liu, L. Zhang, S. Jiao, C. Hu, J. Cui, A. M. Tourapis, D. Podborski, and S. Paluri, “On MV-HEVC profiles”. JVET-AH0046, 34th JVET Meeting, Rennes, FR, 17–24 April 2024

- Y.-K. Wang, B. Bross, I. Moccagatta, C. Rosewarne, and G. J. Sullivan, “Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP”, JVET-AI1004, 35th JVET Meeting, Sapporo, JP, 12–19 July 2024

# Patent rights declaration(s)

Apple Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Or equivalently setting the 12th element of the 1D array general_profile_compatibility_flag  to a value of 1. This is the same as that of the Multiview Extended profile.