| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01-08 November 2024 | Document: JVET-AJ0215 |
| --- | --- |

| Title: | Derived chroma formats using layered HEVC (Status update) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | D. Podborski A.M. Tourapis S. Paluri J. Kim | Tel: Email: | { dpodborski, atourapis. s_paluri, jungsun_kim } @apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

The majority of the deployed decoder implementations of HEVC and even VVC are only capable of supporting 4:2:0 profiles. Although there is considerable interest in the delivery of, especially, 4:4:4 signals, e.g. for screen and gaming content among other applications, unfortunately such content cannot be natively delivered using the existing infrastructure. JVET-AI0257 introduced a new SEI message that could potentially enable the delivery of such content using the layered extensions of HEVC and 4:2:0 signals. This contribution presents a tool, named as ColorPacker, that was added in HDRTools, and can be used to support such functionality. The proponents request that this new SEI message is added in HEVC and also in the V-SEI specification, if appropriate.

# Introduction

Contribution JVET-AI0257 proposed a new SEI message that enables the carriage of higher color/chroma format images using layered 4:2:0 coding in HEVC. The proposal was inspired by the current HEIF amendment, which specifies the Colour Format Enhancement Derived Image Item. Using this image item, a reconstructed image can be formed by combining information from separate images. In particular, the luma planes from one or more input images are used to reconstruct a picture with an enhanced colour format [1].

While this process is currently done on the file format level in HEIF, the same approach can be easily supported on the elementary stream level by utilizing the layered extensions of the HEVC standard. In JVET-AI0257 a method was introduced that uses the layered extensions of HEVC to carry colour components in different layers of the HEVC bitstream in a backwards-compatible manner.

This method outlines how layered extensions of HEVC can distribute colour components across different layers within the HEVC bitstream. A new SEI message, containing the necessary metadata for the reconstruction of images or videos with enhanced colour formats, can be defined similarly to the ColourFormatEnhancement derivation in HEIF [1].

To ensure backward compatibility, layer 0 of the HEVC bitstream can carry the YCbCr 4:2:0 signal. Higher layers can carry enhancement information used to reconstruct higher capability colour formats, such as YCbCr 4:4:4 or RGB, while these additional components are still coded using traditional 4:2:0 coding. Furthermore, a new SEI message can be defined to indicate how the data from different layers can be combined for reconstruction. Key elements of the metadata include:

- Layer Identification: Identifies which layers contain the base and enhancement streams.

- Colour Format Information: Specifies the target enhanced colour format (e.g. YCbCr 4:4:4).

- Component Mapping: Describes how to combine colour components from different layers.

- Component Packing Information: Indicates if multiple components are packed in the referenced video.

The figures below show a couple of examples:

Figure 1: YCbCr 4:4:4 with YCbCr 4:2:0 in layer 0 and full chroma in layers 1 and 2.

Figure 2: YCbCr 4:4:4 with YCbCr 4:2:0 in layer 0 and both full resolution chroma information packed in layer 1.

Obviously, the second solution may impact the decoder level, as higher capability may be required to decode all the layers. Nevertheless, it can be advantageous to achieve 4:4:4 decoding if the hardware does not provide native support for 4:4:4.

# Example SEI message

This section provides an exemplary enhanced colour format SEI message. The SEI message is only provided as an example and the JVET group is welcome, if the application is of interest, to use and refine this example SEI message or define a new one completely from scratch.

| enhanced_colour_format_info( payloadSize ) { | Descriptor |
| --- | --- |
| ecfi_num_layers_minus1 | u(6) |
| ecfi_vui_layer_id | u(6) |
| for( i = 0; i < ecfi_num_layers_minus1 + 1; i++ ) { |  |
| ecfi_layer_id[ i ] | u(6) |
| ecfi_ is_packed_flag[ i ] | u(1) |
| if(ecfi_is_packed_flag)[ i ] { |  |
| ecfi_num_cols_minus1[ i ] | u(3) |
| ecfi_num_rows_minus1[ i ] | u(3) |
| ecfi_hor_guard_band_mul2[ i ] | u(8) |
| ecfi_ver_guard_band_mul2[ i ] | u(8) |
| for( j = 0; j < ecfi_num_rows_minus1[ i ] + 1; j++) { |  |
| for( k = 0; k < ecfi_num_cols_minus1[ i ] + 1; k++) { |  |
| ecfi_packed_channel_id[ i ][ j ][ k ] | u(8) |
| } |  |
| } |  |
| } |  |
| else { |  |
| ecfi_channel_id[ i ] | u(8) |
| } |  |
| } |  |
| } |  |

# Semantics

ecfi_num_layers_minus1 plus 1 specifies the number of layers in the video sequence

ecfi_vui_layer_id points to the layer_id in the current bitstream that contains the VUI information, e.g. colour_primaries, transfer_characteristics, and matrix_coefficients, of the derived video signal after the combination of all layers as specified by this SEI message

ecfi_layer_id[ i ] specifies the layer_id associated with the i-th layer that is indicated within this SEI message.

ecfi_is_packed_flag[ i ] indicates a flag that determines if layer i contains multiple components packed into a single picture. When ecfi_is_packed_flag[ i ] is set to 1, layer i contains multiple components packed into the i-th layer’s first channel, i.e. the luma channel When ecfi_is_packed_flag[ i ] is set to 0 then layer i does not contain packed components in the first channel. When components are packed in the same layer, such components all have the same horizontal and vertical resolution.

ecfi_num_cols_minus1[ i ] plus 1 specifies the number of horizontal component partitions that are present in layer i.

ecfi_num_rows_minus1[ i ] plus 1 specifies the number of vertical component partitions that are present in layer i.

ecfi_hor_guard_band_mul2[ i ] specifies if a horizontal guard band region is present between two picture channels in layer i and, if present, its size in terms of luma samples. If ecfi_hor_guard_band_mul2[ i ] is equal to 0, then no horizontal guard band is present. If ecfi_hor_guard_band_mul2[ i ] is larger than 0, then the horizontal guard band between two picture channels in layer i is equal to (ecfi_hor_guard_band_mul2[ i ] * 2) in luma samples.

ecfi_ver_guard_band_mul2[ i ] specifies if a vertical guard band region is present between two picture channels in layer i and, if present, its size in terms of luma samples. If ecfi_ver_guard_band_mul2[ i ] is equal to 0, then no vertical guard band is present. If ecfi_ver_guard_band_mul2[ i ] is larger than 0, then the vertical guard band between two picture channels in layer i is equal to (ecfi_ver_guard_band_mul2[ i ] * 2) in luma samples.

ecfi_packed_channel_id[ i ][ j ][ k ] provides the identifier for the channel associated with the j-th vertical, i-th horizontal partition in layer i.

ecfi_channel_id[ i ] or ecfi_packed_channel_id provide the identifier for the channel associated with layer i.

| ecfi_channel_id[ i ] or ecfi_packed_channel_id[ i ][ j ][ k ] | Mapping based on VUI |  |
| --- | --- | --- |
| 0 | Unused |  |
| 1 | Unspecified |  |
| 2 | Y | R |
| 3 | Cb | G |
| 4 | Cr | B |
| 5 | Alpha |  |
| 6 | Depth |  |
| 7-255 | Reserved for future use. |  |

# Implementations

A new module is introduced in HDRTools, named as ColorPacker, which can enable the desired functonality. The tool is capable of reading a single 4:4:4 chroma format video sequence, and the result will be 2 or 3 separate video sequences. The first sequence can contain a 4:2:0 representation of the signal, while in the 2 output configuration the second sequence contains an over-under packed version of the two chroma components at their full resolution placed in the luma colour plane of the ouput. In the 3 output configuration the two chroma components are placed in separate sequences instead. The inverse reconstruction is also supported by this tool.

A reference implementation of the metadata in the HM, and if desired in the VTM, will also be provided.

# Proposal

This contribution introduces a method to distribute colour components across different layers in HEVC using layered extensions. It provides examples and details necessary data for an SEI message to guide the player in processing the elementary stream. The base layer can carry the YCbCr 4:2:0 signal, while higher layers contain enhancement data for reconstructing higher capability colour formats. Key metadata elements include layer identification, colour format information, component mapping, and component packing information. We propose that JVET studies the introduction of this new method.

# References

- WD of ISO/IEC 23008-12 3rd edition AMD 2 Support for tone map derivation and others, MDS23837_WG03_N01227, Output document MPEG #146

- A. M. Tourapis, D. Podborski, S. Paluri, and J. Kim, “Derived chroma formats using layered HEVC”, JVET-AI0257, Sapporo, JP, 12-19 July 2024

# Patent rights declaration(s)

Apple Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).