| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0216-v1 |
| --- | --- |

| Title: | On interpolation filter for intra template prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuan Zhou Zhuoyi Lv | Email: | chuan.zhou@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This contribution extends the filtering methods used in block prediction to template prediction.

On top of the ECM14.0, simulation results are reported as below:

Introduction

In current ECM, for intra prediction, a 2-tap interpolation filter with [1 2 1]/4 filter is applied for reference samples, and one of three interpolation filters is utilized to generate prediction samples. The options include a 6-tap DCT-based interpolation filter (DCTIF) and two 4-tap/6-tap smoothing interpolation filters (SIF). The selection of the filter depends on the intra mode and block size. However, the single 4-tap DCTIF is used for the template prediction, and the related tools include TIMD, MPM reorder, etc.

Proposed method

The interpolation filter type is determined as follows:

If all of following conditions are true, 4-tap/6-tap SIF is used for the interpolation:

- The current block size is greater than 32

- The directional mode is non-fractional angles

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ]

Otherwise, 6-tap DCTIF is used for the interpolation.

Simulation results

The proposed method is tested on top of ECM14.0 under the common test conditions [1].

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.03% | 0.00% | -0.08% |  |  |  |  |
| Class E | -0.04% | 0.14% | 0.09% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, Jul. 2024.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).