| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0219-v1 |
| --- | --- |

| Title: | Subpicture support in HEVC (status update) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dimitri Podborski, Alexis M. Tourapis Seethal Paluri, Jungsun Kim Apple Inc. | Email: | {dpodborski, atourapis, s_paluri, jungsun_kim} @apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

Subpictures were newly introduced in the VVC standard to better enable region of interest and immersive media applications compared to motion constrained tile sets (MCTS). In MCTS, inter prediction constraints were applied to each tile defined for a coded picture so as to remove any dependencies between them. This can have a considerable impact in coding efficiency especially in scenes involving high vertical or horizontal motion. Unlike MCTS, subpicture partitions in VVC are completely independent units that utilize padding around subpicture boundaries to aleviate the MCTS inter prediction limitations. In addition, extraction of subpicture units is said to be simpler than that of MCTS since there is no need to change any bits in the VCL NAL units.

This contribution is a followup of contribution JVET-AI0256, which suggested that subpictures could also be introduced in HEVC by extending its layering functionality. In particular, one could segment a picture into multiple regions, and then code each region as an independent layer using the layering capabilities of HEVC. Metadata, e.g. a new SEI message, could be provided that would describe the relationship of each decoded layer to a final reconstructed image. This concept can also support other multilayer functionalities, such as multiview video and auxiliary data such as depth and alpha. In this contribution semantics of the introduced syntax elements are also provided.

# Introduction

Recently, immersive applications have necessitated the need for region of interest coding methods in video coding standards. Such techniques can allow services and decoders to only stream, extract, and.or decode information that only correspond to the viewing area of interest in a bitstream and discard all other information. This can reduce the burden to the network as well as to the decoder.

In HEVC [1], motion constrained tile sets (MCTS) were introduced as a mechanism for achieving region of interest (ROI) coding. The existing tiling functionality in HEVC was exploited where a coded picture could be partitioned into rectangular regions, namely tiles, and coded to some extend independently. Unfortunately, tiles in HEVC allow inter prediction to be performed across tiles, which can be an issue for true region of interest decoding applications. For this reason, MCTS imposes a series of constraints during tile encoding, so as to disallow any inter type of prediction to happen across tile boundaries. Loop filtering is also constrained. Such constraints have to be adhered by encoders through certain constraints in their motion estimation and mode decision engines, which can sometimes be challenging for some encoding designs. Regardless, although the desired outcome for independent decoding of each tiles is achieved when such constraints are followed, these constraints can also negatively impact coding efficiency especially in the presence of high motion. Metadata, in the form of supplemental enhancement information (SEI) messages were also provided to distinguish such constrained tiles and to provide the mechanisms for services and decoders to extract, adjust as needed, and reconstruct the regions of interest given a particular viewing experience. It should be also noted that given that MCTS was defined primarily using non-normative SEI messages there was no guarantee that bitstreams would fully adhere to its requirements.

To aleviate such issues, VVC [2] introduce the concept of subpictures. Unlike MCTS, subpictures normatively define regional structures in an image that are fully independent. Motion vectors can point outside a subpicture through padding, considerably improving coding efficiency. The subpicture layout is also defined in the sequence parameter set of a VVC bitstream and the handling process of all subpictures can be said to be much simpler than that of MCTS.

While looking at the characteristics of layered coding in HEVC we realized that the same functionality introduced by VVC subpictures could also be enabled with HEVC layers. Layers in HEVC currently are used to indicate scalability layers, but also multiview and auxiliary layers, such as depth and alpha. However, there is no limitation that would say that layers could not be also used to indicate a region/partition within a larger virtual image. In an example, assuming that we would like to partition a picture into 4 regions (Regions 0-3) and code them independently (Figure 1). Each one of these layers could be assigned to a subpicture and then each subpicture could be assigned to a layer_id and coded using one of HEVC’s multilayer profiles. Metadata could be defined that would allow a decoder to interpret each layer as a subpicture, and allow it to reconstruct the full or partial picture given this information.

Figure 1. Example partitioning of a picture into subpictures and layers.

In a design, ideally subpictures could be of different size, resolution, and even bitdepth, while regions could also partially or even fully overlap. Overlapping could allow seamless blending or even enable ROI SNR quality enhancement if desired. Subpictures could also be assigned with multiple types of data, i.e. a single subpicture may span multiple layer_ids, with each layer carrying different types of data, e.g. left or right views of a stereo signal, depth or alpha information. All these relationships could be easily defined in one or more (if needed) newly introduced SEI messages for HEVC. It is our belief that other than limitations in the number of layers that could be used (i.e. 64 in the case of HEVC given the range of values allowed for the syntax element nuh_layer_id in the NAL unit header syntax) and the lack of an appropriate SEI message and necessary limits, possibly including the definition of a new multi-layer profile for HEVC, HEVC should be fully capable in supporting such concepts and any applications that could benefit from their introduction.

This concept was proposed in contribution JVET-AI0256 [3]. The original contribution did not include semantics, which are now included in this document.

# Proposal

We propose that JVET studies the introduction of a new (or more if deemed needed) SEI message for the support of subpictures in HEVC using multilayered coding. Such subpictures could be used to enable a variety of region of interest applications including immersive media and region of interest enhancement applications using alread available HEVC HW that is capable of multilayered decoding. The group, if agrees in such an investigation, should also possibly study whether new profiles may be needed.

Exemplary syntax of an SEI message that could be used to indicate a set of subpictures and how these can be mapped to layers is provided in Section 3.

## Example SEI message

In this section an exemplary SEI message that allows the definition of subpictures and their association to layers is provided. This SEI message is quite flexible and allows the definition of overlapping subpictures and optional padding (e.g. to create guardbands that could help reduce seaming artifacts or for applications intending to support ROI scalability), flexible partitioning, and different forms of partitioning blending and blending prioritization identification. The SEI message is only provided as an example and the JVET group is welcome, if the application is of interest, to use and refine this example SEI message or define a new one completely from scratch.

| subpicture_info( payloadSize ) { | Descriptor |
| --- | --- |
| spi_use_case | u(6) |
| spi_single_subpicture_flag | u(1) |
| spi_padding_x | ue(v) |
| spi_padding_y | ue(v) |
| if( spi_single_subpicture_flag == 0 ) { |  |
| // First define the regions |  |
| spi_num_region_columns_minus1 | ue(v) |
| spi_num_region_rows_minus1 | ue(v) |
| spi_uniform_spacing_flag | u(1) |
| if( !spi_uniform_spacing_flag ) { |  |
| for( i = 0; i < spi_num_region_columns_minus1; i++ ) |  |
| spi_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < spi_num_region_rows_minus1; i++ ) |  |
| spi_row_height_minus1[ i ] | ue(v) |
| } |  |
| else { |  |
| spi_nominal_width_minus1 | ue(v) |
| spi_nominal_height_minus1 | ue(v) |
| } |  |
| NumRegionsInPicture = ( spi_num_region_columns_minus1 + 1) * ( spi_num_region_rows_minus1 + 1 ) |  |
| // Group regions into subpictures |  |
| spi_single_region_per_subpicture_flag | u(1) |
| if( !spi_single_region_per_subpicture_flag ) { |  |
| spi_num_subpictures_minus1 | ue(v) |
| for( i = 0; i < spi_num_subpictures_minus1+ 1; i++ ) { |  |
| spi_top_left_region_idx[ i ] | u(v) |
| spi_bottom_right_region_column_offset[ i ] | ue(v) |
| spi_bottom_right_region_row_offset[ i ] | ue(v) |
| } |  |
| } |  |
| else |  |
| spi_num_subpictures_minus1 = NumRegionsInPicture – 1 |  |
| } |  |
| else { |  |
| spi_num_subpictures_minus1 = 0 |  |
| spi_nominal_width_minus1 | ue(v) |
| spi_nominal_height_minus1 | ue(v) |
| } |  |
| // Indicate if a subpicture uses padding |  |
| if (spi_padding_x ¦¦ spi_padding_y) |  |
| for( i = 0; i < spi_num_subpictures_minus1 + 1; i++ ) |  |
| spi_padded_subpicture_flag | u(1) |
| // Label each subpicture |  |
| spi_signalled_subpicture_ids_flag | u(1) |
| if( spi_signalled_subpicture_ids_flag) { |  |
| spi_signalled_subpicture_id_length_minus1 | ue(v) |
| for( i = 0; i < spi_num_subpictures_minus1 + 1; i++ ) { |  |
| spi_subpicture_id[ i ] | u(v) |
| SubpictureIDToIndex[ spi_subpicture_id[ i ] ] = i |  |
| SubpictureIndexToID[ i ] = spi_subpicture_id[ i ] |  |
| } |  |
| } |  |
| else |  |
| for( i = 0; i < spi_num_subpictures_minus1 + 1; i++ ) { |  |
| spi_subpicture_id[ i ] = i |  |
| SubpictureIDToIndex[ i ] = i |  |
| SubpictureIndexToID [ i ] = i |  |
| } |  |
| // Specify the # of layers and assign them to a subpicture id and define their usage |  |
| spi_delta_layer_cnt | ue(v) |
| // number of layers can be same or higher than number of subpictures |  |
| LayerCnt = spi_delta_layer_cnt + spi_num_subpictures_minus1 + 1 |  |
| for( idx = 0; idx < LayerCnt; idx++ ) { |  |
| spi_layer_id[ idx ] | u(6) |
| spi_layer_subpicture_id[ idx ] | u(v) |
| spi_layer_purpose[ idx ] | u(4) |
| spi_overlay_method[ idx ] | u(4) |
| spi_priority_weight[ idx ] | u(8) |
| } |  |
| } |  |

## Semantics

spi_use_case specifies the application scenario for which the signaling is intended as defined in Table 1. It enables a decoder that supports this subpicture information SEI message to determine the intended use case of the associated bitstream. If a decoder does not satisfy the expected requirements of the intended use case, the decoder can decide to terminate the decoding process.

Table 1: Usage interpretation of the syntax element spi_use_case and the corresponding allowed values

| spi_use_case | Usage | Parameter 1 | Parameter 2 | Parameter N |
| --- | --- | --- | --- | --- |
| 0 | Unspecified | - | - | - |
| 1 | ... | ... | ... | ... |

It is a requirement of bitstream conformance that all fields appearing in Table 1 that are not indicated with a dash, shall match the values prescribed for the corresponding spi_use_case value. Subpicure information SEI message that contain invalid values should not be considered by applications and should not be processed.

spi_single_subpicture_flag when set to 1, indicates that a single subpicture with a single region is present. When set to 0, indicates that additional information is present to signal regions and subpicture mapping.

spi_padding_x and spi_padding_y specify the horizontal and vertical padding, respectively, to be applied symmetrically to all subpictures. This padding is added at the boundaries of all the edge regions in each subpicture. Care should be taken to set these values properly to ensure chroma format compatibility. For example, in the case of a 4:2:0 chroma format, the padding values should be even to avoid chroma misalignment or conformance issues. Padding is enabled on a per-subpicture basis using the spi_padded_subpicture_flag.

spi_num_region_columns_minus1 and spi_num_region_rows_minus1 specify the number of region columns and rows within a picture, each reduced by one. The values of spi_num_region_columns_minus1 + 1 and spi_num_region_rows_minus1 + 1 define the total number of vertical and horizontal divisions in the picture, respectively. These parameters facilitate the partitioning of a picture into a grid of regions, where each region may correspond to a specific subpicture or be grouped with other regions as defined by the subpicture configuration.

spi_uniform_spacing_flag when set to 1, all region columns and rows are of equal width and height, respectively. Conversely, if spi_uniform_spacing_flag is equal to 0, non-uniform spacing is allowed, and individual widths and heights for each region column and row are specified explicitly.

spi_column_width_minus1[ i ] and spi_row_height_minus1[ i ] specify the width and height, respectively, of each region column and row within the picture, each reduced by one. These elements are present only when spi_uniform_spacing_flag is equal to 0, indicating non-uniform spacing across columns and rows. The value of spi_column_width_minus1[i] + 1 defines the actual width of the i-th region column, and similarly, spi_row_height_minus1[i] + 1 defines the height of the i-th region row. These parameters allow for precise control over the dimensions of each region when flexible partitioning is required.

spi_nominal_width_minus1 and spi_nominal_height_minus1 specify the nominal width and height, respectively, for uniformly spaced region columns and rows within the picture, each reduced by one. These elements are used only when spi_uniform_spacing_flag is equal to 1, indicating that all columns and rows have uniform dimensions. The values of spi_nominal_width_minus1 + 1 and spi_nominal_height_minus1 + 1 represent the actual width and height for each region column and row. This ensures consistent, evenly spaced partitioning of the picture when uniform spacing is enabled.

spi_single_region_per_subpicture_flag when set to 1, each subpicture corresponds to exactly one region, meaning the number of subpictures is equal to the total number of regions defined by spi_num_region_columns_minus1 and spi_num_region_rows_minus1. If spi_single_region_per_subpicture_flag is equal to 0, multiple regions can be grouped together to form a single subpicture, allowing flexible grouping of regions within each subpicture.

spi_num_subpictures_minus1 plus 1 specifies the number of subpictures in the picture. The value spi_num_subpictures_minus1 + 1 gives the actual number of subpictures, each potentially comprising one or more regions.

spi_top_left_region_idx[ i ] specifies the starting region for each subpicture i, identifying the top-left region that should be included within the subpicture. This index acts as an anchor point, from which additional regions are added based on the specified offsets spi_bottom_right_region_column_offset[ i ] and spi_bottom_right_region_row_offset[ i ].

spi_bottom_right_region_column_offset[ i ] and spi_bottom_right_region_row_offset[ i ] specify the horizontal and vertical offsets, respectively, from the top-left region index to define the bottom-right boundary of subpicture i. These offsets determine the spatial extent of the subpicture by including all regions from the specified top-left position to the bottom-right corner indicated by these offsets. This configuration enables subpictures to span multiple regions, overlap with other subpictures, or cover only single regions as needed.

spi_padded_subpicture_flag when set to 1, padding values defined by spi_padding_x and spi_padding_y are applied symmetrically around the edges of the subpicture, potentially extending its boundaries beyond the original region dimensions. This flag enables flexible handling of subpictures with additional padding for alignment or blending purposes.

spi_signalled_subpicture_ids_flag when set to 1, unique IDs are provided for each subpicture via spi_subpicture_id[i]; otherwise, subpictures are assigned default IDs based on their index. This flag allows for the optional signaling of custom subpicture identifiers, facilitating easier reference and management of specific subpictures within the picture.

spi_signalled_subpicture_id_length_minus1 plus 1 specifies the length of each explicitly signaled subpicture ID. The value of spi_signalled_subpicture_id_length_minus1 + 1 gives the exact bit length of each spi_subpicture_id[ i ] when spi_signalled_subpicture_ids_flag is set to 1. This parameter allows for variable-length IDs.

spi_subpicture_id[ i ] specifies the unique identifier for subpicture i. The length of each spi_subpicture_id[i] is determined by spi_signalled_subpicture_id_length_minus1 + 1.

spi_delta_layer_cnt specifies the number of additional layers, relative to the number of subpictures, to be used within the picture. The total layer count is given by spi_delta_layer_cnt + spi_num_subpictures_minus1 + 1. This parameter allows for the inclusion of extra layers beyond the base set, supporting cases where multiple data types (e.g., auxiliary data or stereo views) are associated with subpictures.

spi_layer_id[ idx ] specifies the layer identifier for layer idx, corresponding to the nuh_layer_id field in the NAL unit header.

spi_layer_subpicture_id[ idx ] specifies the subpicture ID associated with layer idx, linking each layer to a particular subpicture.

spi_layer_purpose[ idx ] specifies the intended function or type of data contained in layer idx. TBD

spi_overlay_method[ idx ] specifies the method to be used for overlaying the content of layer idx onto ... TBD

spi_priority_weight[ idx ] specifies the priority or importance of layer idx relative to other layers. This weight can guide the decoder or renderer in determining the display or processing order, especially in cases where resources are limited, or selective decoding is required. Lower weights indicate higher priority.

# Conclusion

In this document metadata that could enable the usage of sub-pictures using the HEVC video coding standard and its layering capabilities is introduced. It is proposed to include this proposal in the current HEVC TuC.

# References

- ISO/IEC 23008-2:2023, “Information technology — High efficiency coding and media delivery in heterogeneous environments - Part 2: High efficiency video coding”

- ISO/IEC 23090-3:2022, “Information technology — Coded representation of immersive media - Part 3: Versatile video coding”

- A. M. Tourapis, D. Podborski, S. Paluri, and J. Kim, “Subpicture support in HEVC”, JVET-AI0256, 35th JVET Meeting, Sapporo, JP

# Patent rights declaration(s)

Apple Inc,. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).