| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0220-v2 |
| --- | --- |

| Title: | EE2-2.13c: Combination of tests EE2-2.11 and EE2-2.12 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vasily Rufitskiy Alexey Filippov Jacek Konieczny Hongdong Qin Keqin Ding Tianyu Dong Zhuowei Xu 7/F, Block G1, TCL International E-City 1001 Zhongshanyuan Road Nanshan, Shenzhen, Guangdong 518052 People’s Republic of China | Email: | vasily.rufitskiy@tcl.com alexey.filippov@tcl.com jacek.k@tcl.com hongdong.qin@tcl.com keqin.ding@tcl.com tianyu.dong@tcl.com zhuowei.xu@tcl.com |
| Source: | TCL |  |  |

_____________________________

# Abstract

In this document results for EE2-2.13c for the combination of test EE2-2.11 and EE2-2.12. On top of ECM-14.0, the following BD-rate gain results and encoding/decoding runtimes are reportedly obtained {Y/U/V/EncT/DecT}:

{-0.06%, -0.04%, -0.02%, 102%, 103.8%} for AI configuration, and

{x.x%, x.xx%, x.x%, xxx%, xxx%} for RA configuration.

# Introduction

## DIMD with 2x2 edge operator applied to small blocks (Test EE2-2.11)

The method was originally proposed in JVET-AI0140 [1], It is proposed to use smaller filter kernel sizes (i.e., 2x2) for blocks that comprise small number of samples (i.e. 4x4, 4x8 and 8x4) when calculating a Histogram of Gradients in the DIMD process. More details on the method could be found in JVET-AJ0203 [2].

## Non-adjacent DIMD mode derivation for TMRL intra mode candidate list construction (Test EE2-2.12)

In test EE2-2.12 it is proposed to reuse the DIMD processing in TMRL candidate list construction. For this purpose, DIMD is applied to non-adjacent reference lines, i.e. HoG is calculated in the area determined by a reference. More details on the method could be found in JVET-AJ0212 [3].

# Results

The method was implemented in the ECM-14.0[3] codebase and the test results were obtained under the common test conditions [4]. The results are reportedly given in Table 1.

Table 1. Experimental results of the proposed method

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0-gcc-8.5 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.13% | -0.11% | 101.8% | 104.2% | 91.1% | 101.1% |
| Class A2 | -0.02% | -0.07% | -0.04% | 101.6% | 104.1% | 91.3% | 101.2% |
| Class B | -0.03% | -0.04% | 0.04% | 102.8% | 104.0% | 86.3% | 101.5% |
| Class C | -0.06% | 0.00% | -0.05% | 101.0% | 103.6% | 100.2% | 102.9% |
| Class E | -0.13% | 0.02% | 0.04% | 103.2% | 103.2% | 97.9% | 102.4% |
| Overall | -0.06% | -0.04% | -0.02% | 102.1% | 103.8% | 92.8% | 101.9% |
| Class D | -0.10% | 0.07% | -0.06% | 99.8% | 104.2% | 101.2% | 103.4% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0-gcc-8.5 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | 0.03% | 0.01% | 100.8% | 101.0% | 100.6% | 100.6% |
| Class C | -0.03% | -0.12% | -0.06% | 101.1% | 100.9% | 100.5% | 100.5% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.23% | 0.19% | 100.9% | 101.2% | 100.5% | 100.4% |

# Conclusion

This contribution proposes a combination of the technologies studied in test EE2-2.11 and test EE2-2.12. It is recommended that the method will be adopted into ECM-15.0.

# References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 14 (ECM 14)”, document JVET-AI2025, Jul. 2024

- A. Filippov, V. Rufitskiy, J. Konieczny, H. Qin, T. Dong, Z. Xu, “EE2-2.11: DIMD with 2x2 edge operator applied to small blocks”, document JVET-AJ0203, Nov. 2024

- V. Rufitskiy, A. Filippov, J. Konieczny, H. Qin, “EE2-2.12: Non-adjacent DIMD mode derivation for TMRL intra mode candidate list construction”, document JVET-AJ0212, Nov. 2024

- ECM-14.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, document JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

TCL may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).