| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0222-v0 |
| --- | --- |

| Title: | Non-EE2: Additional contexts for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Zhipin Deng2, Mehdi Salehifar1, Yang Wang2, Wenbin Yin2, Na Zhang2, Lei Zhao2, Li Zhang1 18910 University Center Lane, San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, additional contexts for CABAC are proposed. The proposed context modeling method is dependent on a variety of information, including the bin index, the block size, the position, the coding mode, and the corresponding information from a neighbouring block or a cross-component block.

On top of ECM-14.0, simulation results of the proposed method are reported as below:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

# Introduction

In ECM, CABAC with context modeling is widely used to compress syntax elements efficiently. It is worthwhile to further explore the potential of CABAC by introducing more contexts.

# Proposed Method

More contexts for CABAC are proposed for some syntax elements (SEs) in this contribution. Specifically, a class index for each SE is determined based on a variety of information as below:

- Neighbouring block-based context modeling

The class indices for SEs root_cbf, cbf, transform_skip_flag, jccr_flag, non_local_ccp_flag, last_pos_x and last_pos_y are determined by the corresponding SEs signaled in the neighbouring blocks.

In one example, the class index for jccr_flag is set equal to (jccr_flag_L + jccr_flag_A), where jccr_flag_L and jccr_flag_A represent jccr_flag in the left and above neighbouring blocks, respectively.

In another example, the class index for last_pos_x is set equal to (lastNZCoefXA >= WA/2 ? 2 : ( lastNZCoefXA >= WA/4 ? 1 : 0)), where lastNZCoefXA represents the X coordinate of the last non-zero coefficient of the above neighboring block and WAis the width of the above neighbouring block. Moreover, decoded last_pos_x is remapped into last_pos_x’ if lastNZCoefXA >= 7×WA/8, formulated as last_pos_x’ = Map[last_pos_x], where Map[] = { lastNZCoefXA, lastNZCoefXA – 1, lastNZCoefXA + 1, lastNZCoefXA – 2, lastNZCoefXA + 2,…}.

- Bin index-based context modeling

The class indices for SEs ccp_type (indicating the modes of CCP, CCP-L, CCP-T, MM-CCP, MM-CCP-L, MM-CCP-T), intra_chroma_mode, cclm_delta_index and decoder_derived_ccp_index are determined by the bin index. These SEs are binarized with the truncated unary code, and each bin has its own context model.

- Block size-based context modeling

The class indices for SEs isp_mode, coef_sign_flag, decoder_derived_ccp_flag and mts_flag are determined by the block size.

For example, the class index for coef_sign_flag is set equal to ((WC > 8 || HC > 8 ) ? 1 : 0), where WC and HC represent the width and height of the current block, respectively.

- Cross-component context modeling

The class indices for SEs sig_coeff_flag, abs_level_gtx_flag and par_level_flag signaled for chroma components are determined by the corresponding luma block. The class index is set equal to ((lastNZCoefXluma <=Wluma/4 && lastNZCoefYluma <=Hluma/4)? 0 : 1), where lastNZCoefXluma and lastNZCoefYluma are the X and Y coordinates of the last non-zero coefficient of the corresponding luma block, respectively; Wluma and Hluma are the width and height of the corresponding luma block, respectively.

- Position-based context modeling

The class indices for SE sb_coded_flag are determined by the position of sb_coded_flag. The class index is set equal to ((sbX > sbW /2 || sbY > sbH/2)? 1 : 0), where (sbX, sbY) is the coordinate of the coding sb_coded_flag in terms of subblocks; sbW and sbH are the width and height of the current block in terms of subblocks, respectively.

- Coding mode-based context modeling

The class indices for SE cccm_flag are determined by ccp_type. The class index is set equal to (ccp_type == CCP ? 0 : ( ccp_type == MM-CCP ? 1 : 2)).

The class indices for SEs split_cu_flag, split_qt_flag, mtt_split_cu_vertical_flag and mtt_split_cu_binary_flag are determined by sps_qtbtt_dual_tree_intra_flag and the color component.

If a bin of a SE is bypass coded or coded with a single context in ECM-14.0, such as root_cbf and ccp_type, the class index denoted as classId is directly used as the final context index to retrieve a context model. Otherwise, i.e., if a bin of a SE is coded with multiple contexts in ECM-14.0, such as jccr_flag with three contexts in ECM, the context index derivation method in ECM is further invoked to derive the context index offset within the class, denoted as ctxIdOffset. The final context index is set equal to (classId × numOfCtx + ctxIdOffset) wherein numOfCtx represents the total number of contexts for the bin, which is 3 for jccr_flag in the example.

The total number of context models increases by about 40%, i.e. from 1608 to 2248.

# Simulation Results

The proposed method was implemented on top of the ECM-14.0 [1]. Simulations were performed following the common test conditions [2]. The simulation results are summarized in Table 1.  More detailed results could be found in the attached excel sheets.

Table 1: Coding performance over ECM-14.0

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.12% | -0.16% | -0.11% |  |  | -0.07% | -0.20% | -0.12% |  |  |
| Class C | -0.19% | -0.18% | -0.04% |  |  | -0.16% | -0.26% | 0.00% |  |  |
| Class E | -0.20% | -0.19% | 0.07% |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.17% | -0.01% | -0.08% |  |  | -0.08% | -0.65% | -0.27% |  |  |
| Class F |  |  |  |  |  | 0.06% | -0.34% | 0.00% |  |  |

# Conclusions

In this contribution, additional contexts for CABAC are proposed. The proposed method can achieve promising coding performance with negligible coding time changes. It is recommended to include the proposed method in the next round of EE2.

# Reference

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AI2017, JP, July 2024.

# Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).