| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0224-v0 |
| --- | --- |

| Title: | Non-EE2: Boundary-aware Offset Refinement for In-Loop-Filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Yang Wang1, Zhipin Deng1, Na Zhang1, Lei Zhao1, Mehdi Salehifar2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In this contribution, a method of boundary-aware offset refinement for in-loop-filters is proposed. The offset between the input and the output sample of one in-loop-filter is further refined by a non-linear function, depending on the boundary information. The proposed method is applied to Deblocking Filter (DBF) and Adaptive Loop Filter (ALF).

On top of ECM-14.0, simulation results of the proposed method are reported as below:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

LB: %, %, %, %, %.

- Introduction

In ECM-14.0, the in-loop-filters are performed in the following order: DBF, BF/SAO/CCSAO in parallel, ALF. The output sample values of one stage will be directly used as the input of the next stage without any refinement.

- Proposed Method

In this contribution, the offset between the input and the output sample of one in-loop-filter is further refined by a non-linear function, depending on whether the sample is located at a block boundary or not. The proposed method is applied to DBF and ALF.

The refined offset denoted as O' is derived from the original offset O as:

O'=sign(O)×(O-A×log2(O-K>0?O:1+B≫C))

where {A, B, C} are parameters selected from {6, 31, 5} and {5, 31, 5}, which can be signaled to the decoder in SPS.

The partitioning information is utilized to determine the boundary level for each position. The positions located at a CU or TU boundary are classified into boundary level while other positions are classified into non-boundary level. The parameter K for the boundary level and non-boundary level is different.

A flag is signaled in the slice header to achieve on/off control. If the proposed method is enabled for one slice, a parameter set index for K will be signaled in the slice header.

- Simulation Results

The proposed method was implemented on top of the ECM-14.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets.

Table 1 Performance of the Proposed Method

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | -0.02% | -0.01% |  |  |  |  |
| Class C | 0.00% | 0.01% | -0.01% |  |  |  |  |
| Class E | 0.00% | 0.00% | 0.00% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.09% | 0.00% | 0.10% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.03% | 0.10% | -0.11% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.14% | 0.27% | 0.12% |  |  |  |  |
| Class E | -0.23% | 0.72% | -0.50% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.02% | 0.63% | 0.85% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

- Conclusions

In this contribution, the offset refinement for in-loop-filters is proposed. The proposed method can achieve promising coding performance with negligible coding time changes. It is recommended to include the proposed method into the next round of EE2.

- Reference

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AI2017, JP, July 2024.

- Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).