| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0225-v1 |
| --- | --- |

| Title: | AHG3/AHG10: RDO adjustments for inter-layer predictions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mikaël Le Pendu Philippe de Lagrange Fabrice Urban | Email: | mikael.lependu@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes adjustments of the encoder selection of coding modes for dual-layer coding in order to prevent quality degradation of the enhancement layer compared to the base layer. The rate-distortion (RD) criterion is adjusted to favor the selection of inter-layer coding modes over intra-layer coding modes using a modified RD cost. Modifications are also proposed to make sure that inter-layer coding modes are included in the rate distortion optimization process.

# Introduction

During the 31st JVET meeting in Geneva, CH, an expert viewing test has been performed to verify that dual-layer coding outperforms an upscaled anchor at comparable bitrate. The result has been reported in JVET-AE0288: it has been found that at least for CorsairJoy and TallBuildings test sequences, the quality of dual-layer is lower than the upscaled anchor. Although the situation was improved in JVET-AF0186 by moving the inter-layer reference first in the reference picture lists (instead of last), visual artifacts remain in some areas of the dual-layer where the quality is still lower than in the upscaled anchor. It was also analysed in JVET-AF0186 that the artifacts correlate with the selection of a reference picture from the enhancement layer (i.e., intra-layer) instead of the base layer (i.e., inter-layer) even in smooth areas where there is no need for a resolution enhancement.

A possible explanation is that the signaling cost R of an inter-layer coding mode (i.e., using the inter-layer reference picture) may be higher than that of an intra-layer coding mode in some cases. This may result in the inter-layer coding mode not being selected even if it yields the lowest distortion D, as it may have a too high rate-distortion (RD) cost C=D+lambda*R. A quality degradation compared to the upscaled base layer may thus be observed in such a case.

# Proposal

In this contribution, it is proposed to adapt the encoder selection criterion for inter-layer coding modes to prevent quality degradation of the enhancement layer compared to the base layer. The following modifications are proposed:

- When an inter-layer coding mode is compared to an intra-layer coding mode, a modified RD cost C’=D+lambda’*R is computed for both modes where lambda’ is smaller than the original lambda. If the inter-layer mode has a lower modified RD cost than the intra-layer mode, the inter-layer mode is selected. Otherwise, the regular criterion based on the original RD cost C is used. A split mode (e.g., QT, BT, TT) is considered as an inter-layer mode if at least one of its split sub-blocks use an inter-layer mode.

- When selecting the best merge candidate, a fast evaluation pass pre-selects some of the candidates based on the SATD of the prediction to accelerate the search. It is proposed to force the pre-selection of at least one of the merge candidates that performs inter-layer prediction.

- A heuristic that may prevent the search of splits modes of a block is deactivated if the current best coding mode selected for the non-split block is not an inter-layer mode.

# Experimental results

The proposed modifications are implemented and tested in VTM 23.4 using the test conditions in JVET-AF0186. A new encoder configuration flag --EncInterLayerOpt is added to activate the proposed modifications (set to false by default). The flag has no effect for encoding the base layer.

The modified Lagrangian parameter lambda’ is controlled by a modifier (lambda’= lambda * modifier) where the modifier can be set with a new configuration parameter --EncInterLayerOptLambdaModifier (set to 0.1 by default). In the experiments the default value of 0.1 is used for the modifier.

# Conclusion

Modifications of the VTM encoder are proposed to address visual artifacts of the enhancement layer in dual-layer coding.

It is proposed to merge the changes in the VTM.

In addition, it is suggested to run expert viewing tests with the proposed modifications.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).