| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0226-v1 |
| --- | --- |

| Title: | AHG12/AHG10: Using CABAC cost for MTT split modes selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Waqas Ahmad, Per Wennersten, Kenneth Andersson | Tel: Email: | +46761046057 {waqas.ahmad, per.wennersten, kenneth.r.andersson} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents a modification of the MTT split modes early termination method in ECM/VTM. The cost of coding the CU without splitting the CU at all is compared to a threshold based on the QP value and the CABAC cost for coding a certain MTT split mode flag. If the cost is higher than the threshold, the corresponding MTT split mode will not be considered, which is claimed to reduce encoding runtime.

The proposal is implemented on top of ECM 14.0 software and compared to ECM-14.0, and the average BDR impact and runtimes are reported as follows:

Test 1: (Proposed condition is applied to 64x64 & 32x32CUs)

Y  0.03%, U 0.08%, V 0.06%, EncTime 95.6%, DecTime 99.9% for the RA configuration,

Y 0.08%, U 0.06%, V -0.06%, EncTime 96.0%, DecTime 100.1% for the LDB configuration,

The runtime reduction for QP-22 is 86.1 % and 86.6% for RA and LDB configuration.

The proposal is also implemented on top of VTM-23.4 software and compared to VTM-23.4, the average BDR impact and runtimes are reported as follows:

Test 1: (Proposed condition is applied to 64x64 & 32x32CUs)

Y 0.06%, U 0.09%, V 0.04%, EncTime 94.3%, DecTime 99.9% for the RA configuration,

Y 0.12%, U -0.01%, V 0.18%, EncTime 94.1%, DecTime 99.9% for the LDB configuration,

The runtime reduction for QP-22 is 83.1 % and 83.7% for RA and LDB configuration.

# Introduction

ECM supports QT with nested MTT partitioning structures. The MTT structure contains BTH, BTV, TTH, and TTV splitting. The QT and MTT splitting structures in ECM provide support for rectangular CUs as well as square CUs, which allows a better fit to local picture content characteristics and thus improves coding gain. With the variety of partitioning structures in ECM, on the one hand, coding gain is improved, but on the other hand, it increases the encoding time.

# Proposed method

In ECM-14 [1], MTT split modes early termination is performed when the no-split cost of 64x64 luma CU becomes greater than a specified threshold with an assumption that QT splitting would be suitable for such CU. The no-split cost represents the minimum rate-distortion cost found without splitting the CU with either QT or MTT splitting structure.

CU MTT split mode flags are coded using CABAC which achieves good compression performance by selecting probability models for each syntax element according to the element’s context and adapting probability estimates based on local statistics. In general, a large CABAC cost represents a MTT split flag having low probability and a small CABAC cost indicates high probability for that MTT split mode.

In this contribution we propose to modify the threshold of the MTT early termination method [2] so that in addition to QP it also uses the CABAC cost of signaling a MTT split mode. In this way the MTT split modes that are unlikely to be chosen might not be evaluated. The method is applied to 32x32 CUs as well along with 64x64 CUs.

In the proposed method, the threshold is set based on QP value and CABAC cost of coding a MTT split mode flag as follows:

TQP=140-SQP-22×3×(106-M*QCABAC)

MBQP=4                               if BQP<224*0.5BQP - 22 5               if  22 ≥  BQP≤37 0                                if BQP>37

Here,

TQP represents the threshold value.

SQP represents value of a Slice QP.

BQP represents value of a base QP.

M represents the strength applied to the CABAC cost. It is set to 4, 2, 1, 0.5 for QP 22, 27, 32, 37.

QCABAC represents the CABAC cost to code the MTT split mode flag.

# Results

The proposed condition is demonstrated on ECM-14 [1] and VTM-23.4 [4] by following the common test conditions of ECM [3] and VTM [5]. Two tests are performed, in the first test the condition is applied to 64x64 and 32x32 CUs and in the second test the condition is applied to 64x64 CUs.

The encoding and decoding run times are estimated by running the proposed and anchor scheme on the same machine in parallel.

## ECM

An encoding time reduction is obtained for RA and LDB configuration with limited degradation in coding efficiency.

### Test 1

In Test 1, the proposed condition is applied to 64x64 & 32x32 CU sizes. All classes with this test have a decrease of encoding time for RA and LDB configurations.

Table 1 Coding performances of the TEST 1 on top of ECM-14.0 for RA configuration.

|  |  |  | Random Access Main 10 Over ECM-14.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V | EncT | EncT(Q22) | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.03% | 0.04% |  | 0.03% | 97.2% | 90.7% | 100.5% | 99.9% | 100.2% |
| Class A2 | 0.07% | 0.07% |  | -0.08% | 95.6% | 84.5% | 99.9% | 99.7% | 100.0% |
| Class B | 0.01% | 0.07% |  | 0.10% | 95.6% | 86.6% | 99.8% | 100.1% | 99.9% |
| Class C | 0.03% | 0.13% |  | 0.12% | 94.5% | 83.4% | 99.3% | 100.4% | 99.9% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.03% | 0.08% |  | 0.06% | 95.6% | 86.1% | 99.9% | 100.1% | 100.0% |
| Class D | 0.04% | 0.00% |  | -0.05% | 95.3% | 85.5% | 99.2% | 99.8% | 100.0% |
| Class F |  |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |  |

Table 2 Coding performances of the TEST 1 on top of ECM-14.0 for LDB configuration.

|  |  |  | Low delay B Main 10 Over ECM-14.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V | EncT | EncT(Q22) | DecT | EncVmPeak | DecVmPeak |
| Class B | 0.10% | 0.18% |  | -0.20% | 96.5% | 89.0% | 100.3% | 100.0% | 100.1% |
| Class C | 0.05% | 0.00% |  | 0.14% | 94.8% | 83.5% | 100.2% | 100.0% | 100.0% |
| Class E | 0.08% | -0.05% |  | -0.08% | 96.6% | 86.7% | 99.8% | 100.1% | 100.2% |
| Overall | 0.08% | 0.06% |  | -0.06% | 96.0% | 86.6% | 100.1% | 100.0% | 100.1% |
| Class D | 0.05% | 0.51% |  | 0.34% | 95.9% | 86.8% | 100.4% | 99.7% | 100.0% |
| Class F |  |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |  |

### Test 2

In Test 2 the proposed condition is applied to 64x64 CU sizes. All classes with this test have a decrease of encoding time for RA and LDB configurations.

Table 3 Coding performances of the TEST 2 on top of ECM-14.0 for RA configuration.

|  |  |  | Random Access Main 10 Over ECM-14.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V | EncT | EncT(Q22) | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.02% | 0.03% |  | 0.01% | 98.0% | 93.2% | 99.7% | 100.1% | 100.1% |
| Class A2 | 0.05% | 0.05% |  | -0.08% | 97.1% | 89.7% | 99.9% | 100.3% | 100.5% |
| Class B | -0.01% | 0.00% |  | 0.06% | 97.1% | 91.1% | 100.1% | 100.1% | 100.0% |
| Class C | 0.02% | 0.02% |  | 0.16% | 98.2% | 95.4% | 100.2% | 100.4% | 100.0% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.02% | 0.02% |  | 0.05% | 97.6% | 92.4% | 100.0% | 100.2% | 100.1% |
| Class D | 0.01% | 0.14% |  | 0.02% | 99.1% | 97.3% | 100.8% | 100.1% | 100.1% |
| Class F | 0.02% | 0.01% |  | 0.00% | 98.4% | 95.5% | 99.6% | 99.8% | 99.7% |
| Class TGM | 0.01% | 0.07% |  | 0.00% | 99.4% | 99.1% | 100.4% | 100.0% | 99.8% |

Table 4 Coding performances of the TEST 2 on top of ECM-14.0 for LDB configuration.

|  |  |  | Low delay B Main 10 Over ECM-14.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V | EncT | EncT(Q22) | DecT | EncVmPeak | DecVmPeak |
| Class B | 0.08% | 0.00% |  | 0.01% | 97.2% | 91.2% | 100.6% | 100.0% | 100.0% |
| Class C | 0.04% | -0.02% |  | 0.21% | 98.5% | 95.6% | 100.1% | 100.0% | 100.1% |
| Class E | 0.05% | 0.03% |  | -0.03% | 97.0% | 87.7% | 100.0% | 99.9% | 100.1% |
| Overall | 0.06% | 0.00% |  | 0.07% | 97.6% | 91.7% | 100.3% | 100.0% | 100.0% |
| Class D | 0.02% | 0.77% |  | 0.37% | 99.3% | 98.7% | 99.2% | 99.9% | 100.0% |
| Class F | 0.10% | -0.26% |  | -0.15% | 98.9% | 97.4% | 99.4% | 99.3% | 100.1% |
| Class TGM | -0.04% | 0.11% |  | 0.10% | 99.2% | 99.0% | 99.1% | 100.0% | 100.3% |

## VTM

An encoding time reduction is obtained for RA and LDB configuration with limited degradation in coding efficiency.

### Test 1

In Test 1, the proposed condition is applied to 64x64 & 32x32 CU sizes. All classes with this test have a decrease of encoding time for RA and LDB configurations.

Table 5 Coding performances of the TEST 1 on top of VTM-23.4 for RA configuration.

|  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 23.4 |  |  |  |  |
|  | Y | U | V | EncT | EncT(Q22) | DecT |
| Class A1 | 0.05% | 0.04% | 0.01% | 95.4% | 86.2% | 100.3% |
| Class A2 | 0.10% | 0.05% | 0.07% | 92.8% | 78.1% | 99.8% |
| Class B | 0.05% | 0.13% | 0.06% | 94.7% | 83.8% | 99.3% |
| Class C | 0.05% | 0.10% | -0.01% | 94.1% | 83.9% | 100.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.06% | 0.09% | 0.04% | 94.3% | 83.1% | 99.9% |
| Class D | 0.06% | -0.04% | 0.09% | 95.3% | 86.7% | 100.6% |
| Class F | 0.07% | 0.11% | 0.06% | 96.5% | 89.8% | 100.7% |

Table 6 Coding performances of the TEST 1 on top of VTM-23.4 for LDB configuration.

|  |  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 23.4 |  |  |  |  |
|  | Y | U | V | EncT | EncT(Q22) | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.08% | -0.14% | 0.42% | 93.6% | 82.4% | 100.1% |
| Class C | 0.16% | 0.01% | -0.17% | 93.4% | 83.7% | 99.4% |
| Class E | 0.13% | 0.18% | 0.26% | 95.9% | 86.0% | 100.4% |
| Overall | 0.12% | -0.01% | 0.18% | 94.1% | 83.7% | 99.9% |
| Class D | 0.03% | 0.33% | 0.29% | 94.8% | 84.8% | 100.5% |
| Class F | 0.05% | 0.28% | -0.10% | 95.7% | 88.8% | 99.1% |

### Test 2

In Test 2 the proposed condition is applied to 64x64 CU sizes. All classes with this test have a decrease of encoding time for RA and LDB configurations.

Table 7 Coding performances of the TEST 2 on top of VTM-23.4 for RA configuration.

|  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 23.4 |  |  |  |  |
|  | Y | U | V | EncT | EncT(Q22) | DecT |
| Class A1 | 0.04% | 0.05% | 0.01% | 97% | 89.6% | 100.2% |
| Class A2 | 0.09% | 0.02% | 0.07% | 95% | 82.9% | 100.6% |
| Class B | 0.04% | 0.11% | 0.01% | 95.9% | 87.5% | 99.1% |
| Class C | 0.02% | -0.01% | 0.02% | 96.9% | 93.5% | 100.9% |
| Class E |  |  |  |  |  |  |
| Overall | 0.04% | 0.05% | 0.02% | 96.1% | 88.5% | 100.1% |
| Class D | 0.00% | -0.04% | 0.01% | 98.8% | 98.1% | 100.4% |
| Class F | 0.02% | 0.08% | 0.05% | 97.9% | 94.2% | 100.3% |

Table 8 Coding performances of the TEST 2 on top of VTM-23.4 for LDB configuration.

|  |  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 23.4 |  |  |  |  |
|  | Y | U | V | EncT | EncT(Q22) | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.07% | -0.06% | 0.36% | 95.6% | 86.5% | 100.3% |
| Class C | 0.10% | 0.08% | -0.01% | 97.5% | 94.2% | 100.0% |
| Class E | 0.13% | 0.25% | 0.20% | 96.1% | 86.3% | 100.0% |
| Overall | 0.09% | 0.06% | 0.19% | 96.3% | 88.9% | 100.1% |
| Class D | -0.05% | 0.47% | 0.27% | 99.4% | 98.4% | 100.1% |
| Class F | -0.01% | 0.26% | -0.45% | 97.6% | 93.3% | 99.9% |

# Conclusion

In this contribution, a modification of the MTT split modes early termination scheme is proposed. In addition to the no-split cost of the CU, the method also uses the CABAC cost of coding a MTT split mode flag to determine whether to evaluate a corresponding MTT splitting for the current CU or skip that MTT split mode.

It has been shown that the proposed MTT splitting early termination scheme reduces encoding time with limited coding loss.

The proposed condition is tested with two settings. In first setting the condition is applied on 32x32 and 64x64 CUs and in second setting the condition is only applied to 64x64 CUs.  Each test shows encoding time reduction with limited loss in coding efficiency. The encoding time reduction is significant for QP-22.

# References

- ECM-14.0 reference software: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-14.0

- W. Ahmad, P. Wennersten, K. Andersson, " AhG10: MTT split modes early termination", JVET- AF0111, October 2023

- M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AI2017, Sapporo, July 2024.

- VTM-23.4, reference software:  https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-23.4

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-AB2010, Mainz, January 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).