| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0229-v1 |
| --- | --- |

| Title: | EE2-related: Non-adjacent DIMD fusion modes for TMRL intra mode candidate list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vasily Rufitskiy Alexey Filippov Jacek Konieczny Hongdong Qin Tianyu Dong Keqin Ding 7/F, Block G1, TCL International E-City 1001 Zhongshanyuan Road Nanshan, Shenzhen, Guangdong 518052 People’s Republic of China | Email: | vasily.rufitskiy@tcl.com alexey.filippov@tcl.com jacek.k@tcl.com hongdong.qin@tcl.com tianyu.dong@tcl.com keqin.ding@tcl.com |
| Source: | TCL |  |  |

_____________________________

# Abstract

In this document a method where non-adjacent DIMD fusion modes are added to the TMRL intra mode candidate list. On top of ECM-14.0, the following BD-rate gain results and encoding/decoding runtimes are reportedly obtained {Y/U/V}: {-0.07%, -0.03%, 0.01%} for AI configuration.

# Introduction

## DIMD with 2x2 edge operator applied to small blocks (Test EE2-2.11)

The method was originally proposed in JVET-AI0140 [1], It is proposed to use smaller filter kernel sizes (i.e., 2x2) for blocks that comprise small number of samples (i.e. 4x4, 4x8 and 8x4) when calculating a Histogram of Gradients in the DIMD process. More details on the method could be found in JVET-AJ0203 [2].

## Non-adjacent DIMD mode derivation for TMRL intra mode candidate list construction (Test EE2-2.12)

In test EE2-2.12 it is proposed to reuse the DIMD processing in TMRL candidate list construction. For this purpose, DIMD is applied to non-adjacent reference lines, i.e. HoG is calculated in the area determined by a reference. Intra prediction modes obtained using the HoG are used to get the list of intra prediction modes for TMRL lines. More details on the method could be found in JVET-AJ0212 [3].

# Proposed method

In this method in addition to directional (non-fusion) intra prediction modes obtained using the HoG to get the list of intra prediction modes for TMRL lines, a candidate with non-adjacent DIMD fusion enabled is also added to the list of intra prediction modes that is specified for a TMRL line.

Non-adjacent DIMD HoG derivation process is performed using 2x2 edge operators (as described in [3]). DIMD HoG derivation process is performed as described in JVET-AJ0203  [2] (see Figure 1).

Figure 1. Non-adjacent DIMD HoG derivation process is performed using 2x2 edge operators for a reference line

In TMRL, a list of intra prediction modes is constructed using a mechanism of MPM list construction. In this proposal, DC mode is not included into the list of intra prediction modes that is being checked for TMRL reference lines. As shown in Figure 2, each TMRL candidate added to a TMRL candidate list is a pair of

- an intra-prediction mode (or a set of modes if DIMD fusion is used in the case of non-adjacent DIMD applied to TMRL), and

- a non-adjacent reference line (reference line index).

Figure 2. TMRL candidate list comprising non-adjacent DIMD fusion candidates

To compare template matching (TM) costs (which is estimated by using the SAD metric in TMRL as implemented in ECM-14.0), each non-adjacent reference line added to the TMRL candidate list is projected onto the adjacent reference line of a current block according to the angular mode direction of a TMRL. If DIMD fusion is enabled for a TMRL candidate, a projection of each mode is obtained and, then, these projections are combined according to the weights assigned to each angular mode belonging to this set.

Both mechanisms are used to obtain weights from HoG: DIMD-based (according to the non-adjacent DIMD process) and TIMD-based, when weights are computed as the values, which are inversely proportional to the TM costs calculated for each individual angular mode (as done in TIMD).

# Results

The method was implemented in the ECM-14.0[2] codebase and the test results were obtained under the common test conditions [6]. The results are reportedly given in Table 1.

Table 1. Experimental results of the proposed method

|  | All Intra Main 10 |  |  |
| --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |
|  | Y | U | V |
| Class A1 | -0.07% | -0.11% | -0.08% |
| Class A2 | -0.03% | 0.00% | -0.04% |
| Class B | -0.03% | -0.06% | 0.05% |
| Class C | -0.08% | -0.02% | -0.08% |
| Class E | -0.14% | 0.05% | 0.23% |
| Overall | -0.07% | -0.03% | 0.01% |
| Class D | -0.07% | -0.14% | -0.22% |

* 4 RD points for Class A2 were taken from the ECM-14.0 anchor

# Conclusion

In this contribution, a method of non-adjacent DIMD fusion modes for TMRL intra mode candidate list construction is proposed. We recommended the proposed method is included in the next round of EE2.

# References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 14 (ECM 14)”, document JVET-AI2025, Jul. 2024

- A. Filippov, V. Rufitskiy, J. Konieczny, H. Qin, T. Dong, Z. Xu, “EE2-2.11: DIMD with 2x2 edge operator applied to small blocks”, document JVET-AJ0203, Nov. 2024

- V. Rufitskiy, A. Filippov, J. Konieczny, H. Qin, “EE2-2.12: Non-adjacent DIMD mode derivation for TMRL intra mode candidate list construction”, document JVET-AJ0212, Nov. 2024

- V. Rufitskiy, A. Filippov, J. Konieczny, H. Qin, K. Ding, T. Dong, Z. Xu, ” EE2-2.13c: Combination of tests EE2-2.11 and EE2-2.12” , document JVET-AJ0220, Nov. 2024

- ECM-14.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, document JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

TCL may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).