| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0230-v1 |
| --- | --- |

| Title: | Non-EE2: On reference sample processing for angular intra-prediction modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov Vasily Rufitskiy Tianyu Dong Jacek Konieczny Hongdong Qin 7/F, Block G1, TCL International E-City 1001 Zhongshanyuan Road Nanshan, Shenzhen, Guangdong 518052 People’s Republic of China | Email: | alexey.filippov@tcl.com vasily.rufitskiy@tcl.com tianyu.dong@tcl.com jacek.k@tcl.com hongdong.qin@tcl.com |
| Source: | TCL |  |  |

_____________________________

# Abstract

In this document, a method of reference sample processing for angular intra-prediction modes is proposed to reveal what regions of the reference samples have significant values of gradient magnitudes and gradient directionality similar to the angular prediction directions applied to a given block. The derived information on gradient magnitudes and directionalities is used to make a decision on whether to propagate into the block the difference between the samples belonging to the 2 reference lines used for the angular prediction of a given sample within the block. Thus, the proposed mechanism can be considered an extension of gradient PDPC, which is currently not available for negative angular modes. On top of ECM-14.0, the following BD-rate gain results are reportedly obtained {Y/U/V }: {-0.03%, 0.05%, 0.00%} for AI configuration.

# Introduction

## Gradient PDPC in ECM-14.0

In VVC, for a few scenarios, PDPC may not be applied due to the unavailability of the secondary reference samples. In these cases, a gradient based PDPC, extended from horizontal/vertical mode, is applied (JVET-Q0391). The PDPC weights (wT / wL) and nScale parameter for determining the decay in PDPC weights with respect to the distance from left/top boundary are set equal to corresponding parameters in horizontal/vertical mode, respectively. When the secondary reference sample is at a fractional sample position, bilinear interpolation is applied [1].

## DIMD with 2x2 edge operator applied to small blocks (Test EE2-2.11)

The method was originally proposed in JVET-AI0140 [2] to apply smaller filter kernel sizes (i.e., 2x2) to blocks that comprise small number of samples (i.e. 4x4, 4x8 and 8x4) when calculating a Histogram of Gradients in the DIMD process. More details on the method could be found in JVET-AJ0203 [3].

## Intra Prediction fusion

This intra prediction method derives predicted samples as a weighted combination of multiple predictors generated from different reference lines. In this process multiple intra predictors are generated and then fused by weighted averaging. The process of deriving the predictors to be used in the fusion process is described as follows:

- For angular intra prediction modes including the single mode case of TIMD and DIMD, the proposed method derives intra prediction by weighting intra predictions obtained from multiple reference lines represented as pfusion=w0pline+w1pline+1, where pline is the intra prediction from the default reference line and pline+1 is the prediction from the line above the default reference line. The weights are set as w0=3/4 and w1=1/4.

- For TIMD mode with blending, pline is used for the first mode (w0=1,w1=0) and pline+1 is used for the second mode (w0=0, w1=1).

- For DIMD mode with blending, the number of predictors selected for a weighted average is increased from 3 to 6.

The angular intra prediction fusion method is applied to luma blocks when angular intra mode has non-integer slope (required reference samples interpolation) and the block size is greater than 16, it is used with MRL and not applied for ISP coded blocks. In the method studied in the sub-test a, PDPC is applied for the intra prediction mode using the closest to the current block reference line. [1]

# Proposed method

In this method, reference samples are processed by the 2x2 edge operator [2-3] to derive the magnitude and the directionality associated with each region of the reference samples as shown in Fig. 1. This processing is needed to detect such regions, which have

- significant values (more than a threshold defined based on the statistics collected within the reference samples such as the actual sample range and HoG) of gradient magnitudes and

- the directionality close to the angular direction used for predicting a given block (see Fig. 2).

Applying both aspects allows us to find such regions, where gradient parameters are consistent with the prediction direction. Then, as shown in Fig. 2, the difference ΔG between the sample values taken from the 2 neighboring lines involved in the angular prediction process is computed and propagated into the blocks using the gradient PDPC rules:

- for vertical directions (i.e. directions with the mode index greater than or equal to 34 for the normal angular mode precision and 66 for the extended angular mode precision specified in TIMD and TMRL) predicted sample is determined as:

p(x,y)=px,y+fy⋅ΔG,

where ΔG is a gradient magnitude determined for the reference samples that are used to intra-predict a sample at position (x,y); and fy is a decay function which could be defined, e.g. as:

fy=1/dy,

where d is the base, which defines how fast the propagated value decays to 0 with increasing of y. This parameter d equals 2 in the experiments described below.

-similarly, for horizontal directions (i.e. directions with the mode index less than 34 for the normal angular mode precision and 66 for the extended angular mode precision specified in TIMD and TMRL):

p(x,y)=px,y+fx⋅ΔG,

and

fx=1/dx,

where d is the base, which defines how fast the propagated value decays to 0 with increasing of x.

Figure 1: Proposed processing to detect the regions of reference samples, which have significant values of gradient magnitudes and gradient directionality similar to the angular prediction directions applied to a given block.

Figure 2: Comparison of the two directions, i.e. (1) the direction derived by the edge operator and         (2) angular direction to predict the given block.

# Results

The method was implemented in the ECM-14.0[2] codebase and the test results were obtained under the common test conditions [5]. The results are reportedly given in Table 1.

Table 1. Experimental results of the proposed method

|  | All Intra Main 10 |  |  |
| --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |
|  | Y | U | V |
| Class A1 | -0.04% | 0.08% | -0.06% |
| Class A2 | -0.01% | -0.06% | -0.07% |
| Class B | -0.04% | 0.06% | -0.01% |
| Class C | 0.00% | 0.00% | -0.04% |
| Class E | -0.03% | 0.17% | 0.20% |
| Overall | -0.03% | 0.05% | 0.00% |
| Class D | 0.00% | 0.03% | -0.09% |

* 2 RD points for Class A2 were taken from the ECM-14.0 anchor.

# Conclusion

In this contribution, a method of reference sample processing for angular intra-prediction modes is proposed to reveal what regions of the reference samples have significant values of gradient magnitudes and gradient directionality similar to the angular prediction directions applied to a given block. This information is used to enable for negative angular modes a mechanism similar to the gradient PDPC.

We recommend the proposed method is included in the next round of EE2.

# References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 14 (ECM 14)”, document JVET-AI2025, Jul. 2024

- A. Filippov, V. Rufitskiy, J. Konieczny, H. Qin, T. Dong, Z. Xu, “Non-EE2: On DIMD edge operators”, document JVET-AI0140, Jul. 2024

- A. Filippov, V. Rufitskiy, J. Konieczny, H. Qin, T. Dong, Z. Xu, “EE2-2.11: DIMD with 2x2 edge operator applied to small blocks”, document JVET-AJ0203, Nov. 2024

- ECM-14.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, document JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

TCL may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).