| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0231-v1 |
| --- | --- |

| Title: | EE2-5.5: Reuse of ALF control information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Hongtao Wang, Nan Hu, Vadim Seregin 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | martak@qti.qualcomm.com hongtaow@qti.qualcomm.com nanh@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM-14.0, for each picture, adaptive loop filter (ALF) control information is derived and signalled at both picture and CTB levels. In test EE2-5.5, reuse of ALF control information is proposed. In test EE2-5.5a, ALF coefficients and control information are derived jointly for multiple pictures and the control information is signalled for each picture as in ECM-14.0. On top of EE2-5.5a, in test EE2-5.5b, for each picture, when ALF is enabled, a flag is signalled to indicate whether ALF control information reuses the information from a previously encoded picture. On top of ECM-14.0 following common test conditions, simulation results of the proposed method are reported below:

EE2-5.5a:

AI: %, %, %, % EncT, % DecT.

RA: %, %, %, % EncT, % DecT.

LB: %, %, %, % EncT, % DecT.

EE2-5.5b:

AI: %, %, %, % EncT, % DecT.

RA: %, %, %, % EncT, % DecT.

LB: %, %, %, % EncT, % DecT.

# Introduction

In ECM-14.0, a picture may use ALF derived from the current or previously encoded pictures. Filter usage information and on/off flags are signalled at both picture and coding tree block level.

# Proposed method

In test EE2-5.5, ALF control information, including on/off decision and ALF APS usage information, from a previously encoded picture is reused. To indicate this mode, a flag is signalled if ALF is enabled for a picture, ALF control information is reused and not signalled.

At encoder side, ALF filter is derived for a group of pictures jointly which share the ALF on/off decision. In the current implementation, the proposed method is only applied to non-reference pictures. Only ALF optimization of this group of pictures is postponed until the ALF encoding process of the last picture in the group is done. The bit streams between the first and last pictures are not written out until the joint ALF optimization of the group is done. Under common test conditions defined in JVET-AI2017, in random access, a group contains at most 8 non-reference pictures and the longest delay is 11 frames at encoder side. There is no delay at decoder side.

In EE2-5.5a, encoder-only part is tested. In this test, ALF coefficients and control information are derived for a group of pictures and signalled for each frame. The signalling is the same as ECM-14.0. On top of EE2-5.5a, in test EE2-5.5b, a flag is signalled if ALF is enabled for a picture and ALF control information from a previously coded picture is reused and not signalled for the current picture.

# Simulation results

The proposed method is implemented on top of ECM-14.0. The simulations were performed under common test conditions defined in JVET-AI2017. The simulation results are reported in the tables below.

Table 1. Results of test EE2-5.5a.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99.3% | 100.3% | 98.5% | 97.1% |
| Class C | 0.00% | 0.00% | 0.00% | 100.0% | 99.7% | 100.1% | 99.3% |
| Class E | 0.00% | 0.00% | 0.00% | 100.3% | 100.7% | 100.9% | 98.5% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 99.4% | 99.9% | 100.1% | 99.7% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.05% | -0.22% | -0.12% | 99.5% | 102.7% | 106.9% | 98.3% |
| Class C | -0.06% | -0.15% | -0.08% | 98.8% | 102.9% | 109.0% | 99.3% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.16% | -0.12% | -0.24% | 98.3% | 102.1% | 103.4% | 99.7% |
| Class F | 0.01% | 0.04% | 0.02% | 98.8% | 102.3% | 110.8% | 98.6% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E | 0.00% | 0.00% | 0.00% | 99.6% | 100.0% | 99.7% | 98.4% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 100.1% | 100.4% | 99.9% | 99.7% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 2. Results of test EE2-5.5b.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100.0% | 99.9% | 98.9% | 97.2% |
| Class C | 0.01% | 0.00% | 0.00% | 100.1% | 100.2% | 99.6% | 99.3% |
| Class E | 0.00% | 0.00% | 0.00% | 99.9% | 99.3% | 101.0% | 98.5% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 100.4% | 100.2% | 99.9% | 99.7% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.13% | -0.75% | -0.69% | 99.4% | 100.0% | 106.4% | 98.0% |
| Class C | -0.13% | -0.78% | -0.86% | 99.1% | 102.1% | 109.4% | 99.4% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.31% | -0.72% | -0.81% | 98.2% | 101.3% | 103.7% | 99.8% |
| Class F | -0.04% | -0.51% | -0.59% | 98.2% | 102.6% | 111.0% | 98.5% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.01% | 0.01% | 0.01% | 100.9% | 99.8% | 99.9% | 99.0% |
| Class E | 0.01% | 0.01% | 0.01% | 101.1% | 100.5% | 99.8% | 98.3% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | 0.01% | 0.01% | 100.3% | 99.3% | 99.9% | 99.7% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).