| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ232-v2 |
| --- | --- |

| Title: | AHG8: Combination tests with postfilter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Lukasz Litwic, Kenneth Andersson, Per Wennersten and Jacob Ström | Tel: Email: | {christopher.hollmann, lukasz.litwic, kenneth.r.andersson, per.wennersten, jacob.strom} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this contribution results for tests combining two different tools with the neural network postfilter originally proposed in JVET-AG0212 are reported. These two tools are the region of interest-based adaptive QP method from JVET-AB0275 and the reduced resolution coding from JVET-AH0130. It is claimed that the results indicate that in most cases the tools work orthogonal to each other, and the combination provides a benefit over using just one of them.

# Introduction

JVET has over the last years been working on a technical report (TR) on the topic of coding video for machine consumption. The latest version is available in JVET-AI2030 [1]. One topic that has been discussed several times over the course of the preparation of the report is the combination of different technologies that are described in the report.

Various contributions have presented results of combined methods, such as:

- JVET-AB0275 [2]: adaptive QP and increased QP offsets for temporal layers

- JVET-AE0081 [3]: de-noising filter and increased QP offsets for temporal layers

- JVET-AH0157 [4]: pre-processing and neural network postfilter

- JVET-AI0144 [5]: conversion to 4:4:4 and reduced resolution coding

In this contribution, the neural network-based postfilter (NNPF) that has been proposed for inclusion in the TR firstly in JVET-AG0212 [6] has been tested in combination with two other tools: Region of Interest-based adaptive QP (ROIaQP) and reduced resolution coding (RRC). The former method has been described in JVET-AB0275 [2], whereas results for the latter one were presented in JVET-AH0130 [7].

# Results for combination tests

## ROIaQP and NNPF

In the first test we present results for the combination of ROIaQP and the NNPF. More detailed descriptions can be found in the respective contributions. Results for each method by itself and the combination can be found in Table 1. All tests were run under the random access configuration described in the common test conditions (CTC) [8].

Table 1. Results for combination of ROIaQP and NNPF

| Dataset | Only NNPF | Only ROIaQP | ROIaQP + NNPF |
| --- | --- | --- | --- |
| SFU-HW | −24.7% | −5.7% | −30.0% |
| TVD | −26.8% | −20.3% | −42.5% |
| PandaSet | −32.4% | 0.0% | −33.2% |

As the results show, both methods provide a compression benefit for video coding for machines applications. This benefit can be improved by combining these two methods. For all three datasets the combination provides better results than taking only one of the methods.

## RRC and NNPF

The second test combines encoding sequences in reduced resolution while scaling with the reference picture resampling (RPR) filters available in the VTM software. Two different scaling factors were tested, 1.25x and 1.5x. Results are summarized in Table 2. Similar to the previous test, results were obtained in random access configuration with settings according to the CTC.

Table 2. Results for combination of RRC and NNPF

| Dataset | NNPF | RRC 1.25x | RRC 1.25x + NNPF | RRC 1.5x | RRC 1.5x + NNPF |
| --- | --- | --- | --- | --- | --- |
| SFU-HW | −24.7% | −4.1% | −26.7% | −2.1% | −20.3% |
| TVD | −26.8% | −4.0% | −38.6% | −4.1% | −35.6% |
| PandaSet | −32.4% | −1.0% | −32.8% | −3.7% | −29.0% |

For RRC the results are not as clear as for the ROIaQP case. When looking at scaling factor 1.25x the combination of NNPF and RRC gives an additional benefit for all three datasets. However, for scaling factor 1.5x only TVD benefits from the combination. For both SFU-HW and PandaSet the combination gives a lower benefit than using only the NNPF, despite using only RRC gives a benefit compared to the reference (VTM in its default configuration).

# Discussion

In this contribution we have tested two different combinations of methods that are either described in the TR on VCM or proposed to be added. The combined results show that the compression efficiency can be improved by combining the evaluated methods.

If it is decided to add a section or clause on combinations of technologies to the TR, it is proposed to include the combinations tested in this contribution to that section or clause.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

| [1] | C. Hollmann, J. Chen and S. Liu, "Optimization of encoders and receiving systems for machine analysis of coded video content (draft 6)," JVET-AI2030, Sapporo, JP, July 2024. |
| --- | --- |
| [2] | C. Hollmann, J. Ström, P. Wennersten, L. Litwic, R. Sjöberg and M. Damghanian, "Information about Ericsson’s response to the CfP for Video Coding for Machines," JVET-AB0275, Mainz, DE, October 2022. |
| [3] | C. Kim, D. Gwak and J. Lim, "[AHG8] De-noising filter as pre-processing for machine task," JVET-AE0081, Geneva, CH, July 2023. |
| [4] | S. Wang, B. Li, J. Chen and Y. Ye, "AHG8: On combined pre-processing and post-processing for machine vision," JVET-AH0157, Rennes, FR, April 2024. |
| [5] | S. Keating, "AHG8: 444 processing for VCM," JVET-AI0144, Sapporo, JP, July 2024. |
| [6] | B. Li, S. Wang, J. Chen, Y. Ye and S. Wang, "AHG8: A post-processing algorithm for machine consumption," JVET-AG0212, Online, January 2024. |
| [7] | C. Hollmann, J. Ström, P. Wennersten and K. Andersson, "AHG8: RPR for machine consumption," JVET-AH0130, Rennes, FR, April 2024. |
| [8] | S. Liu and C. Hollmann, "Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content," JVET-AI2031, Sapporo, JP, July 2024. |