| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0234-v1 |
| --- | --- |

| Title: | AHG9: On Spatial Extrapolation for NNPF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Following is proposed related to spatial extrapolation in neural-network post filtering:

- Proposal 1: To allow using patch-wise operation when using spatial extrapolation, it is proposed to modify the calculation of outPatchWidth and outPatchHeight for the patches which are on the border of the picture.

- Proposal 2: Modifications are proposed to the semantics of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset to correctly define the spatial area of the input picture and the constraint on their values.

# Introduction

The VSEI V4 draft 3 [1] includes a Neural-Network Post-Filter Characteristics SEI.

The Neural network post filter characteristics (NNPFC) SEI [1] includes a spatial extrapolation purpose which allows generating content outside of the spatial area of the input pictures (with possibly also removal of content).

Following is proposed related to spatial extrapolation in neural-network post filtering:

- Proposal 1: To allow using patch-wise operation when using spatial extrapolation, it is proposed to modify the calculation of outPatchWidth and outPatchHeight for the patches which are on the border of the picture.

- Proposal 2: Modifications are proposed to the semantics of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_‌extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset to correctly define the spatial area of the input picture and the constraint on their values.

# Proposal

## Proposal 1:

It is asserted that NNPF operates in a patch-wise manner which is manifested in equation 98. In that equation, input image is processed in patches of size (inpPatchHeight x inpPatchWidth), such that for each input patch inputTensor is created using DeriveInputTensors( ) function, followed by PostProcessingFilter( ) which takes as input the inputTensor to generate the filtered and/or interpolated picture(s) in a patch-wise manner returning outputTensor, which is in turn stored in Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, (as indicated by nnpfc_out_order_idc) of output patch size (outPatchHeight x outPatchWidth).

It is asserted that as opposed to resolution resampling where each input patch may increase or decrease in size based on the resampling ratio, the spatial extrapolation only expands area outside the border of the spatial area of the input pictures or contracts the area around the border of the spatial area of the input pictures. Thus, it is asserted that the patch-wise operation utilizing same equations as those for resolution resampling - for spatial extrapolation would not work correctly if image is broken in smaller patches, as each patch will get expanded by a factor which is dependent on how much spatial area is added or removed by spatial extrapolation. In VSEI v4 draft 3, this is addressed by setting input patch to be equal to the spatial area of the input pictures and by defining inferences/ constraints.

To allow more flexible patch-wise operation compared to VSEI v4 draft 3, when using spatial extrapolation, following approach is proposed:

The calculation of outPatchWidth and outPatchHeight is proposed to be modifed for the patches which are on the border of the picture. This is proposed to be done when spatial extrapolation is used, by making adjustment to the calculated values of outPatchWidth and outPatchHeight from equation 88 and 89 respectively by adding appropriate left/ right/ top/ bottom offset (i.e. nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset) when a patch is the left-most/ right-most/ top-most/ bottom-most patch respectively in the input image when performing patch-wise processing in NNPF PostProcessingFilter( ) in equation 98. This is proposed to be done before using actual values of outPatchWidth and outPatchHeight in PostProcessingFilter () and StoreOutputTensors() functions. Since one or more of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset could be negative, additional conformance constraints are proposed on outPatchWidth and outPatchHeight. The inferences for nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 are proposed to be removed and it is proposed to signal these syntax elements when SpatialExtrapolationFlag is equal to 1.

## The proposed modifications are shown on top of JVET-AI2006 [1].

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| … |  |
| nnpfc_chroma_sample_loc_type_frame | ue(v) |
| if( !SpatialExtrapolationFlag ) { |  |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| } |  |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| … |  |
| } |  |

The variables nnpfcOutputPicWidth and nnpfcOutputPicHeight respectively represent the width and height of the luma sample arrays of the NNPF-generated pictures when SpatialExtrapolationFlag is equal to 0.

The variables nnpfcOutputPicWidth1 and nnpfcOutputPicHeight1 respectively represent the width and height of the luma sample arrays of the NNPF-generated pictures when SpatialExtrapolationFlag is equal to 1.

The variable nnpfcOutputPicWidth, representing the width of the luma sample arrays of the NNPF-generated pictures, is derived as follows:

nnpfcOutputPicWidth = Ceil( CroppedWidth *	(77)
		( nnpfc_pic_width_num_minus1 + 1 ) ÷ ( nnpfc_pic_width_denom_minus1 + 1 ) )

When SpatialExtrapolation is equal to 1, nnpfcOutputPicWidth1 is updatedderived as follows:

nnpfcOutputPicWidth1 += nnpfcOutputPicWidth + outSubWidthC * ( nnpfc_spatial_extrapolation_left_offset + nnpfc_spatial_extrapolation_right_offset )

It is a requirement of bitstream conformance that when SpatialExtrapolation is equal to 0 nnpfcOutputPicWidth shall be greater than 0 and nnpfcOutputPicWidth % outSubWidthC shall be equal to 0.

It is a requirement of bitstream conformance that when SpatialExtrapolation is equal to 1 nnpfcOutputPicWidth1 shall be greater than 0 and nnpfcOutputPicWidth1 % outSubWidthC shall be equal to 0.

…

The variable nnpfcOutputPicHeight, representing the height of the luma sample arrays of the NNPF-generated pictures, is derived as follows:

nnpfcOutputPicHeight = Ceil( CroppedHeight *	(78)
		( nnpfc_pic_height_num_minus1 + 1 ) ÷ ( nnpfc_pic_height_denom_minus1 + 1 ) )

When SpatialExtrapolation is equal to 1, nnpfcOutputPicHeight1 is updatedderived as follows:

nnpfcOutputPicHeight1 += nnpfcOutputPicHeight + outSubHeightC * ( nnpfc_spatial_extrapolation_top_offset +	 nnpfc_spatial_extrapolation_bottom_offset )

It is a requirement of bitstream conformance that when SpatialExtrapolation is equal to 0 nnpfcOutputPicHeight shall be greater than 0 and nnpfcOutputPicHeight % outSubHeightC shall be equal to 0.

It is a requirement of bitstream conformance that when SpatialExtrapolation is equal to 1 nnpfcOutputPicHeight1 shall be greater than 0 and nnpfcOutputPicHeight1 % outSubHeightC shall be equal to 0.

When ResolutionResamplingFlag is equal to 1, at least one the following conditions shall be true:

–	The value of nnpfcOutputPicWidth is not equal to CroppedWidth.

–	The value of nnpfcOutputPicHeight is not equal to CroppedHeight.

–	SpatialExtrapolationFlag is equal to 1.

…

nnpfc_patch_width_minus1 plus 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the horizontal sample counts of the patch size required for the input to the NNPF. The value of nnpfc_patch_width_minus1 shall be in the range of 0 to Min( 32 766, CroppedWidth − 1 ), inclusive. When SpatialExtrapolationFlag is equal to 1, nnpfc_patch_width_minus1 is inferred to be equal to CroppedWidth − 1.

nnpfc_patch_height_minus1 plus 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the vertical sample counts of the patch size required for the input to the NNPF. The value of nnpfc_patch_height_minus1 shall be in the range of 0 to Min( 32 766, CroppedHeight − 1 ), inclusive. When SpatialExtrapolationFlag is equal to 1, nnpfc_patch_height_minus1 is inferred to be equal to CroppedHeight − 1.

…

It is a requirement of bitstream conformance that when SpatialExtrapolation is equal to 0 outPatchWidth * CroppedWidth shall be equal to nnpfcOutputPicWidth * inpPatchWidth and outPatchHeight * CroppedHeight shall be equal to nnpfcOutputPicHeight * inpPatchHeight.

It is a requirement of bitstream conformance that when SpatialExtrapolation is equal to 1 outPatchWidth * CroppedWidth shall be equal to nnpfcOutputPicWidth1 * inpPatchWidth and outPatchHeight * CroppedHeight shall be equal to nnpfcOutputPicHeight1 * inpPatchHeight.

…

An NNPF PostProcessingFilter( ) is the target NNPF as derived in the semantics of the NNPFA SEI message. The following example process may be used, with the NNPF PostProcessingFilter( ), to generate, in a patch-wise manner, the filtered and/or interpolated picture(s), which contain Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, as indicated by nnpfc_out_order_idc:

if( nnpfc_inp_order_idc  = =  0  | |  nnpfc_inp_order_idc  = =  2 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth ) {
		if( SpatialExtrapolationFlag ) {
		if( cTop = = 0) outPatchHeight +=  (outSubHeightC * nnpfc_spatial_extrapolation_top_offset)
		if( cLeft = = 0) outPatchWidth +=  (outSubWidthC * nnpfc_spatial_extrapolation_left_offset)
		if( cTop = = (CroppedHeight−inpPatchHeight)) outPatchHeight += (outSubHeightC * nnpfc_spatial_extrapolation_bottom_offset)
		if( cLeft = = (CroppedWidth−inpPatchWidth)) outPatchWidth += (outSubWidthC * nnpfc_spatial_extrapolation_right_offset)
		}
			inputTensor = DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( outputTensor )
		} 
else if( nnpfc_inp_order_idc  = =  1 )
	for( cTop = 0; cTop < CroppedHeight / SubHeightC; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth / SubWidthC; cLeft  +=  inpPatchWidth ) {(98) 
		if( SpatialExtrapolationFlag ) {
		if( cTop = = 0) outPatchHeight += (outSubHeightC * nnpfc_spatial_extrapolation_top_offset)
		if( cLeft = = 0) outPatchWidth += (outSubWidthC * nnpfc_spatial_extrapolation_left_offset)
		if( cTop = = ((CroppedHeight/ SubHeightC)−inpPatchHeight))) outPatchHeight += (outSubHeightC * nnpfc_spatial_extrapolation_bottom_offset)
		if( cLeft = = ((CroppedWidth/ SubWidthC)−inpPatchWidth))) outPatchWidth +=  (outSubWidthC * nnpfc_spatial_extrapolation_right_offset)
		}
			inputTensor = DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( outputTensor )
		}
else if( nnpfc_inp_order_idc  = =  3 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight * 2 )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth * 2 ) {
		if( SpatialExtrapolationFlag ) {
		if( cTop = = 0) outPatchHeight += (outSubHeightC * nnpfc_spatial_extrapolation_top_offset)
		if( cLeft = = 0) outPatchWidth += (outSubWidthC * nnpfc_spatial_extrapolation_left_offset)
		if( cTop = = (CroppedHeight−2*inpPatchHeight)) outPatchHeight += (outSubHeightC * nnpfc_spatial_extrapolation_bottom_offset)
		if( cLeft = = (CroppedWidth−2*inpPatchWidth)) outPatchWidth += (outSubWidthC * nnpfc_spatial_extrapolation_right_offset)
		}
			inputTensor = DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( outputTensor )
		}

It is a requirement of bitstream conformance that outPatchWidth is > 0.

It is a requirement of bitstream conformance that outPatchHeight is > 0.

## Proposal 2:

Modifications are proposed to the semantics of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset to correctly define the spatial area of the input picture and the constraint on their values (to avoid corner case of all of them being equal to 0).

Since nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset are se(v) coded, one or more of those could be negative. It is asserted that the current definition for spatial area of the input picture in the NNPF generated output picture does not work correctly for negative values.

Additionally, when spatial extrapolation is used and nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset syntax elements are signalled it is expected that not all of them are equal to 0 (which would be contradictory to there being spatial extrapolation). However, since one or more of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset can be negative the constraint on them should be changed from that at least one of them shall be greater than 0 to that at least one of them shall not be equal to 0.

The proposed modifications are shown on top of JVET-AI2006 [1].

nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset specify the spatial extrapolation area. When nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset are each greater than or equal to 0, tThe luma samples with horizontal picture coordinates from outSubWidthC * nnpfc_spatial_extrapolation_left_offset to nnpfcOutputPicWidth − ( outSubWidthC * nnpfc_spatial_extrapolation_right_offset ) and vertical picture coordinates from outSubHeightC * nnpfc_spatial_extrapolation_top_offset to nnpfcOutputPicHeight − ( outSubHeightC * nnpfc_spatial_extrapolation_bottom_offset ) correspond to the spatial area of the input picture. The value of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_‌extrapolation_top_offset and nnpfc_spatial_extrapolation_bottom_offset shall be in the range of −65 536 to 65 536, inclusive. At least one of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset and nnpfc_spatial_extrapolation_bottom_offset shall be greater than not equal to 0.

when proposal 2 is used with proposal 1:

nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset specify the spatial extrapolation area. When nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset  are each greater than or equal to 0, tThe luma samples with horizontal picture coordinates from outSubWidthC * nnpfc_spatial_extrapolation_left_offset to nnpfcOutputPicWidth1 − ( outSubWidthC * nnpfc_spatial_extrapolation_right_offset ) and vertical picture coordinates from outSubHeightC * nnpfc_spatial_extrapolation_top_offset to nnpfcOutputPicHeight1 − ( outSubHeightC * nnpfc_spatial_extrapolation_bottom_offset ) correspond to the spatial area of the input picture. The value of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_‌extrapolation_top_offset and nnpfc_spatial_extrapolation_bottom_offset shall be in the range of −65 536 to 65 536, inclusive. At least one of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset and nnpfc_spatial_extrapolation_bottom_offset shall be greater than not equal to 0.

# Conclusion

It is proposed to adopt the proposed changes to the next draft of VSEI version 4.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JVET-AI2006, J. Boyce, S. McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullivan, Y.-K. Wang, SEI messages for VSEI version 4 (Draft 3), July 2024