| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0237-v1 |
| --- | --- |

| Title: | AHG12: 12-bit internal bit depth for ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Vadim Seregin Muhammed Coban Marta Karczewicz | Email: | ruoyyu@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this contribution, it is asserted that ECM developed thus far has been primarily designed and exercised with a focus on using 10-bit internal bit depth. It is reported that when using 12-bit internal bit depth, the current ECM-14.0 either stops encoding or delivers lower compression performance comparing to 10-bit internal bit depth.

This contribution proposes modifications to certain identified tools to achieve better operation at 12-bit internal bit depth. The modifications are reported to be implemented on top of ECM-14.0. With the proposed modifications, the BD-rate impact of using 12-bit over 10-bit internal bit depth under ECM CTC is reported as follows:

AI: -0.25%, 0.11%, 0.05% for Y, U, V respectively, EncT 100.7% DecT 100.5%

RA: (to be updated when full results become available)

It is suggested to include the proposed modifications in ECM and enable 12-bit internal bit depth for ECM CTC.

# Introduction and Proposal

ECM developed thus far has been primarily designed and exercised with a focus on using 10-bit internal bit depth. When using 12-bit internal bit depth, the current ECM-14.0 either stops encoding or delivers lower compression performance comparing to 10-bit internal bit depth.

This contribution proposes modifications on some identified tools to achieve better operation at 12-bit internal bit depth. A high-level summary of modifications are as follows:

- BIF:

For MAD based scale factor look-up, the MAD values are scaled according to internal bit-depth. For BIF offset derivation, its input sample value difference as well as the derived BIF offsets are scaled according to internal bit-depth.

- CCSAO:

For CCSAO edge-based classification, thresholds are scaled according to internal bit-depth. The CCSAO edge offsets and band offsets are scaled according to internal bit-depth before applying to the reconstructed sample values.

- ALF:

For variance calculation in ALF classification, the variance is normalized according to internal bit-depth.

For ALF residual scaling, the internal bit-depth is considered in residual correction calculation, as well as in the setting of min/max classifier index.

- Diversity based ARMC:

The lambda values used in the diversity based ARMC are scaled according to internal bit-depth.

- Motion compensated picture boundary padding:

The clipping value (used after DC value correction) is set according to the internal bit-depth instead of a hard-coded value.

- DMVR:

In the current DMVR, the precision of intermediate interpolated sample values is kept at 10-bit regardless of internal bit depth. It is proposed to make the precision of intermediate interpolated sample values to be the same as the internal bit depth (with the maximum possible precision being capped at 14-bit).

- CCCM and tools which uses CCCM-like model

The precision for covariance matrices is increased to 32-bit and the precision for decimals is increased to 22-bit when the internal bit depth is greater than 10.

# Results

The proposed modifications have been implemented on top of ECM-14.0. The performance of using 12-bit internal bit depth comparing to 10-bit internal bit depth under ECM CTC are as follows:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.33% | -0.68% | -1.23% | 100.7% | 99.7% | 100.1% | 99.0% |
| Class A2 | -0.33% | 0.54% | 0.88% | 100.8% | 100.2% | 100.1% | 99.2% |
| Class B | -0.19% | -0.02% | -0.08% | 101.2% | 100.6% | 96.8% | 96.4% |
| Class C | -0.17% | 0.56% | 0.41% | 99.8% | 100.8% | 99.1% | 98.9% |
| Class E | -0.30% | 0.11% | 0.25% | 101.0% | 100.8% | 103.2% | 103.8% |
| Overall | -0.25% | 0.11% | 0.05% | 100.7% | 100.5% | 99.5% | 99.1% |
| Class D | -0.13% | 0.65% | 0.50% | 100.1% | 100.2% | 100.4% | 100.3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | -0.42% | -0.21% | -0.56% | 102.4% | 99.8% | 100.3% | 100.4% |
| Class C | -0.31% | 1.03% | 0.85% | 101.2% | 100.0% | 100.5% | 100.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | -0.21% | 0.52% | 0.42% | 100.8% | 100.5% | 100.2% | 100.3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# Conclusion

This contribution proposes modifications to certain identified tools to achieve better operation at 12-bit internal bit depth. It is asserted that the objective results indicate a reasonable trade-off in terms of BD-rate improvements and run-time impacts. It is suggested to include the modifications in ECM and enable 12-bit internal bit depth for ECM CTC.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).