| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0238-v2 |
| --- | --- |

| Title: | AHG12: OBMC modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Han Huang Chun-Chi Chen Vadim Seregin Marta Karczewicz | Email: | ruoyyu@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this contribution, two modifications related to OBMC operation orders are proposed. The first modification proposes to apply OBMC separately for each partition of a GPM coded block and then apply GPM blending to create the final GPM prediction block. The second modification proposes to first apply OBMC on subblock boundaries, and then apply OBMC on block boundaries. The proposed modifications are reported to be implemented on top of ECM-14.0. The BD-rate impact under ECM CTC is reported to be -0.06% for RA (estimated by copying the anchor result for ParkRunning3 at QP22). The run time impact is reported to be EncT 100.9%, DecT 100.2%. It is proposed to investigate the proposed modifications in the next round of EE2.

# Introduction and Proposal

Two modifications related to OBMC operation orders are proposed.

The first modification is about GPM and OBMC. In the current ECM, when the two GPM partitions are coded with non-affine inter, the GPM blending is carried out first, and then OBMC is applied on top of blended GPM samples. It is proposed to apply OBMC separately on top of each GPM partition, and then apply blending on top of OBMC modified samples of the two partitions. Furthermore, if the corresponding partition contains subblock motion, OBMC is applied on those subblock boundaries as well.

The second modification is about the order of applying OBMC on block and sub-block boundaries. In the current ECM, OBMC is first carried out on block (CU) boundaries, and is then carried out on subblock boundaries. It is proposed to swap the order to first apply OBMC on subblock boundaries (if they are present within the CU), and then apply OBMC on CU boundaries.

# Results

The proposed modifications have been implemented on top of ECM-14.0. The performance of the proposed modifications on top of ECM-14.0 under ECM CTC are as follows:

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.18% | -0.22% | 100.5% | 100.5% | 100.1% | 100.0% |
| Class A2 | -0.06%* | 0.00% | -0.11% | 100.5% | 100.3% | 99.7% | 99.7% |
| Class B | -0.07% | 0.02% | -0.05% | 101.3% | 99.6% | 99.8% | 99.7% |
| Class C | -0.07% | -0.14% | 0.04% | 101.2% | 100.7% | 99.4% | 99.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.06%* | -0.07% | -0.07% | 100.9% | 100.2% | 99.7% | 99.6% |
| Class D | -0.05% | -0.21% | -0.29% | 100.7% | 100.2% | 99.2% | 98.9% |

(*) The numbers are estimated by copying the anchor result for ParkRunning3 at QP22

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0.06% | -0.09% | -0.16% | 100.1% | 100.2% | 98.9% | 98.5% |
| Class E | -0.02% | -0.16% | -0.94% | 100.2% | 99.3% | 100.3% | 100.2% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | 0.02% | 0.26% | 0.26% | 100.3% | 97.8% | 99.1% | 99.0% |

# Conclusion

This contribution proposes two modifications related to OBMC operation orders. It is asserted that the modifications are straightforward. It is suggested to investigate the proposed modifications in the next round of EE2.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).