| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0240-v1 |
| --- | --- |

| Title: | [AHG9] Lens Optical Correction SEI message updates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Gilles Teniou (Tencent) Stephan Wenger (Tencent) | Tel: Email: | [phone contact number] [teniou/swenger]@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution addresses the comments made on the Lens Optical Correction SEI message during the last meeting and proposes a small update to the TuC addressing the behavior with frame packing.

# Introduction

During the last JVET meeting in Sapporo, a proposal for a new SEI message on Lens Optical Correction [1] was proposed together with an implementation and a demo [2] highlighting the benefits of signalling lens properties for image correction.

The proposed SEI message was agreed to be included into the Technologies under consideration for future exentions of VSEI [3] in which an editor’s note was inserted to keep track of the comments made during the presentation of [1].

[Ed. (GT): To be checked:

- validity of cancel flag and persistence flag,

- relation with conformance cropping,

- behavior with frame packing.]

# Validity of cancel and persistence flags

Lens characteristics are expected to remain unchanged at least for a given shot which corresponds to a sequence of pictures in a CLVS. In the case of a sequence combining shots from various sources the loc_cancel_flag is meant to cancel the persistence of the lens correction parameters associated to the previous pictures. We acknowledge that it is unlikely to define a persistence of only one picture (loc_persistence_flag = 0) however, the use of this single bit ensures consistency across SEI messages generic syntax.

There is no action needed, and we consider this point addressed.

# Relation with conformance cropping

The conformance cropping window, signalled in the SPS, may be used to explicitely indicate the lines to be cropped in a fullHD picture coded in 1920x1088.

The Lens Optical Correction SEI message has the goal to geometrically manipulate the output signal regarless of the pixels cropped in the conformance window. The purpose of lens correction is to allow the entire picture to be full of useful data, and the lens correction at the renderer may translate that into a non-rectangular shape. Insofar, there’s no overlap between the conformance window and the lens correction SEI.

There is no action needed, and we consider this point addressed.

# Behavior with frame packing

When frame packing is signaled for a video sequence the output picture not only contains different images but also requires upconversion to retrieve the full resolution per view. This required post-processing step is likely to introduce artificial pixels resulting from the upconversion for which the characteristics from the lens may not be fully applicable.

We therefore suggest to disallow the use of the Lens Optical Correction SEI message together with the Frame packing arrangement SEI message in the picture.

To address this aspect, it is proposed to add the following sentence at the end of the Lens Optical Correction SEI message semantics:

"It is a condition of bitstream conformance that no pictures in the persistence scope of a frame packing arrangement SEI message are in the persistence scope of a lens optical correction SEI message."

# Proposal

It is proposed to update the TuC by removing the editor’s note and add the quoted sentence of clause 4.

# References

[1]	JVET-AI0158 [AHG9] Lens Optical Correction SEI message

[2]	JVET-AI0160 [AHG9] Lens Optical Correction SEI message implementation

[3]	JVET-AI2032 Technologies under consideration for future extensions of VSEI (version 5)

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).