| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0240-v3 |
| --- | --- |

| Title: | [AHG9] Lens Optical Correction SEI message updates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Gilles Teniou (Tencent) Stephan Wenger (Tencent) | Tel: Email: | [phone contact number] [teniou/swenger]@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution addresses the comments made on the Lens Optical Correction SEI message during the last meeting and proposes a small update to the TuC addressing the behavior with frame packing.

# Introduction

During the last JVET meeting in Sapporo, a proposal for a new SEI message on Lens Optical Correction [1] was proposed together with an implementation and a demo [2] highlighting the benefits of signalling lens properties for image correction.

The proposed SEI message was agreed to be included into the Technologies under consideration for future exentions of VSEI [3] in which an editor’s note was inserted to keep track of the comments made during the presentation of [1].

[Ed. (GT): To be checked:

- validity of cancel flag and persistence flag,

- relation with conformance cropping,

- behavior with frame packing.]

# Validity of cancel and persistence flags

Lens characteristics are expected to remain unchanged at least for a given shot which corresponds to a sequence of pictures in a CLVS. In the case of a sequence combining shots from various sources the loc_cancel_flag is meant to cancel the persistence of the lens correction parameters associated to the previous pictures. We acknowledge that it is unlikely to define a persistence of only one picture (loc_persistence_flag = 0) however, the use of this single bit ensures consistency across SEI messages generic syntax.

There is no action needed, and we consider this point addressed.

Decision after presentation of v1 of the present contribution:

Agreed, no action needed

# Relation with conformance cropping

The conformance cropping window, signalled in the SPS, may be used to explicitely indicate the lines to be cropped in a fullHD picture coded in 1920x1088.

The Lens Optical Correction SEI message has the goal to geometrically manipulate the output signal regarless of the pixels cropped in the conformance window. The purpose of lens correction is to allow the entire picture to be full of useful data, and the lens correction at the renderer may translate that into a non-rectangular shape. Insofar, there’s no overlap between the conformance window and the lens correction SEI.

There is no action needed, and we consider this point addressed.

Decision after presentation of v1 of the present contribution:

Add a sentence indicating that the parameters apply to the cropped decoded picture when the frame packing SEI message is not present in the CLVS

# Behavior with frame packing

When frame packing is signaled for a video sequence the output picture not only contains different images but also requires upconversion to retrieve the full resolution per view. This required post-processing step is likely to introduce artificial pixels resulting from the upconversion for which the characteristics from the lens may not be fully applicable.

We therefore suggest to disallow the use of the Lens Optical Correction SEI message together with the Frame packing arrangement SEI message in the picture.

To address this aspect, it is proposed to add the following sentence at the end of the Lens Optical Correction SEI message semantics:

"It is a requirement of bitstream conformance that no pictures in the persistence scope of a frame packing arrangement SEI message are in the persistence scope of a lens optical correction SEI message."

Decision after presentation of v1 of the present contribution:

Add a sentence indicating that the parameters apply to the cropped decoded picture when the frame packing SEI message is not present in the CLVS, or apply to each constituent frame when the frame packing arrangement SEI message is present.

# Proposal

It is proposed to update the TuC by removing the editor’s note and add the sentence discussed during presentation of v1.

8.42 Lens optical correction SEI message

8.42.1 Lens optical correction SEI message syntax

| lens_optical_correction( payloadSize ) { | Descriptor |
| --- | --- |
| loc_cancel_flag | u(1) |
| if( !loc_cancel_flag ) { |  |
| loc_focal_parameters_present_flag |  |
| if( loc_focal_parameters_present_flag ) { |  |
| loc_focal_center_x | u(16) |
| loc_focal_center_y | u(16) |
| loc_focal_length | u(16) |
| } |  |
| loc_rd_parameters_present_flag | u(1) |
| if( loc_rd_parameters_present_flag ) { |  |
| loc_rd_model_id | u(8) |
| if( loc_rd_model_id = = 0 ) { |  |
| loc_rd_poly3_k1 | i(25) |
| } |  |
| if( loc_rd_model_id = = 1 ) { |  |
| loc_rd_poly5_k1 | i(25) |
| loc_rd_poly5_k2 | i(25) |
| } |  |
| } |  |
| loc_tca_parameters_present_flag | u(1) |
| if( loc_tca_parameters_present_flag ) { |  |
| loc_tca_model_id | u(8) |
| if( loc_tca_model_id = = 0 ) { |  |
| loc_tca_kr | u(25) |
| loc_tca_kb | u(25) |
| } |  |
| if( loc_tca_model_id = = 1 ) { |  |
| loc_tca_br | i(25) |
| loc_tca_cr | i(25) |
| loc_tca_vr | u(25) |
| loc_tca_bb | i(25) |
| loc_tca_cb | i(25) |
| loc_tca_vb | u(25) |
| } |  |
| } |  |
| loc _persistence_flag | u(1) |
| } |  |
| } |  |

8.42.2 Lens optical correction SEI message semantics

The Lens optical correction SEI message provides the decoder with a lens distortion model to enable image correction.

The parameters of this SEI message apply to the cropped decoded picture.

When a frame packing arrangement SEI message is present in the CLVS, the parameters of this SEI messages apply to each constituent frame of the frame packing arrangement SEI message.

loc_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous lens optical correction SEI message in output order that applies to the current layer. loc_cancel_flag equal to 0 indicates that lens optical correction follows.

loc_focal_parameters_present_flag equal to 1 indicates that focal parameters of the lens are present in the lens optical correction SEI message syntax. loc_focal_parameters_present_flag equal to 0 indicates that no focal parameter is present.

loc_focal_center_x defines the horizontal location in the picture of the focal point. It is by default half of the width of the picture.

loc_focal_center_y defines the vertical location in the picture of the focal point. It is by default half of the height of the picture.

loc_focal_length defines the focal length of the lens, i.e., the distance from the lens in which rays of light that fall parallel to the optical axis into the lens are converged. It is expressed in units of 0.1 millimeters. A value of 0 indicates that the focal length is undefined. When not present, the value of loc_focal_length is inferred to be equal to 0.

loc_rd_parameters_present_flag equal to 1 indicates that radial distortion parameters are present in the lens optical correction SEI message syntax. loc_rd_parameters_present_flag equal to 0 indicates that no radial correction is expected.

loc_rd_model_id identifies the type of radial distortion model as specified in Table 1. The value of loc_rd_model_id shall be in the range of 0 to 1, inclusive. The values greater than 1 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore lens optical correction SEI messages with loc_rd_model_id greater than 1.

Table 1 – loc_rd_model_id

| Value | Description |
| --- | --- |
| 0 | Poly3 |
| 1 | Poly5 |

When loc_rd_model_id equals to 0 the polynomial 3rd order (Poly3) model is specified as

rd = ru * ( 1 − k1 + k1 * ru * ru )		(X-1)

with rd referring to the distorted radius (normalized distance to image center), ru referring to the corrected undistorted radius, and k1 being the distortion coefficient.

When loc_rd_model_id equals to 1 the polynomial 5th (Poly5) order model is specified as

rd = ru * ( 1 − k1 * ru * ru + k2 * ru * ru* ru* ru )		(X-2)

with rd referring to the distorted radius (normalized distance to image center), ru referring to the corrected undistorted radius, and k1 and k2 being the distortion coefficients.

loc_rd_poly3_k1 ÷224 indicates the value of the k1 parameter from the Poly3 radial distortion model from equation (X-1). When not present, the value of loc_rd_poly3_k1 is inferred to be equal to 0.

loc_rd_poly5_k1 ÷224 indicates the value of the k1 parameter from the Poly5 radial distortion model from equation (X-2). When not present, the value of loc_rd_poly5_k1 is inferred to be equal to 0.

loc_rd_poly5_k2 ÷224 indicates the value of the k2 parameter from the Poly5 radial distortion model from equation (X-2). When not present, the value of loc_rd_poly5_k2 is inferred to be equal to 0.

loc_tca_parameters_present_flag equal to 1 indicates that transversal chromatic aberration parameters are present in the lens optical correction SEI message syntax. loc_tca_parameters_parameters_present_flag equal to 0 indicates that no transversal chromatic aberration correction is expected.

loc_tca_model_id identifies the type of radial distortion model as specified in Table 2. The value of loc_tca_model_id shall be in the range of 0 to 1, inclusive. The values greater than 1 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore lens optical correction SEI messages with loc_tca_model_id greater than 1.

Table 2 – loc_tca_model_id

| Value | Description |
| --- | --- |
| 0 | Linear |
| 1 | Poly3 |

When loc_tca_model_id equals to 0 the linear model is specified as

rd,R = ru,R * kR		(X-3)

rd,B = ru,B * kB		(X-4)

with rd,R and rd,B referring to the distorted radius (normalized distance to image center) of the red and blue components respectively, ru,R and ru,B referring to the corrected undistorted radius of the red and blue components respectively, and kR and kB being the transversal chromatic aberration coefficients of the red and blue components respectively. kR and kB are equal to 1 by default.

NOTE – The  transversal chromatic aberration is corrected in the linear model on the red and blue components with the green component as the reference.

When loc_tca_model_id equals to 1 the polynomial 3rd order (Poly3) model is specified as

rd,R = ru,R * ( bR * ru,R * ru,R + cR * ru,R + vR )		(X-5)

rd,B = ru,B * ( bB * ru,B * ru,B + cB * ru,B + vB )		(X-6)

with rd,R and rd,B referring to the distorted radius (normalized distance to image center) of the red and blue components respectively, ru,R and ru,B referring to the corrected undistorted radius of the red and blue components respectively, and {bR, cR, vR} and {bB, cB, vB} being the transversal chromatic aberration coefficients of the red and blue components respectively. The bR, cR, bB,and cB coefficients are by default equal to 0, and the vR and vB coefficients are by default equal to 1.

NOTE - The transversal chromatic aberration is corrected in the Poly3 model on the red and blue components with the green component as the reference.

loc_tca_kr ÷224 indicates the value of the kR parameter from the linear transversal chromatic aberration model from equation (X-3). When not present, the value of loc_tca_kr is inferred to be equal to 224.

loc_tca_kb ÷224 indicates the value of the kB parameter from the linear transversal chromatic aberration model from equation (X-4). When not present, the value of loc_tca_kr is inferred to be equal to 224.

loc_tca_br ÷224 indicates the value of the bB parameter from the Poly3 transversal chromatic aberration model from equation (X-5). When not present, the value of loc_tca_br is inferred to be equal to 0.

loc_tca_cr ÷224 indicates the value of the cR parameter from the Poly3 transversal chromatic aberration model from equation (X-5). When not present, the value of loc_tca_cr is inferred to be equal to 0.

loc_tca_vr ÷224 indicates the value of the vR parameter from the Poly3 transversal chromatic aberration model from equation (X-5). When not present, the value of loc_tca_vr is inferred to be equal to 224.

loc_tca_bb ÷224 indicates the value of the bB parameter from the Poly3 transversal chromatic aberration model from equation (X-6). When not present, the value of loc_tca_bb is inferred to be equal to 0.

loc_tca_cb ÷224 indicates the value of the cB parameter from the Poly3 transversal chromatic aberration model from equation (X-6). When not present, the value of loc_tca_cb is inferred to be equal to 0.

loc_tca_vb ÷224 indicates the value of the vB parameter from the Poly3 transversal chromatic aberration model from equation (X-6). When not present, the value of loc_tca_vb is inferred to be equal to 224.

loc_persistence_flag specifies the persistence of the lens optical correction SEI message for the current layer.

loc_persistence_flag equal to 0 specifies that the lens optical correction SEI message applies to the current decoded picture only.

loc_persistence_flag equal to 1 specifies that the lens optical correction SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a lens optical correction SEI message is output that follows the current picture in output order.

# References

[1]	JVET-AI0158 [AHG9] Lens Optical Correction SEI message

[2]	JVET-AI0160 [AHG9] Lens Optical Correction SEI message implementation

[3]	JVET-AI2032 Technologies under consideration for future extensions of VSEI (version 5)

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).