| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0241-v1 |
| --- | --- |

| Title: | AHG9: Text description SEI purpose for encoder description |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela | Email: | {jill.boyce, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is proposed to add a new purpose to the Text description SEI message to provide a description of the encoder used to create the bitstream.

# Proposal

The text descriptions SEI message in the VSEI WD in JVET-AI2006 currently supports multiple purposes, as listed in the table below.

Table xx – Definition of txt_descr_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General comment information |
| 4 | Content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) |
| 5 | Tag URI for identifying the bitstream |
| 6..255 | Reserved |

It is proposed to add another purpose to describe the encoder used to create the bitstream. This description may include the encoder implementation name and version as well as the configuration of the encoder.

Some example text descriptions for the encoder description purpose:

- VTM10.0 RA

- VVenC v1.11 AI

It is asserted that such a feature would have been useful during development of the conformance test suite for VVC and earlier standards.

# Proposed syntax and semantics

Modifications are shown in yellow.

Table xx – Definition of txt_descr_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General comment information |
| 4 | Content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) |
| 5 | Tag URI for identifying the bitstream |
| 6 | Encoder description |
| 67..255 | Reserved |

When txt_descr_purpose is equal to 6, the txt_descr_string[ i ] specifies a description of the encoder used to produce the coded picture(s) in the persistence scope of this SEI message.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).