| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0242-v2 |
| --- | --- |

| Title: | AHG9: On initialization of target picture for DOI SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Thibaud Biatek Jill Boyce Miska M. Hannuksela | Email: | thibaud.biatek@nokia.com jill.boyce@nokia.com miska.hannuksela@nokia.com |
| Source: | AHG9 |  |  |

_____________________________

# Abstract

In the current DOI SEI design, the description does not specify how the target picture is initialized. This may lead to an unexpected behavior and degraded picture quality if the first overlay comes with an alpha channel. This contribution proposes to optionally signal initialization values for the target picture.

# Motivation

The DOI SEI describes a process for forming target pictures based on applying individual display overlays in index order. For sample locations that are not represented by any display overlay, the sample values for the target picture are undefined. Additionally, if samples in a first display overlay have alpha layers with transparency or semi-transparency, the process for forming the target picture samples at that location are undefined.

# Proposed changes

It is proposed to optionally explicitly signal sample initialization values for each color component. The proposed changes are summarized as follows:

- Add a target picture initialization presence flag

- Add a flag to signal initialization values for 1 vs 3 color components

- Add a syntax element to indicate the length of the signaled initialization values

- Add syntax elements for 1 or 3 initialization values

In addition to resolving the initialization issue, this would give more flexibility to create diverse overlays using the DOI SEI.

The proposed changes are highlighted in yellow below.

SPECIFICATION CHANGES ON TOP OF AI-2032

8.43 Display overlays SEI message

8.43.1 Display overlays SEI message syntax

| display_overlays_info( payloadSize ) { | Descriptor |
| --- | --- |
| doi_id | u(6) |
| doi_cancel_flag | u(1) |
| if( !doi_cancel_flag ) { |  |
| doi_persistence_flag | u(1) |
| doi_num_display_overlays_minus2 | ue(v) |
| doi_target_pic_size_present_flag | u(1) |
| if( doi_target_pic_size_present_flag ) |  |
| doi_target_pic_width_minus1 | u(16) |
| doi_target_pic_height_minus1 | u(16) |
| } |  |
| doi_target_init_present_flag | u(1) |
| if( doi_target_init_present_flag ) { |  |
| doi_target_init_three_comp_flag | u(1) |
| doi_target_init_bitdepth_minus8 | ue(v) |
| for ( i = 0; i < ( doi_three_component_flag > 0 ? 3 : 1 ); i++ ) |  |
| doi_target_init_sample[ i ] | u(v) |
| } |  |
| doi_nuh_layer_id_present_flag | u(1) |
| doi_pic_partition_flag | u(1) |
| if( doi_pic_partition_flag ) { |  |
| doi_partition_type_flag | u(1) |
| doi_partition_id_len_minus1 | u(4) |
| } |  |
| doi_offset_params_present_flag | u(1) |
| if( doi_offset_params_present_flag ) |  |
| doi_offset_param_length_minus1 | u(4) |
| doi_resampling_enabled_flag | u(1) |
| if( doi_resampling_enabled_flag ) |  |
| doi_size_param_length_minus1 | u(4) |
| for( i = 0; i < doi_num_display_overlays_minus2 + 2; i++ ) { |  |
| if ( doi_nuh_layer_id_present_flag ) |  |
| doi_nuh_layer_id[ i ] | u(6) |
| if ( doi_pic_partition_flag ) |  |
| doi_partition_id[ i ] | u(v) |
| doi_alpha_present_flag[ i ] | u(1) |
| if ( doi_alpha_present_flag[ i ] ) { |  |
| if (doi_nuh_layer_id_present_flag) |  |
| doi_alpha_nuh_layer_id[ i ] | u(6) |
| if (doi_pic_partition_flag) |  |
| doi_alpha_partition_id[ i ] | u(v) |
| } |  |
| if( i > 0 ) { |  |
| if (doi_offset_params_present_flag ) |  |
| doi_top_left_x[ i ] | u(v) |
| doi_top_left_y[ i ] | u(v) |
| } |  |
| if( doi_resampling_enabling_flag ) { |  |
| doi_width_minus1[ i ] | u(v) |
| doi_height_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

8.43.2 Display overlays SEI message semantics

The display overlays information (DOI) SEI message provides metadata to enable formation of a target display picture formed by overlaying multiple ordered display overlays in a specified order. A display overlay contains texture and optionally an alpha channel, each contained within a cropped decoded picture, a subpicture, and/or a constituent rectangle.

Use of this SEI message requires the definition of the following variables, where i is the layer identifier of a layer that may be present in the current CVS:

–	An array of picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples[ i ] and PicHeightInLumaSamples[ i ], respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc[ i ], as described in subclause ‎7.3.

–	Bit depth BitDepth[ i ] for the picture containing the samples.

–	An array of subpicture counts, denoted by NumSubpics[ i ].

–	Arrays of the width and height of the subpictures, denoted herein by SubPicWidth[ i ][ j ] and SubPicHeight[ i ][ j ] respectively, where j is the subpicture index in 0 .. NumSubpics[ i ] – 1.

doi_id specifies an identifier of the DOI SEI message.

doi_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous DOI SEI message with the same doi_id in output order. doi_cancel_flag equal to 0 indicates that display overlays information follows.

doi_persistence_flag specifies the persistence of the DOI SEI message for the CVS.

doi_persistence_flag equal to 0 specifies that the DOI SEI message applies to the current AU only.

doi_persistence_idc equal to 1 specifies that the DOI SEI message applies to the current AU and persists for all subsequent AUs in output order until one or more of the following conditions are true:

–	A new CVS begins.

–	The bitstream ends.

–	A picture in the current AU with a DOI SEI message with the same value of doi_id is output that follows the current picture in output order.

doi_num_display_overlays_minus2 plus 2 specifies the number of display overlays for which information is signalled in the SEI message. The value of doi_num_display_overlays_minus2 shall be in the range of 0 to 30, inclusive.

doi_target_pic_size_present_flag equal to 1 specifies that the doi_target_pic_width_minus1 and doi_target_pic_height_minus1 syntax elements are present. doi_target_pic_size_present_flag equal to 0 specifies that the doi_target_pic_width_minus1 and doi_target_pic_height_minus1 syntax elements are present.

doi_target_pic_width_minus1 + 1, when present, indicates the width of the target picture.

doi_target_pic_height_minus1+ 1, when present, indicates the height of the target picture.

doi_target_init_present_flag equal to 1 specifies that the doi_target_init_three_comp_flag, doi_target_init_bitdepth_minus8, and doi_target_init_sample[ i ]  syntax elements are present in the SEI message. doi_target_init_present_flag equal to 0 specifies that the he doi_target_init_three_comp_flag, doi_target_init_bitdepth_minus8, and doi_target_init_sample[ i ]  syntax element are not present in the SEI message.

doi_target_init_three_comp_flag equal to 1 specifies that doi_target_init_sample[ i ] syntax element will be signaled for three color components. doi_target_init_three_comp_flag equal to 0 specifies that doi_target_init_sample[ i ] syntax element will be signaled for one color component.

doi_target_init_bitdepth_minus8 plus8 specifies the bit depth of the target picture initialization sample values. doi_target_init_bitdepth_minus8 shall be in the range of 0 to8, inclusive.

The variable initBitDepth is set equal to doi_target_init_bitdepth_minus8 + 8.

doi_target_init_sample[ i ] indicates the target picture initialization value for the i-th colour component. The length of the syntax element is initBitDepth bits.

When not present, the value of doi_target_init_sample[ i ] is set equal to 1 << ( BitDepth[ i ]− 1 ) for each value of i in the range of 0 to 2, inclusive..

doi_nuh_layer_id_present_flag equal to 1 specifies that the doi_nuh_layer_id[ i ] syntax element is present in the SEI message. doi_nuh_layer_id_present_flag equal to 0 specifies that the doi_nuh_layer_id[ i ] syntax element is not present in the SEI message.

doi_pic_partition_flag equal to 1 specifies that display overlay components are coded as constituent rectangles or subpictures. doi_pic_partition_flag equal to 0 specifies that display overlay components are coded as pictures.

It is a requirement of bitstream conformance that at least one of doi_nuh_layer_id_present_flag or doi_pic_partition_flag shall be equal to 1.

doi_partition_type_flag equal to 1 specifies that a display overlay component is coded as a constituent rectangle. doi_partition_type_flag equal to 0 specifies that a display overlay component is coded as a subpicture.

When doi_partition_type_flag equal to 1, it is a requirement of bitstream conformance that there is a constituent rectangles SEI message preceding the DOI SEI message in decoding order in the current PU.

doi_partition_id_len_minus1 + 1 specifies the length of the doi_partition_id[ i ] syntax elements.

doi_offset_params_present_flag[ i ] equal to 1 specifies that offset parameters are present for the i-th display overlay. doi_offset_params_present_flag[ i ] equal to 0 specifies that offset parameters are not present for the i-th display overlay.

doi_offset_param_length_minus1 plus 1 specifies the length of the doi_top_left_x[ i ] and doi_top_left_y[ i ], syntax elements in bits.

doi_resampling_enabled_flag equal to 1 specifies that display overlay components may be resampled in the target display picture. doi_resampling_enabled_flag equal to 0 specifies that display overlay components are not resampled in the target display picture.

doi_size_param_length_minus1 plus 1 specifies the length of the doi_width_minus1[ i ] and doi_height_minus1[ i ] syntax elements in bits.

doi_nuh_layer_id[ i ], when present, specifies the layer identifier of the texture component of the i-th display overlay. When not present, the value of doi_nuh_layer_id[ i ] is inferred to be equal to the layer identifier of the PU containing the DOI SEI message.

If this SEI message is present in any layer in the current AU, it is a requirement of bitstream conformance that a DOI SEI message with the same value of doi_id and the same payload is present in the layer with layer identifier doi_nuh_layer_id[ 0 ].

doi_partition_id[ i ], when present and doi_partition_type_flag equal to 1, specifies  that the texture component of the i-th display overlay is represented by the j-th constituent rectangle when cr_rect_id[ j ] equal doi_partition_id[ i ]. doi_partition_id[ i ], when present and doi_partition_type_flag equal to 0, specifies the subpicture index of the texture component of the i-th display overlay.  When not present, the value of doi_partition_id[ i ] is inferred to be equal to 0.

When doi_partition_type_flag equal to 1, doi_partition_id[ i ] shall be in the range of 0 ..  cr_num_rects_minus1[ doi_nuh_layer_id[ i ] ] – 1.  When doi_partition_type_flag equal to 0, doi_partition_id[ i ] shall be in the range of 0 .. NumSubpics[ doi_nuh_layer_id[ i ] ] – 1.

doi_alpha_present_flag[ i ] equal to 1 specifies that an alpha component is provided for the i-th display overlay. doi_alpha_present_flag[ i ] equal to 0 specifies that an alpha component is not provided for the i-th display overlay.

doi_alpha_nuh_layer_id[ i ], when present, specifies the layer identifier value of the alpha component of the i-th display overlay. When not present, the value of doi_alpha_nuh_layer_id[ i ] is inferred to be equal to the layer identifier of the PU containing the DOI SEI message.

doi_alpha_partition_id[ i ], when present and doi_partition_type_flag equal to 1, specifies the cr_rect_id[ j ] of the alpha component of the i-th display overlay. doi_partition_id[ i ], when present and doi_partition_type_flag equal to 0, specifies the subpicture index of the alpha component of the i-th display overlay.  When not present, the value of doi_alpha_partition_id[ i ] is inferred to be equal to 0.

When doi_partition_type_flag equal to 1, doi_alpha_partition_id[ i ] shall be in the range of 0 ..  cr_num_rects_minus1[ doi_nuh_layer_id[ i ] ] – 1.  When doi_partition_type_flag equal to 0, doi_alpha_partition_id[ i ] shall be in the range of 0 .. NumSubpics[ doi_nuh_layer_id[ i ] ] – 1.

doi_top_left_x[ i ] and doi_top_left_y[ i ] specify the horizontal and vertical positions, respectively, of the top left corner of the i-th display overlay in the target display picture, in luma samples. When not present, the values of doi_top_left_x[ i ] and doi_top_left_y[ i ] are inferred to be equal to 0. The length of the syntax elements is doi_offset_param_length_minus1 + 1 bits.

doi_width_minus1[ i ] plus 1 and doi_height_minus1[ i ] plus 1, when present, specify the width and height, respectively, in luma samples arrays of the i-th display overlay in the target display picture. The length of the syntax elements is doi_size_param_length_minus1 + 1 bits.

The variables CodedOverlayTexture[ i ] and CodedOverlayAlpha[ i ] are picture sample arrays with luma resolution CodedOverlayWidth[ i ] × CodedOverlayHeight[ i ], derived as follows:

–	If doi_pic_partition_flag is equal to 0, the following applies:

–	CodedOverlayWidth[ i ] is set equal to PicWidthInLumaSamples[ doi_nuh_layer_id[ i ] ] ].

–	CodedOverlayHeight[ i ] is set equal to PicHeightInLumaSamples[ doi_nuh_layer_id[ i ] ].

–	If there is a picture in the AU for the layer with layer identifier doi_nuh_layer_id[ i ], CodedOverlayTexture[ i ] is set equal to the cropped decoded picture from the layer with layer identifier doi_nuh_layer_id[ i ] in the AU. Otherwise (there is no picture in the AU for the layer with layer identifier doi_nuh_layer_id[ i ]), CodedOverlayTexture[ i ] is set equal to the previous cropped decoded picture in output order in the layer with layer identifier doi_nuh_layer_id[ i ].

–	Otherwise, the following applies:

–	If doi_alpha_present_flag[ i ] equal to 1, the following applies:

- If there is a picture in the AU for the layer with layer identifier doi_alpha_nuh_layer_id[ i ], CodedOverlayAlpha[ i ]  is set equal to the cropped decoded picture from the layer with layer identifier doi_alpha_nuh_layer_id[ i ] in the AU.

- Otherwise (there is no picture in the AU for the layer with layer identifier doi_nuh_layer_id[ i ]), CodedOverlayAlpha[ i ] is set equal to the previous cropped decoded picture in output order in the layer with layer identifier doi_alpha_nuh_layer_id[ i ].

–	Otherwise, if doi_partition_type_flag is equal to 0, the following applies:

–	CodedOverlayTexture[ i ] is set equal to the subpicture with subpicture index doi_partition_id[ i ] from the layer with layer identifier doi_nuh_layer_id[ i ].

–	CodedOverlayWidth[ i ] is set equal to SubPicWidth[ doi_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayHeight[ i ] is set equal to SubPicHeight[ doi_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	If doi_alpha_present_flag[ i ] equal to 0, the following applies:

–	CodedOverlayAlphaWidth[ i ]  is set equal to SubPicWidth[ doi_alpha_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayAlphaHeight[ i ]  is set equal to SubPicHeight[ doi_alpha_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayAlpha[ i ]  is set equal to the subpicture with subpicture index doi_alpha_partition_id[ i ] from the layer with layer identifier doi_alpha_nuh_layer_id[ i ].

–	Otherwise (doi_partition_type_flag is equal to 1), the following applies:

–	CodedOverlayWidth[ i ] is set equal to CrRectWidth[ doi_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayHeight[ i ] is set equal to CrRectHeight[ doi_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayTexture[ i ] is set equal to the constituent rectangle with cr_rect_id[ j ] equal to doi_partition_id[ i ] from the layer with layer identifier doi_nuh_layer_id[ i ].

–	If doi_alpha_present_flag[ i ] equal to 0

–	CodedOverlayAlphaWidth[ i ] is set equal to CrWRectidth[ doi_alpha_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayAlphaHeight[ i ] is set equal to CrRectHeight[ doi_alpha_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayAlpha is set equal to the constituent rectangle with cr_rect_id[ j ] equal to doi_alpha_partition_id[ i ] from the layer with layer identifier doi_alpha_nuh_layer_id[ i ].

The variables DisplayOverlayTexture[ i ] and DisplayOverlayAlpha[ i ] are sample arrays with luma resolution DisplayOverlayWidth[ i ] × DisplayOverlayHeight[ i ], derived as follows:

–	If doi_resampling_enabled_flag is equal to 1, the following applies:

–	DisplayOverlayWidth[ i ]  is set equal to doi_width_minus1[ i ] + 1.

–	DisplayOverlayHeight[ i ] is set equal to doi_height_minus1[ i ] + 1.

–	OverlayTexture[ i ] is derived by resampling CodedOverlayTexture[ i ] from a luma resolution of ( CodedOverlayWidth[ i ] × CodedOverlayHeight[ i ] ) to a luma resolution of ( DisplayOverlayWidth[ i ] × DisplayOverlayHeight[ i ] ).

–	OverlayAlpha[ i ] is derived by resampling CodedOverlayAlpha from a luma resolution of ( CodedOverlayAlphaWidth[ i ] × CodedOverlayAlphaHeight[ i ] ) to a luma resolution of ( DisplayOverlayWidth[ i ]  × DisplayOverlayHeight[ i ] )

–	Otherwise (doi_resampling_enabled_flag is equal to 0), the following applies:

–	DisplayOverlayWidth[ i ] is set equal to CodedOverlayWidth[ i ].

–	DisplayOverlayHeight[ i ] is set equal to CodedOverlayHeight[ i ].

– 	OverlayTexture[ i ] is set equal to CodedOverlayTexture[ i ].

– 	OverlayAlpha[ i ] is set equal to CodedOverlayAlpha[ i ].

The variables TargetPicWidth and TargetPicHeight, for the target display picture width and height, respectively, are derived as follows:

–	If doi_target_pic_size_present_flag)

–	TargetPicWidth is set equal to doi_target_pic_width_minus1 + 1

–	TargetPicHeight is set equal to doi_target_pic_height_minus1 + 1

–	Otherwise

–	TargetPicWidth is set equal to DisplayOverlayWidth[ 0 ].

–	TargetPicHeight is set equal to DisplayOverlayHeight[ 0 ].

Let OverlayWithAlpha( tgtPic[ x ][ y ], ovlTex[ w ][ h ], ovlAlp[ w ][ h ] ) be specified as a function that returns the sample values derived by applying the alpha channel using tgtPic[ x ][ y ] as the background sample, ovlTex[ w ][ h ] as the foreground sample, and ovlAlp[ w ][ h ] as the alpha channel sample, using the process specified in subclause 8.23 if an alpha channel information SEI message is present in the CVS, or using a process determined via external means.

A target display picture should be formed as follows, with picture arrays, TargetPicture[ cIdx ][ x ][ y ] , with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..( cIdx = = 0 ) ? TargetPicWidth : TargetPicWidth / SubWidthC − 1, y = 0..( cIdx = = 0 ) ? TargetPicHeight : TargetPicHeight / SubHeightC − 1.

The target display picture array values are derived as follows:

for( cIdx = 0; cIdx <= 2 * doi_target_init_three_comp_flag; cIdx++ )

for( y = 0;  y < ( ( cIdx  = =  0 ) ? TargetPicHeight : TargetPicHeight / SubHeightC ); y++ )

for( x = 0; x <  ( ( cIdx  = =  0 ) ? TargetPicWidth : TargetPicWidth / SubWidthC ); x++ )

TargetPicture[ cIdx ][ x ][ y ] = doi_target_init_sample[ cIdx ]

for( i = 0; i < doi_num_display_overlays_minus2 + 2; i++ )  {
	for( h = 0, y = doi_top_left_y[ i ]; y < DisplayOverlayHeight[ i ]; h++, y++ ) 
		for( w = 0, x = doi_top_left_x[ i ]; x < DisplayOverlayWidth[ i ]; w++, x++ )
			if( !doi_alpha_present_flag[ i ] )
				TargetPicture[ 0 ][ x ][ y ] = OverlayTexture[ i ][ 0 ][ w ][ h ]	
			else
				TargetPicture[ c ][ x ][ y ] = OverlayWithAlpha( TargetPicture[ 0 ][ x ][ y ], 
						OverlayTexture[ 0 ][ i ][ w ][ h ], OverlayAlpha[ i ][ w ][ h ] )

for( ( cIdx = 1; cIdx < ChromaFormatIdc = = 0 ) ? 1 : 3; cIdx++ ++ )  {
		for( h = 0, y = doi_top_left_y[ i ]/SubHeightC; y < DisplayOverlayHeight[ i ]/ SubHeightC; h++, y++ ) 
			for( w = 0, x = doi_top_left_x[ i ]/SubWidthC; x < DisplayOverlayWidth[ i ]/ SubWidthC; w++, x++ )
				if( !doi_alpha_present_flag[ i ] )
					TargetPicture[ cIdx ][ x ][ y ] = OverlayTexture[ i ][ cIdx ][ w ][ h ]					
				else
					TargetPicture[ cIdx ][ x ][ y ] = OverlayWithAlpha( TargetPicture[ cIdx ][ x ][ y ], 
							OverlayTexture[ cIdx ][ i ][ w ][ h ], OverlayAlpha[ i ][ w ][ h ] )

# Conclusion

This contribution proposes to properly signal in the DOI SEI how the target picture should be initialized. With the proposed changes, the formation of the target picture would be fully under control, without potential unexpected behaviors when overlays with alpha are applied on top of uninitialized data.  Furthermore, it enables more options in terms of overlay creation, with the possibility to directly built from a uniform basis.

It is proposed to adopt the proposed solution in the VSEI TMUC to solve the identified issue.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).