| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0243-v1 |
| --- | --- |

| Title: | AHG9: Display overlay information SEI flags for full occupancy and suitability for display |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Thibaud Biatek Miska M. Hannuksela | Email: | {jill.boyce, thibaud.biatek, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is proposed to add flags to display overlay information SEI message to indicate the following:

- A target picture full occupancy flag to indicate that all sample locations in the target picture are occupied by at least one display overlay

- A display overlay complete flag per display overlay to indicate that sample locations in the display overlay are suitable for display even when there is a higher order display overlay representing the same sample location

- A layer order constrained flag to constrain the coding order of layers in an AU to match the display overlay index order

# Proposal

The Display overlays information SEI message is included in the VSEI TuC document in JVET-AI2032.

The DOI SEI message does not require that all sample locations in the formed target picture be represented by at least one display overlay. It is proposed to signal a flag, doi_target_pic_full_occ_flag, to indicate that all sample locations in the target picture are occupied by one or more display overlays. This information may be useful for a decoding system in determining whether to form and display the target picture using the contents of the DOI SEI.

When a DOI SEI target picture is formed, sample values at some locations represented by a lower order display overlay may be replaced by sample values for the same location in a higher order display overlay. An encoder may choose to reduce bitrate in those sample locations. It is proposed to signal a display overlay complete flag per display overlay, doi_overlay_complete_flag[ i ], to indicate that all sample locations represented by the display overlay are suitable for display.

The target picture formation process defined in the DOI SEI message applies the display overlays to the target picture in index order. However, there is no requirement that coding order of pictures in an access unit follows the index order, so decoders are not guaranteed to be able to apply a display overlay to the target picture immediately after decoding the picture. The doi_layer_order_constrained_flag is proposed to indicate a constraint that the coding order of pictures in an AU, when present, follow the index order.

Proposed syntax changes are highlighted in yellow.

| display_overlays_info( payloadSize ) { | Descriptor |
| --- | --- |
| doi_id | u(6) |
| doi_cancel_flag | u(1) |
| if( !doi_cancel_flag ) { |  |
| doi_persistence_flag | u(1) |
| doi_num_display_overlays_minus2 | ue(v) |
| doi_target_pic_size_present_flag | u(1) |
| if( doi_target_pic_size_present_flag ) |  |
| doi_target_pic_width_minus1 | u(16) |
| doi_target_pic_height_minus1 | u(16) |
| doi_target_pic_full_occ_flag | u(1) |
| } |  |
| doi_nuh_layer_id_present_flag | u(1) |
| if( doi_nuh_layer_id_present_flag ) |  |
| doi_layer_order_constrained_flag | u(1) |
| doi_pic_partition_flag | u(1) |
| if( doi_pic_partition_flag ) { |  |
| doi_partition_type_flag | u(1) |
| doi_partition_id_len_minus1 | u(4) |
| } |  |
| doi_offset_params_present_flag | u(1) |
| if( doi_offset_params_present_flag ) |  |
| doi_offset_param_length_minus1 | u(4) |
| doi_resampling_enabled_flag | u(1) |
| if( doi_resampling_enabled_flag ) |  |
| doi_size_param_length_minus1 | u(4) |
| for( i = 0; i < doi_num_display_overlays_minus2 + 2; i++ ) { |  |
| if ( i != doi_num_display_overlays_minus2 + 1) |  |
| doi_overlay_complete_flag[ i ] | u(1) |
| if ( doi_nuh_layer_id_present_flag ) |  |
| doi_nuh_layer_id[ i ] | u(6) |
| if ( doi_pic_partition_flag ) |  |
| doi_partition_id[ i ] | u(v) |
| doi_alpha_present_flag[ i ] | u(1) |
| if ( doi_alpha_present_flag[ i ] ) { |  |
| if (doi_nuh_layer_id_present_flag) |  |
| doi_alpha_nuh_layer_id[ i ] | u(6) |
| if (doi_pic_partition_flag) |  |
| doi_alpha_partition_id[ i ] | u(v) |
| } |  |
| if( i > 0 ) { |  |
| if (doi_offset_params_present_flag ) |  |
| doi_top_left_x[ i ] | u(v) |
| doi_top_left_y[ i ] | u(v) |
| } |  |
| if( doi_resampling_enabling_flag ) { |  |
| doi_width_minus1[ i ] | u(v) |
| doi_height_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

doi_target_pic_full_occ_flag equal to 1 indicates that all sample positions in the target picture have at least one display overlay for which the i-th display overlay representation includes a sample that occupies the sample position. doi_target_pic_full_occ_flag equal to 0 does not apply a constraint.

doi_layer_order_constrained_flag equal to 1 specifies a constraint on the values of doi_nuh_layer_id[ i ] and doi_alpha_nuh_layer_id[ i ]. doi_layer_order_constrained_flag equal to 0 does not impose a constraint.

doi_nuh_layer_id_present_flag equal to 1 specifies that the doi_nuh_layer_id[ i ] syntax element is present in the SEI message. doi_nuh_layer_id_present_flag equal to 0 specifies that the doi_nuh_layer_id[ i ] syntax element is not present in the SEI message.

doi_overlay_complete_flag[ i ] equal to 0 indicates that one or more samples represented by the i-th display overlay may contain sample values that are not suitable for display in sample locations represented by a j-th display overlay, with j > i.

doi_overlay_complete_flag[ i ] equal to 1 indicates that all samples represented by the i-th display overlay are suitable for display.

When not present, the value of doi_overlay_complete_flag[ i ] is inferred to be equal to 1.

…

doi_nuh_layer_id[ i ], when present, specifies the layer identifier of the texture component of the i-th display overlay. When not present, the value of doi_nuh_layer_id[ i ] is inferred to be equal to the layer identifier of the PU containing the DOI SEI message.

If this SEI message is present in any layer in the current AU, it is a requirement of bitstream conformance that a DOI SEI message with the same value of doi_id and the same payload is present in the layer with layer identifier doi_nuh_layer_id[ 0 ].

When doi_layer_order_constrained_flag equal to 1, it is a requirement of bitstream conformance that when  a picture with layer identifier equal to doi_nuh_layer_id[ i ] is present in an AU, for any value j less than i, there shall not be a picture with layer identifier doi_nuh_layer_id[ j ] that follows the picture with layer identifier equal to doi_nuh_layer_id[ i ] in decoding order present in the AU.

…

doi_alpha_nuh_layer_id[ i ], when present, specifies the layer identifier value of the alpha component of the i-th display overlay. When not present, the value of doi_alpha_nuh_layer_id[ i ] is inferred to be equal to the layer identifier of the PU containing the DOI SEI message.

When doi_layer_order_constrained_flag equal to 1, it is a requirement of bitstream conformance that when  a picture with layer identifier equal to doi_alpha_nuh_layer_id[ i ] is present in an AU, for any value j less than i, there shall not be a picture with layer identifier doi_alpha_nuh_layer_id[ j ] that follows the picture with layer identifier equal to doi_alpha_nuh_layer_id[ i ] in decoding order present in the AU.

…

doi_width_minus1[ i ] plus 1 and doi_height_minus1[ i ] plus 1, when present, specify the width and height, respectively, in luma samples arrays of the i-th display overlay in the target display picture. The length of the syntax elements is doi_size_param_length_minus1 + 1 bits.

When doi_target_pic_full_occ_flag equal to 1, it is a requirement of bitstream conformance that for each (x, y) sample position for x in 0 .. doi_target_pic_width_minus1, y in 0 .. doi_target_pic_height_minus1, there is at least one value of i in 0 .. doi_num_display_overlays_minus2 + 1 for which the following two conditions are met:

–	doi_top_left_x[ i ] <= x < doi_top_left_x[ i ] + doi_width_minus1[ i ]

–	doi_top_left_y[ i ] <= y < doi_top_left_x[ i ] + doi_height_minus1[ i ]

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).