| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0244-v1 |
| --- | --- |

| Title: | AHG9: Guard bands for Constituent Rectangles SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela | Email: | {jill.boyce, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is proposed to extend the Constituent Rectangles (CR) SEI message to provide support for signaling of guard bands between rectangles for “same size” signaling of rectangle sizes and locations.

# Proposal

The constituent rectangles SEI message in the TuC in JVET-AI2032 provides a flexible standardized method to describe multiple “rectangles” composited within a single coded picture. Neighboring constituent rectangles may differ significantly and may benefit from having guard bands of samples between them in a coded picture.

The CR SEI provides several options to indicate the size and location of rectangles:

- using subpicture parameters

- “same size” rectangles with # rows and # cols signalled

- explicit signalling of size and location

The “same size” option is more bitrate efficient than the explicit signalling when all CRs in the coded picture are the same size and arranged in a grid. The “same size” signaling option cannot currently be used with guard bands.

The explicit signaling option of size and location already provides support for guard bands. This contribution proposes adding support for guard bands for “same size” rectangle signalling, to save bitrate vs.  explicit signalling.

It is also noted that the subpicture signaling option does not support guard bands, but this contribution does not attempt to address the subpicture signaling option.

Figure 1 shows an example coded picture with 2 CRs, representing a texture picture and a depth picture, using same size signalling on the left without a guard band, and on the right with a guard band between the CRs.

Figure 1. Picture with 2 constituent rectangles, with no guard band on left and a guard band on right

The changes proposed are summarized as follows:

- Add cr_guardbands_present_flag

- Add cr_guardband_hor_size_minus1 and cr_guardband_ver_size_minus1 syntax elements, conditioned on cr_rect_same_size_flag and cr_guardbands_present_flag

- Modify the process of deriving the position of and size of CRs when cr_rect_same_size_flag is equal to 1, to incorporate the cr_guardband_hor_size_minus1 and cr_guardband_ver_size_minus1 syntax elements

# Specification modifications

The proposed modifications to the CR SEI syntax and semantics are highlighted below in yellow.

| constituent_rectangles( payloadSize ) { | Descriptor |
| --- | --- |
| cr_num_rects_minus1 | u(12) |
| cr_rect_id_enabled__flag | u(1) |
| cr_associated_rect_id_enabled_flag | u(1) |
| cr_rect_group_id_present_flag | u(1) |
| if ( cr_rect_d_present_flag ¦ ¦ cr_associated_rect_id_enabled_flag ¦ ¦ cr_rect_group_id_present_flag ) |  |
| cr_rect_id_len | u(4) |
| cr_rect_type_enabled_flag | u(1) |
| cr_rect_type_descriptions_enabled_flag | u(1) |
| cr_subpics_partitioning_flag | u(1) |
| if( !cr_subpics_partitioning_flag ) { | u(1) |
| cr_rect_same_size_flag | u(1) |
| if ( cr_rect_same_size_flag ) { |  |
| cr_num_cols_minus1 | ue(v) |
| cr_num_rows_minus1 | ue(v) |
| cr_guardbands_present_flag | u(1) |
| if ( cr_guardbands_present_flag ) { |  |
| cr_guardband_hor_size_minus1 | ue(v) |
| cr_guardband_ver_size_minus1 | ue(v) |
| } |  |
| } else { |  |
| cr_log2_unit_size | u(4) |
| cr_rect_size_len_minus1 | u(4) |
| } |  |
| } |  |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) { |  |
| if ( cr_rect type_enabled_flag ) { |  |
| cr_rect_type_present_flag[ i ] | u(1) |
| if ( cr_rect_type_present_flag[ i ] ) |  |
| cr_rect_type_idc[ i ] | u(8) |
| } |  |
| if ( cr_rect type_idc[ i ] != 255 ) { |  |
| if ( cr_rect id_enabled_flag ) { |  |
| cr_rect_type_present_flag[ i ] | u(1) |
| if ( cr_rect id_present_flag ) |  |
| cr_rect_id[ i ] | u(v) |
| } |  |
| if ( cr_associated_rect id_enabled_flag ) { |  |
| cr_associated_rect_id_present_flag[ i ] | u(1) |
| if ( cr_associated_rect_id_present_flag[ i ] ) |  |
| cr_associated_rect_id[ i ] | u(v) |
| } |  |
| if ( cr_group_id_present_flag ) { |  |
| cr_rect_group_id[ i ] | u(v) |
| if ( cr_rect type_description_enabled_flag ) |  |
| cr_rect_type_description_present_flag[ i ] | u(1) |
| } |  |
| if( !cr_subpics_partitioning_flag && !cr_rects_same_size_flag ) { |  |
| cr_rect_top_left_in_units_x[ i ] | u(v) |
| cr_rect_top_left_in_units_y[ i ] | u(v) |
| cr_rect_width_in_units_minus1[ i ] | u(v) |
| cr_rect_height_in_units_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( cr_rect_type_descriptions_enabled_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| cr_bit_equal_to_zero /* equal to 0 */ | f(1) |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) |  |
| if( rect_type_description_present_flag[ i ] ) |  |
| cr_rect_type_description[ i ] | st(v) |
| } |  |
| } |  |

cr_guardband_hor_size_minus1 plus 1 specifies the size of horizontal guard bands between rectangles when cr_rect_same_size_flag is equal to 1 in units of luma samples.

The variable GuardbandHor is set equal to cr_guardbands_present_flag ? cr_guardband_hor_size_minus1 + 1 : 0.

It is a requirement of bitstream conformance that GuardbandHor %  SubWidthC shall be equal to 0.

cr_guardband_ver_size_minus1 plus 1 specifies the size of vertical guard bands between rectangles when cr_rect_same_size_flag is equal to 1 in units of luma samples.

The variable GuardbandVer is set equal to cr_guardbands_present_flag ? cr_guardband_ver_size_minus1 + 1 : 0.

It is a requirement of bitstream conformance that GuardbandVer %  SubHeightC shall be equal to 0.

…

Otherwise (cr_rect_same_size_flag is equal to 1), the following applies:

–	The variable crRectTopLeftX[ i ] is set equal to (i % crNumCols ) * MaxPicWidth / crNumCols

–	The variable crRectTopLeftY[ i ] is set equal to (i / crNumCols ) * MaxPicHeight / crNumRows

–	The variable crRectWidth [ i ] is set equal to MaxPicWidth / crNumCols

–	The variable crRectHeight [ i ] is set to equal to MaxPicHeight / crNumRows

–	The variable currRow is set equal to i % crNumCols

–	The variable currCol is set equal to i / crNumCols

–	The variable crRectWidth [ i ] is set equal to ( MaxPicWidth −

GuardbandHor * ( crNumCols – 1 ) ) / crNumCols

–	The variable crRectHeight [ i ] is set to equal to MaxPicHeight −

GuardbandVer * ( crNumRows – 1 ) ) / crNumRows

–	The variable crRectTopLeftX[ i ] is set equal to currCol * ( crRectWidth + GuardbandHor )

–	The variable crRectTopLeftY[ i ] is set equal to currRow * ( crRectHeight + GuardbandVer )

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).