| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0245-v1 |
| --- | --- |

| Title: | AHG9: Multi-layer support for Constituent Rectangles SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela | Email: | {jill.boyce, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is proposed to extend the constituent rectangles (CR) SEI message to add support for multiple layers, with a common CR identifier numbering space. This contribution proposes to signal the number of layers for which the CR SEI message applies and the layer identifier (nuh_layer_id) for each layer index.  Parameters associated with the partitioning of the picture into CRs are also proposed to be signaled per layer.

# Proposal

The CR SEI message in the TuC in JVET-AI2032 describes partitioning of a picture into constituent rectangles. The CR index numbering space associated with the constituent rectangles described in an individual SEI message is limited to a single layer.

Some applications which use constituent rectangles may benefit from using multiple layers, such as those with large number of rectangles or using scalability.

The CR index numbering space is proposed to apply across multiple layers, rather than having a separate CR index numbering space per layer.

The proposed changes are summarized as follows:

- Add a syntax element to indicate the number of layers

- Add a syntax element to indicate the layer ID for each layer index

- Add a syntax element to indicate the number of CRs per layer

- Add a loop per layer to signal per rectangle syntax elements, and change those syntax element and related variables to be arrays with an array index of [ lIdx ]

# Syntax and semantics changes

  - Modifications to VSEI

The modifications to the syntax and semantics are highlighted in cyan.

An unrelated editing bug fix is highlighted in yellow.

| constituent_rectangles( payloadSize ) { | Descriptor |
| --- | --- |
| cr_num_rects_minus1 | u(12) |
| cr_num_layers_minus1 | ue(v) |
| for( lIdx = 0; lIdx <= cr_num_layers_minus1; lIdx ++ ) { |  |
| cr_layer_id[ lIdx ] | u(6) |
| cr_num_rects_in_layer_minus1[ lIdx ] | u(8) |
| } |  |
| cr_rect_id_enabled_flag | u(1) |
| cr_associated_rect_id_enabled_flag | u(1) |
| cr_rect_group_id_present_flag | u(1) |
| if( cr_rect_id_present_flag ¦ ¦ cr_associated_rect_id_enabled_flag ¦ ¦ cr_rect_group_id_present_flag ) |  |
| cr_rect_id_len | u(4) |
| cr_rect_type_enabled_flag | u(1) |
| cr_rect_type_descriptions_enabled_flag | u(1) |
| for( lIdx = 0, i = 0; lIdx <= cr_num_layers_minus1; lIdx++ ) { |  |
| cr_subpics_partitioning_flag[ lIdx ] | u(1) |
| if( !cr_subpics_partitioning_flag[ lIdx ] ) { | u(1) |
| cr_rect_same_size_flag[ lIdx ] | u(1) |
| if( cr_rect_same_size_flag[ lIdx ] ) { |  |
| cr_num_cols_minus1[ lIdx ] | ue(v) |
| cr_num_rows_minus1[ lIdx ] | ue(v) |
| } |  |
| } else { |  |
| cr_log2_unit_size[ lIdx ] | u(4) |
| cr_rect_size_len_minus1[ lIdx ] | u(4) |
| } |  |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) { |  |
| for( crIdx = 0; crIdx <= cr_num_rects_in_layer_minus1[ lIdx ]; crIdx++, i++ ) |  |
| if( cr_rect type_enabled_flag ) { |  |
| cr_rect_type_present_flag[ i ] | u(1) |
| if( cr_rect_type_present_flag[ i ] ) |  |
| cr_rect_type_idc[ i ] | u(8) |
| } |  |
| if( cr_rect_type_idc[ i ] != 255 ) { |  |
| if( cr_rect id_enabled_flag ) { |  |
| cr_rect_id_present_flag[ i ] | u(1) |
| if( cr_rect id_present_flag[ i ] ) |  |
| cr_rect_id[ i ] | u(v) |
| } |  |
| if( cr_associated_rect_id_enabled_flag ) { |  |
| cr_associated_rect_id_present_flag[ i ] | u(1) |
| if( cr_associated_rect_id_present_flag[ i ] ) |  |
| cr_associated_rect_id[ i ] | u(v) |
| } |  |
| if( cr_rect_group_id_present_flag ) |  |
| cr_rect_group_id[ i ] | u(v) |
| if( cr_rect type_descriptions_enabled_flag ) |  |
| cr_rect_type_description_present_flag[ i ] | u(1) |
| } |  |
| if( !cr_subpics_partitioning_flag[ lIdx ] && !cr_rects_same_size_flag[ lIdx ] ) { |  |
| cr_rect_top_left_in_units_x[ lIdx ][ i ] | u(v) |
| cr_rect_top_left_in_units_y[ lIdx ][ i ] | u(v) |
| cr_rect_width_in_units_minus1[ lIdx ][ i ] | u(v) |
| cr_rect_height_in_units_minus1[ lIdx ][ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( cr_rect_type_descriptions_enabled_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| cr_bit_equal_to_zero /* equal to 0 */ | f(1) |
| for( i = 0; i < CrNumRects = cr_num_rects_minus1; i++ ) |  |
| if( rect_type_description_present_flag[ i ] ) |  |
| cr_rect_type_description[ i ] | st(v) |
| } |  |
| } |  |

The constituent rectangles SEI message enables composition of multiple rectangles within a coded picture and provides information about the rectangles, including ID, type, text description, location, and size.

Use of this SEI message requires the definition of the following variables:

–	A picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples[ lId ] and PicHeightInLumaSamples[ lId ], respectively.

–	A maximum picture width and maximum picture height in units of luma samples, denoted herein by MaxPicWidth[ lId ]  and MaxPicHeight[ lId ], respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3.

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY, and when ChromaFormatIdc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein by BitDepthC.

–	A subpicture count, denoted by NumSubpics[ lId ].

–	Arrays of the top left X and Y positions of the subpictures, denoted herein by SubPicTopLeftX[ i ] and SubPicTopLeftY[ i ], respectively.

–	Arrays of the width and height of the subpictures, denoted herein by SubPicTopLeftX[ i ] and SubPicTopLeftY[ i ], respectively.

If this SEI message is present in any picture unit that is not the first picture unit of a CLVS in decoding order, a constituent rectangles SEI message with the same payload content shall be present in the first picture unit of the CLVS in decoding order.

cr_num_rects_minus1 plus 1 specifies the number of constituent rectangles for which information is signalled in the SEI message.

cr_num_layers_minus1 plus 1 specifies the number of layers for which constituent rectangles are described in the SEI message.

cr_layer_id[ lIdx ] specifies the layer identifier of the lIdx-th layer.

The variable lId is set equal to cr_layer_id[ lIdx ].

cr_num_rects_in_layer_minus1[ lIdx ] plus 1 specifies the number of the number of constituent rectangles in the lIdx-th layer for which information is signalled in the SEI message.

The variable CrNumRects is derived as follows:

CrNumRects = 0

for( lIdx= 0; lIdx <= cr_num_layers_minus1; lIdx ++ )

CrNumRects += cr_num_rects_in_layer[ lIdx ] + 1

cr_rect_id_enabled_flag equal to 1 specifies that the cr_rect_id_present_flag syntax element is present in the SEI message. cr_rect_id_present_flag equal to 0 specifies that the cr_rect_id_present_flag syntax element is not present in the SEI message.

cr_associated_rect_id_enabled_flag equal to 1 specifies that the cr_associated_rect_id_present_flag[ i ] syntax elements are present. cr_associated_rect_id_enabled_flag equal to 0 specifies that the cr_associated_rect_id_present_flag[ i ] syntax elements are not present.

cr_rect_group_id_present_flag equal to 1 specifies that cr_rect_group_id[ i ] syntax elements are present. cr_rect_group_present_flag equal to 0 specifies that_cr_rect_group_id[ i ] syntax elements are not present.

cr_rect_id_len specifies the length of the cr_rect_id[ i ],  cr_associated_rect_id [ i ], and cr_rect_group_id[ i ] syntax elements. syntax elements.

cr_rect_type_enabled_flag equal to 1 specifies that the cr_rect_type_present_flag[ i ] syntax element is present in the SEI message. cr_rect_type_enabled_flag equal to 0 specifies that the cr_rect_type_ present_flag[ i ] syntax element is not present in the SEI message.

cr_rect_type_descriptions_enabled_flag equal to 1 specifies that the cr_rect_type_description_present_flag[ i ] syntax element is present in the SEI message. cr_rect_type_descriptions_enabled_flag equal to 0 specifies that the cr_rect_type_description_present_flag[ i ] syntax element is not present in the SEI message.

cr_subpics_partitioning_flag[ lIdx ]  equal to 1 indicates that the subpic partitioning parameters in the SPS of the lIdx-th layer are used to determine the of constituent rectangle sizes and positions. cr_subpics_partitioning_flag[ lIdx ] equal to 0 indicates that determination of the constituent rectangle sizes and positions is not based on subpic partitioning parameters in the SPS of the lIdx-th layer.

When cr_subpics_partitioning_flag[ lIdx ] is equal to 1, it is a requirement of bitstream conformance that NumSubpics[ lId ] be greater than or equal to cr_num_rects_in_layer_minus1[ lIdx ] + 1.

cr_rect_same_size_flag[ lIdx ]  equal to 1 indicates that all constituent rectangles in the lIdx-th layer have the same size and are arranged in a grid pattern. cr_rect_same_size_flag[ lIdx ] equal to 0 indicates that the sizes of the constituent rectangles may differ.

cr_num_cols_minus1[ lIdx ]  plus 1 and cr_num_rows_minus1[ lIdx ]  plus 1 specify the number of columns and rows, respectively, of the constituent rectangle grid of the lIdx-th layer when cr_rect_same_size_flag[ lIdx ]  equal to 1.

The variable crNumCols[ lIdx ] is set equal to cr_num_cols_minus1[ lIdx ] + 1.

The variable crNumRows[ lIdx ] is set equal to cr_num_rows_minus1[ lIdx ] + 1.

cr_log2_unit_size[ lIdx ] specifies a unit size used in variable calculations for the constituent rectangle parameters of the lIdx-th layer.

The variable crUnitSize[ lIdx ]  is set equal to 1 << cr_log2_unit_size[ lIdx ].

cr_rect_size_len_minus1[ lIdx ]  plus 1 specifies the length of syntax elements cr_rect_top_left_in_units_x[ lIdx ][ i ], cr_rect_top_left_in_units_y[ lIdx ][ i ], cr_rect_width_in_units_minus1[ lIdx ][ i ], and cr_rect_height_in_units minus1[ lIdx ][ i ].

cr_rect_type_present_flag[ i ] equal to 1 specifies that the cr_rect_type_idc[ i ] syntax element is present in the SEI message. cr_rect_type_present_flag[ i ] equal to 0 specifies that the cr_rect_type_idc[ i ] syntax element is not present in the SEI message.

cr_rect_type_idc[ i ] indicates the constituent picture type of the i-th rectangle from Table X. When not present and i equal to 0, the value of cr_rect_type_idc[ i ] is inferred to be equal to 0. When not present and i greater than 0, cr_rect_type_idc[ i ] is inferred to be equal to cr_rect_type_idc[ i − 1 ].

Table X – Mapping of cr_rect_type_idc[ i ] to the type of constituent rectangle

| cr_rect_type_idc[ i ] | Name | Type of constituent rectangle |
| --- | --- | --- |
| 0 | CR_TEXTURE | Texture |
| 1 | CR_ALPHA | Alpha plane |
| 2 | CR_DEPTH | Depth picture |
| 3 | CR_OBJECT_MASK | Object mask |
| 4 .. 127 |  | Reserved |
| 128..254 |  | Unspecified |
| 255 | CR_EMPTY | Empty |

cr_rect_id_present_flag equal to 1 specifies that the cr_rect_id[ i ] syntax element is present in the SEI message. cr_rect_id_present_flag equal to 0 specifies that the cr_rect_id[ i ] syntax element is not present in the SEI message.

cr_rect_id[ i ] indicates the ID of the i-th rectangle. The length of the syntax element is cr_rect_id_len bits. When not present and i equal to 0, the value of cr_rect_id[ i ] is inferred to be equal to 0. When not present and i greater than 0, the value of cr_rect_id[ i ] is inferred to be equal to cr_rect_id[ i – 1 ] + 1.

It is a requirement of bitstream conformance that when j not equal to k, cr_rect_id[ j ] shall not be equal to cr_rect_id[ k ].

cr_associated_rect_id_present_flag equal to 1 specifies that the cr_associated_rect_id[ i ] syntax element is present in the SEI message. cr_rect_id_present_flag equal to 0 specifies that the cr_associated_rect_id[ i ] syntax element is not present in the SEI message.

cr_associated_rect_id[ i ] indicates the ID of an associated primary rectangle of the i-th rectangle. The length of the syntax element is cr_rect_id_len bits.

It is a requirement of bitstream conformance that cr_associated_rect_id[ i ] equal cr_rect_id[ j ] for some value of j in 0 .. CrNumRects − 1 cr_num_rects_minus1.

When not present, the value of cr_associated_rect_id[ i ] is undefined.

cr_rect_group_id[ i ] indicates the group ID of the i-th rectangle. The length of the syntax element is cr_rect_id_len bits. When not present, the value of cr_rect_group_id[ i ] is inferred to be equal to 0.

cr_rect_type_description_present_flag[ i ] equal to 1 specifies that the cr_rect_type_description[ i ] syntax element is present in the SEI message. cr_rect_type_description_present_flag[ i ] equal to 0 specifies that the cr_rect_type_description[ i ] syntax element is not present in the SEI message. When not present, the value of cr_rect_type_description_present_flag[ i ] is inferred to be equal to 0.

cr_rect_top_left_in_units_x[ lIdx ]  [ i ] and cr_rect_top_left_in_units_y[ lIdx ]  [ i ], when present,  indicate the horizontal and vertical positions, respectively, of the top left position of the i-th constituent picture rectangle in units of the lIdx-th layer. The length of the syntax elements are cr_rect_size_len_minus1[ lIdx ]  + 1.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

cr_rect_width_in_units_minus1[ lIdx ] [ i ] plus 1 and cr_rect_height_in_units_minus1[ lIdx ][ i ] plus 1, when present, indicate the width and height, respectively, of the i-th constituent rectangle in units.  The length of the syntax elements are cr_rect_size_len_minus1[ lIdx ]  + 1.

The variables crRectTopLeftX[ lIdx ][ i ] and crRectTopLeftY[ lIdx ][ i ], representing the x and y location, respectively, and variables crRectWidth[ i ] and crRectHeight[ i ], representing the width and height, respectively, in the lIdx-th layer of the i-th constituent rectangle are derived as follows.

If cr_subpics_partitioning_flag[ lIdx ]   is equal to 0 and cr_rect_same_size_flag[ lIdx ]   is equal to 0, the following applies:

–	The variable crRectTopLeftX[ lIdx ][ i ] is set equal to cr_rect_top_left_in_units_x[ lIdx ][ i ] * crUnitSize[ lIdx ]

–	The variable crRectTopLeftY[ lIdx ][  [ i ] is set equal to cr_rect_top_left_in_units_y[ lIdx ][ i ] * crUnitSize[ lIdx ]

–	The variable crRectWidth[ lIdx ][ i ] is set equal to ( cr_rect_width_in_units_minus1[ lIdx ][ i ] + 1 ) * crUnitSize[ lIdx ]

–	The variable crRectHeight[ lIdx ][ i ] is set equal to ( cr_rect_height_in_units_minus1[ lIdx ][ i ] + 1 ) * crUnitSize[ lIdx ]

Otherwise, if cr_subpics_partitioning_flag[ lIdx ]   is equal to 1, the following applies:

–	The variable crRectTopLeftX[ lIdx ] [ i ] is set equal to SubPicTopLeftX[ lIdx ][ i ]

–	The variable crRectTopLeftY[ lIdx ] [ i ] is set equal to SubPicTopLeftY[ lIdx ][ i ]

–	The variable crRectWidth[ lIdx ] [ i ] is set equal to SubPicWidth[ lIdx ][ i ]

–	The variable crRectHeight[ lIdx ] [ i ] is set to equal to SubPicHeight[ lIdx ][ i ]

Otherwise (cr_rect_same_size_flag[ lIdx ] is equal to 1), the following applies:

–	The variable crRectTopLeftX[ lIdx ] [ i ] is set equal to (i % crNumCols[ lIdx ] ) * MaxPicWidth[ lId ] / crNumCols[ lIdx ]

–	The variable crRectTopLeftY[ lIdx ] [ i ] is set equal to (i / crNumCols[ lIdx ] ) * MaxPicHeight[ lId ] / crNumRows[ lIdx ]

–	The variable crRectWidth[ lIdx ] [ i ] is set equal to MaxPicWidth[ lId ] / crNumCols[ lId ]

–	The variable crRectHeight[ lIdx ] [ i ] is set to equal to MaxPicHeight[ lId ] / crNumRows[ lId ]

When PicWidthInLumaSamples[ cr_layer_id[ lIdx ] ] is not equal to MaxPicWidth[ cr_layer_id[ lId ], the following applies:

–	crRectTopLeftX[ lIdx ] [ i ] is set equal to (crRectTopLeftX[ lIdx ] [ i ] * PicWidthInLumaSamples[ cr_layer_id[ lId  ] + MaxPicWidth[ lId ] / 2 ) / MaxPicWidth[ lId ]

–	crRectWidth[ lIdx ] [ i ] is set equal to (crRectWidth[ lIdx ] [ i ] * PicWidthInLumaSamples[ cr_layer_id[ lId ] ] + MaxPicWidth[ lId ] / 2 ) / MaxPicWidth[ lId ]

When PicHeightInLumaSamples[ lIdx ] is not equal to MaxPicHeight[ lId ], the following applies:

–	crRectTopLeftY[ lIdx ] [ i ] is set equal to (crRectTopLeftY[ lIdx ] [ i ] * PicWidthInLumaSamples[ lId ] + MaxPicHeight[ lIdx ] / 2 ) / MaxPicHeight[ lId ]

–	crRectHeight[ i ] [ lIdx ] is set equal to (crRectHeight[ lIdx ] [ i ] * PicWidthInLumaSamples[ lId ] + MaxPicHeigh[ lId ]t / 2 ) / MaxPicHeight[ lId ]

It is a requirement of bitstream conformance that for each sample position (x, y) in the coded picture there shall be at most one rectangle, j, for which both of the following conditions apply:

–	x is in (crRectTopLeftX[ lIdx ] [ j ] .. crRectTopLeftX[ lIdx ] [ j ] + crRectWidth[ lIdx ] [ j ] - 1)

–	y is in (crRectTopLeftY[ lIdx ] [ j ] .. crRectTopLeftY[ lIdx ] [ j ] + crRectHeight[ lIdx ] [ i ] - 1)

It is a requirement of bitstream conformance that crRectTopLeftX[ lIdx ] [ i ] % SubWidthC shall be equal to 0, crRectTopLeftX[ lIdx ] [ i ] % SubHeightC shall be equal to 0, crRectWidth[ lIdx ] [ i ] % SubWidthC shall be equal to 0, and crRectHeight[ lIdx ] [ i ] % SubHeightC shall be equal to 0.

cr_bit_equal_to_zero shall be equal to 0.

cr_rect_type_description[ i ] specifies a text description of the constituent rectangle. The length of the syntax element shall be less than or equal to 4097 bytes, not including the null termination byte.

  - Modifications to VVC

For purposes of interpretation of the constituent rectangles SEI message, the following variables are specified:

–	PicWidthInLumaSamples[ lId ] and PicHeightInLumaSamples[ lId ] are set equal to pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples, respectively, of the picture with nuh_layer_id equal to lId.

–	MaxPicWidth and MaxPicHeight are set equal to sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples, respectively, of the picture with nuh_layer_id equal to lId.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

–	NumSubpics[ lId ]  is set equal to sps_num_subpics_minus1 + 1, of the picture with nuh_layer_id equal to lId.

–	SubPicTopLeftX[ i ] is set equal to sps_subpic_ctu_top_left_x[ i ] * CtbSizeY, of the picture with nuh_layer_id equal to lId.

–	SubPicTopLeftY[ i ] is set equal to sps_subpic_ctu_top_left_y[ i ] * CtbSizeY, of the picture with nuh_layer_id equal to lId.

–	SubPicWidth[ i ] is set equal to (sps_subpic_width_minus1[ i ] + 1 ) * CtbSizeY − 1, of the picture with nuh_layer_id equal to lId.

–	SubPicHeight[ i ] is set equal to (sps_subpic_height_minus1[ i ] + 1 ) * CtbSizeY – 1, of the picture with nuh_layer_id equal to lId.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).