| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0246-v1 |
| --- | --- |

| Title: | AHG15/AHG9: Graphics rendering information SEI for gaming content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Vlad Zakharchenko Sebastian Schwarz Miska M. Hannuksela | Email: | {jill.boyce, vlad.zakharchenko, sebastian.schwarz, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

An SEI message is proposed to carry the graphics rendering parameters documented in JVET-AI2027 “Draft Common Test Conditions (CTC) for gaming applications” for JSON files containing auxiliary information for gaming content. The proposed SEI message supports a variety of game/graphics engines by providing syntax elements to describe the key characteristics that differ among commonly used GPUs and frameworks.

Carrying graphics rendering parameters in an SEI message is asserted to be useful for use cases such as transcoding, image/video analysis, reprojection, and creation of derivative content.

It is also proposed to update the JSON file format used for auxiliary information for gaming content to explicitly include several parameters provided in the proposed SEI message that describe key characteristics of the gaming engine used to generate gaming content, which are not currently included in JVET-AI2027.

# Proposal

A graphics rendering information SEI message is proposed to carry auxiliary information describing parameters of graphics rendering used to create source video content, such as gaming content or AI generated content. Carrying graphics rendering parameters in an SEI message is asserted to be useful for use cases such as transcoding, image/video analysis, reprojection, and creation of derivative content.

Annex B of JVET-AI2027 “Draft Common Test Conditions (CTC) for gaming applications” documents a JSON Text file format for camera parameters. Section 2 of JVET-AI2027 describes an EXR file format to carry per-pixel motion vectors and depth maps per picture.

JVET-AI0054 includes an attachment containing an example JSON file, which includes per frame auxiliary information for gaming content provided by the gaming engine used to generate the source gaming content. The header and first two entries contained in the JSON file attached to JVET-AI0054 are copied in section 3 of this contribution, for convenience.

The header in the example JSON file contains the following fields:

- projectionMatrix

- worldToCameraMatrix

- farClipPlane

- nearClipPlane

- focalLength

The proposed SEI message carries syntax elements to represent the above fields, as well as additional information describing key characteristics of the gaming engine used to generate the content. These include parameters associated with the Class of gaming content described in JVET-AI2027, which are not included in the JSON file.

The proposed SEI message groups parameters into the four categories listed below, each with a presence flag:

- game engine parameters

- projection matrix

- world to camera matrix

- depth map parameters

If the first graphics rendering SEI message applicable to a CLVS does not have the presence flag for a particular category set to 1, default values are inferred for the syntax elements gated by that presence flag. The values of syntax elements persist in decoding order until the graphics rendering SEI message is cancelled or until the syntax element is present in an applicable graphics rendering SEI message.

This persistence method allows groups of parameters that change frequently to be updated while avoiding the need to signal other groups of parameters whose values are not available, are static for the entire sequence, or change infrequently.

Some, but not all, of the information proposed to be provided in the GRI SEI message is related to information provided in the multiview acquisition information (MAI) SEI message and the depth representation information (DRI) SEI message. However, those SEI messages were not designed to address this particular use case. It is asserted that it is more bitrate efficient and simpler for graphics rendering applications to use a dedicated single SEI message, than to use 3 separate SEI messages.

It is proposed to include the graphics rendering SEI message in the TuC. It is also proposed to update the JSON file format defined in the CTC for gaming content to include the parameters in the game engine parameters category.

# Proposed syntax and semantics

| graphics_rendering_info( payloadSize ) { | Descriptor |
| --- | --- |
| gri_cancel_flag | u(1) |
| if ( !gri_cancel_flag ) { |  |
| gri_game_engine_params_present_flag | u(1) |
| gri_projection_matrix_present_flag | u(1) |
| gri_world_to_cam_matrix_present_flag | u(1) |
| gri_depth_params_present_flag | u(1) |
| if( gri_game_engine_parameters_present_flag ) { |  |
| gri_y_axis_up_flag | u(1) |
| gri_right_handed_flag | u(1) |
| gri_normalization_range_flag | u(1) |
| gri_perspective_projection_flag | u(1) |
| gri_data_precision_format_idc | u(2) |
| } |  |
| if( gri_projection_matrix_present_flag ) |  |
| for( i = 0; i < 3; i++ ) |  |
| for( j = 0; j < 4; j++ ) |  |
| gri_projection_coeff[ i ][ j ] | u(v) |
| if( gri_world_to_cam_matrix_present_flag ) |  |
| for( i = 0; i < 3; i++ ) |  |
| for( j = 0; j < 4; j++ ) |  |
| gri_world_to_cam_coeff[ i ][ j ] | u(v) |
| if( gri_depth_params_present_flag ) { |  |
| gri_near_plane | u(v) |
| gri_far_plane | u(v) |
| gri_depth_inverse_flag | u(1) |
| gri_focal_length | u(v) |
| } |  |
| } |  |
| } |  |

The graphics rendering information (GRI) SEI message provides parameters used to create the source content by a graphics renderer.  A GRI SEI message may optionally indicate various parameters for auxiliary pictures of type AUX_DEPTH used to create of the source content by a graphics renderer.

When the SDI SEI message is present in the current CVS, the following applies:

- If the GRI SEI message is present, it shall be present in a primary layer.

- The GRI SEI message additionally applies to auxiliary layers of type AUX_DEPTH associated with the primary layer containing the GRI SEI message.

gri_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any GRI SEI message in output order that applies to the current layerlayer. gri_cancel_flag equal to 0 indicates that graphics rendering information follows.

The graphics rendering information applies applies to the current picture and all subsequent pictures in the same layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in a PU containing a GRI SEI message with gri_cancel_flag equal to 1 is output that follows the current picture in output order.

The variable PrevGriSeiPresentFlag is derived as follows:

–	If gri_cancel_flag equal to 1, the variable PrevGriSeiPresentFlag is set equal to 0.

–	Otherwise, the following applies:

–	If within the current CLVS there is a GRI SEI message present in a PU preceding the current PU in decodingoutput order, PrevGriSeiPresentFlag is set equal to 1

–	Otherwise, PrevGriSeiPresentFlag is set equal to 0

gri_game_engine_params_present_flag equal to 1 indicates that the gri_y_axis_up_flag, gri_left_hand_flag, gri_normalization_range_flag, and gri_data_precision_format_idc syntax elements are present. gri_game_engine_params_present_flag equal to 0 indicates that the gri_y_axis_up_flag, gri_right_hand_flag, gri_normalization_range_flag, and gri_data_precision_format_idc syntax elements are not present.

gri_world_to_cam_matrix_present_flag equal to 1 indicates that the gri_projection_coeff[ i ][ j ] syntax elements are present. gri_world_to_cam_matrix_present_flag equal to 0 indicates that the gri_projection_coeff[ i ][ j ] syntax  elements are not present.

gri_world_to_cam_matrix_present_flag equal to 1 indicates that the gri_world_to_cam_coeff[ i ][ j ] syntax elements are present. gri_world_to_cam_matrix_present_flag equal to 0 indicates that the gri_world_to_cam_coeff[ i ][ j ] syntax elements are not present.

gri_depth_params_present_flag equal to 1 indicates that the gri_disparity_flag, gri_near_plane, gri_far_plane, and gri_focal_length syntax elements are present. gri_depth_params_present_flag equal to 0 indicates that the gri_depth_inverse_flag, gri_near_plane, gri_far_plane, and gri_focal_length syntax elements are not present.

gri_y_axis_up_flag equal to 1 indicates that the graphics engine used to create of the source content has a vertical Y axis in its coordinate system. gri_y_axis_up_flag equal to 0 indicates that the graphics engine used to create of the source content has a vertical Z axis in its coordinate system.

When not present and PrevGriSeiPresentFlag equal to 0, the value of gri_y_axis_up_flag is inferred to be equal to 1.

When not present and PrevGriSeiPresentFlag equal to 1, the value of gri_y_axis_up_flag is inferred to be equal to the value of the corresponding syntax element in the previous GRI SEI message.

gri_right_handed_flag equal to 1 indicates that the the graphics engine used to create of the source content indicates that the graphics engine used to create of the source content uses a right handed coordinate system. gri_right_handed_flag equal to 1 indicates that the the graphics engine used to create of the source content indicates that the graphics engine used to create of the source content uses a left handed coordinate system.

When not present and PrevGriSeiPresentFlag equal to 0, the value of gri_right_handed_flag is inferred to be equal to 1.

When not present and PrevGriSeiPresentFlag equal to 1, the value of gri_ right_handed_flag is inferred to be equal to the value of the corresponding syntax element in the previous GRI SEI message.

gri_normalization_range_flag equal to 1 indicates that the graphics engine used to create of the source content uses normalized coordinates in range of -1 to 1 inclusive. gri_normalization_range_flag equal to 0 indicates that the graphics engine used to create of the source content uses normalized coordinates in range of 0 to 1 inclusive.

When not present and PrevGriSeiPresentFlag equal to 0, the value of gri_normalization_range_flag is inferred to be equal to 1.

When not present and PrevGriSeiPresentFlag equal to 1, the value of gri_normalization_range_flag is inferred to be equal to the value of the corresponding syntax element in the previous GRI SEI message.

gri_perspective_projection_flag equal to 1 indicates that the graphics engine used to create the source content uses projection projection. gri_perspective_projection_flag equal to 1 indicates that the graphics engine used to create the source content uses orthographic projection.

When not present and PrevGriSeiPresentFlag equal to 0, the value of gri_perspective_projection_flag is inferred to be equal to 1.

When not present and PrevGriSeiPresentFlag equal to 1, the value of gri_perspective_projection_flag is inferred to be equal to the value of the corresponding syntax element in the previous GRI SEI message.

gri_data_format_idc indicates the data format used to signal the gri_projection_coeff[ i ][ j ], gri_world_to_cam_coeff[ i ][ j ], gri_near_plane, gri_far_plane, and gri_focal_length  syntax elements, as specified in Table X.1.

The value of gri_data_format_idc shall be in the range of 0 to 1, inclusive. The values 2 and 3 are reserved for future use by ITU-T | ISO/IEC.

Table X.1 – Mapping of gri_data_format_idc

| gri_data_format_idc | DataFormatType | GriParamLen |
| --- | --- | --- |
| 0 | binary32 in ISO/IEC 60559:2020 | 32 |
| 1 | binary16 in ISO/IEC 60559:2020 | 16 |
| 2..3 | Reserved |  |

When not present and PrevGriSeiPresentFlag equal to 0, the value of gri_data_format_idc is inferred to be equal to 0.

When not present and PrevGriSeiPresentFlag equal to 1, the value of gri_data_format_idc is inferred to be equal to the value of the corresponding syntax element in the previous GRI SEI message.

gri_projection_coeff[ i ][ j ] indicates the (i, j)-th coefficient of a 4x4 projection matrix used by a game engine to render the source picture, in a floating point format of type DataFormatType, as specified in Table X.1. The length of the syntax element is GriParamLen bits, as determined by Table X.1.

When not present and PrevGriSeiPresentFlag equal to 0, the value of gri_projection_coeff[ i ][ j ] is inferred to be equal to (i = = j) ? 1 : 0.

When not present and PrevGriSeiPresentFlag equal to 1, the value of gri_projection_coeff[ i ][ j ] is inferred to be equal to the value of the corresponding syntax element in the previous GRI SEI message.

The variable GriProjectionCoeff[ i ][ j ] is a 2-dimensional array of size 4x4 derived as follows:

for (i = 0; i < 3; i++)

for (j = 0; j < 4; j++)

GriProjectionCoeff [ i ][ j ] = gri_projection_coeff[ i ][ j ]

if ( gri_perspective_projection_flag )

GriProjectionCoeff[ 3 ] = { 0, 0, -1, 0}

else

GriProjectionCoeff[ 3 ] = { 0, 0, 0, 1}

gri_world_to_cam_coeff[ i ][ j ] indicates the (i, j)-th coefficient of a 4x3 array that represents the first three rows of a 4x4 world to camera matrix used by a game engine to render the source picture, in a floating point format of type DataFormatType, as specified in Table X.1. The length of the syntax element is GriParamLen bits, as determined by Table X.2.

When not present and PrevGriSeiPresentFlag equal to 0, the value of gri_world_to_cam_coeff[ i ][ j ] is inferred to be equal to (i = = j) ? 1 : 0.

When not present and PrevGriSeiPresentFlag equal to 1, the value of gri_world_to_cam_coeff[ i ][ j ] is inferred to be equal to the value of the corresponding syntax element in the previous GRI SEI message.

The variable GriWorldToCamCoeff[ i ][ j ] is a 2-dimensional array of size 4x4 derived as follows:

for (i = 0; i < 3; i++)

for (j = 0; j < 4; j++)

GriWorldToCamCoeff[ i ][ j ] = gri_world_to_cam_coeff [ i ][ j ]

GriWorldToCamCoeff[ 3 ] = { 0, 0, 0, 1}

gri_near_plane specifies the the closest distance to the camera used for rendering the source content, as used in the depth mapping process described in subclause X.X.X.1. The length of the syntax element is GriParamLen bits.

When not present and PrevGriSeiPresentFlag equal to 0, the value of gri_near_plane is  inferred to be equal to 0.01.

When not present and PrevGriSeiPresentFlag equal to 1, the value of gri_near_plane is inferred to be equal to the value of the corresponding syntax element in the previous GRI SEI message.

gri_far_plane specifies the the farthest distance to the camera used for rendering the source content, as used in the depth mapping process described in subclause X.X.X.1. The length of the syntax element is GriParamLen bits.

When not present and PrevGriSeiPresentFlag equal to 0, the value of gri_far_plane is undefined.

When not present and PrevGriSeiPresentFlag equal to 1, the value of gri_far_plane is inferred to be equal to the value of the corresponding syntax element in the previous GRI SEI message.

gri_depth_inverse_flag equal to 1 specifies that each decoded luma sample value of an associated auxiliary picture of type AUX_DEPTH represents an inverse of Z value in the range of gri_far_plane to gri_near_plane, . gri_depth_inverse_flag equal to 0 specifies that each decoded luma sample value of an associated auxiliary picture of type AUX_DEPTH represents a Z value in the range of gri_near_plane to gri_far_plane.

When not present and PrevGriSeiPresentFlag equal to 0, the value of gri_depth_inverse_flag is inferred to be equal to 1.

When not present and PrevGriSeiPresentFlag equal to 1, the value of gri_depth_inverse_flag is inferred to be equal to the value of the corresponding syntax element in the previous GRI SEI message.

gri_focal_plane specifies the focal length of the camera used for rendering the source content. The length of the syntax element is GriParamLen bits.

When not present and PrevGriSeiPresentFlag equal to 0, the value of gri_focal_plane is undefined.

When not present and PrevGriSeiPresentFlag equal to 1, the value of gri_focal_plane is inferred to be equal to the value of the corresponding syntax element in the previous GRI SEI message.

X.X.X.1 Depth mapping process

This process is used to interpret the sample values of auxiliary pictures of type AUX_DEPTH associated with the GRI SEI message.

The value of depth Z associated with a luma sample value of an auxiliary picture of type AUX_DEPTH is represented in a normalized device coordinates with a range indicated by gri_normalization_range_flag and gri_depth_inverse_flag. The normalized device coordinate depth ZNDC is evaluated as:

ZNDC= aZ+c

In case when gri_normalization_range_flag is equal to 0:

a=2 ∙ gri_near_plane,   c=-1.

when gri_normalization_range_flag is equal to 1:

a=-2 ∙ gri_near_plane ∙ gri_far_planegri_far_plane-gri_near_plane,   c=gri_far_plane+gri_near_planegri_far_plane-gri_near_plane;

In case when gri_depth_inverse_flag is equal to 0:

ZbufEngine=zNDC+12;

when gri_depth_inverse_flag is equal to 1:

ZbufEngine=1-zNDC2

X.X.X.2 Coordinate system conversion process

This process is used to to convert the projection matrix and the world to camera matrix depending on the values of gri_y_axis_up_flag and gri_right_handed_flag.

If a further process utilizing the decoded bitstream is using a coordinate system that is different from a coordinate system indicated by gri_y_axis_up_flag and gri_right_handed_flag syntax elements in the VSEI message a coordinate system conversion process shall be invoked.

To convert projection matrix gri_projection_coeff[ i ][ j ] or world to camera matrix gri_world_to_cam_coeff[ i ][ j ] M from vertical Y axis (gri_y_axis_up_flag is equal to 1) to projection matrix gri_projection_coeff[ i ][ j ] or world to camera  matrix gri_world_to_cam_coeff[ i ][ j ]  M' to verical Z axis the following process is applied.

M'=M∙1000001001000001.

To convert projection matrix gri_projection_coeff[ i ][ j ] or world to camera matrix gri_world_to_cam_coeff[ i ][ j ] M from vertical Z axis (gri_y_axis_up_flag is equal to 0) to projection matrix gri_projection_coeff[ i ][ j ] or world to camera  matrix gri_world_to_cam_coeff[ i ][ j ]  M' to verical Y axis the following process is applied.

M'=M∙1000001001000001.

If further processing of the decoded picture is to be performed with a graphics engine with a left handed coordinate system and gri_right_handed_flag is equal to 1, or the graphics engine with a right handed coordingate system and gri_right_handed_flag is equal to 0,

To convert projection matrix gri_projection_coeff[ i ][ j ]  or world to camera matrix gri_world_to_cam_coeff[ i ][ j ] M from the right hand coordinate system (gri_right_handed_flag is equal to 1) to projection matrix gri_projection_coeff[ i ][ j ] or world to camera matrix gri_world_to_cam_coeff[ i ][ j ] M' to the left hand coordinate system the following process is applied.

M'=M∙-1001000000001001.

To convert projection matrix gri_projection_coeff[ i ][ j ]  or world to camera matrix gri_world_to_cam_coeff[ i ][ j ] M from the left hand coordinate system (gri_right_handed_flag is equal to 0) to projection matrix gri_projection_coeff[ i ][ j ] or world to camera matrix gri_world_to_cam_coeff[ i ][ j ] M' to the right hand coordinate system the following process is applied.

M'=M∙-1001000000001001.

# Example JSON file from JVET-AI0054

The header and first two entries contained in the JSON file attached to JVET-AI0054 are copied below.

Version 0.0 - projectionMatrix, worldToCameraMatrix, farClipPlane, nearClipPlane, focalLength

{"frame":0,"projectionMatrix":{"e00":1.0805524587631226,"e01":0.0,"e02":0.0,"e03":0.0,"e10":0.0,"e11":1.9209821224212647,"e12":0.0,"e13":0.0,"e20":0.0,"e21":0.0,"e22":-1.000400185585022,"e23":-0.6001200675964356,"e30":0.0,"e31":0.0,"e32":-1.0,"e33":0.0},"worldToCameraMatrix":{"e00":1.0,"e01":0.0,"e02":0.0,"e03":0.0,"e10":0.0,"e11":1.0,"e12":0.0,"e13":-35.0,"e20":0.0,"e21":0.0,"e22":-1.0,"e23":0.0,"e30":0.0,"e31":0.0,"e32":0.0,"e33":1.0},"farClipPlane":1500.0,"nearClipPlane":0.30000001192092898,"focalLength":23.05178451538086}

{"frame":1,"projectionMatrix":{"e00":1.0805524587631226,"e01":0.0,"e02":0.0,"e03":0.0,"e10":0.0,"e11":1.9209821224212647,"e12":0.0,"e13":0.0,"e20":0.0,"e21":0.0,"e22":-1.000400185585022,"e23":-0.6001200675964356,"e30":0.0,"e31":0.0,"e32":-1.0,"e33":0.0},"worldToCameraMatrix":{"e00":1.0,"e01":0.00014381069922819734,"e02":0.00014385205577127635,"e03":0.019688259810209276,"e10":-0.00014385205577127635,"e11":0.9999999403953552,"e12":0.0002876007347367704,"e13":-35.01235580444336,"e20":0.00014381069922819734,"e21":0.00028762139845639467,"e22":-0.9999999403953552,"e23":-0.018307488411664964,"e30":0.0,"e31":0.0,"e32":0.0,"e33":1.0},"farClipPlane":1500.0,"nearClipPlane":0.30000001192092898,"focalLength":23.05178451538086}

…

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).