| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0248-v1 |
| --- | --- |

| Title: | AHG9: Constituent Rectangles SEI extension to support 4:4:4 source content coded in bitstream with 4:2:0 chroma format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposa, |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela | Email: | {jill.boyce, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is proposed to extend the Constituent Rectangles (CR) SEI message to support coding of 4:4:4 chroma format source content in a bitstream coded with 4:2:0 or monochrome chroma format, by coding each of the 3 colour components of the 4:4:4 source content in a separate constituent rectangle. This allows 4:4:4 source content to be coded with HEVC or VVC single layer profiles that do not support 4:4:4, such as VVC Main 10 profile. In combination with related contribution JVET-AJ0245, coding of 4:4:4 source content is supported in multi-layer profiles that do not support 4:4:4, such as VVC Multilayer Main 10 profile.

# Proposal

The constituent rectangles (CR) SEI message in the TuC in JVET-AI2032 provides a flexible standardized method to describe multiple “rectangles” composited within a single coded picture, to address a variety of applications. Contribution JVET-AJ0245 proposes extending the CR SEI to apply to multiple layers in multi-layer bitstream.

JVET-AI0257 proposed a new SEI message to use multi-layer HEVC bitstreams to distribute colour components of 4:4:4 pictures across different layers within the HEVC bitstream.

This contribution proposes an alternative approach to support coding of 4:4:4 source content using HEVC or VVC profiles that don’t support 4:4:4, such as VVC Main 10 profile. It is proposed to extend the Constituent Rectangles (CR) SEI message to support compositing the 3 colour components of a 4:4:4 source picture as 3 constituent rectangles in a 4:2:0 or monochrome coded picture. This allows 4:4:4 source content to be coded with HEVC or VVC profiles that do not support 4:4:4 and do not support multiple layers, or in combination with contribution JVET-AJ0245, in multi-layer profiles that do not support 4:4:4.

To use this feature, 3 constituent rectangles containing the 3 colour components are assigned the same value of cr_rect_group_id[ i ]. It is possible to code more than one 4:4:4 source picture in a single coded 4:2:0 or monochrome picture with the CR SEI message by using different values of cr_rect_group_id[ i ] for each source picture.

The proposed changes to the CR SEI message are summarized as follows

- Add a 4:4:4 enable flag

- Add a flag per CR group to indicate that the CR group represents 4:4:4.

- Assign values of cr_rect_type_idc[ i ] to indicate Y/G, Cb/B, or Cr/R.

- Define a process to generate a 4:4:4 target picture from the rectangles representing the 3 colour components

- Optionally signal colour description parameters (colour primaries, transfer characteristics, matrix coefficients, full range flag) of the target picture

- Change cr_group_id_present_flag to cr_num_groups

- Change the length of the cr_group_id syntax element to Ceil(Log2(cr_num_groups))

It is also proposed to move the CR SEI message to the WD for VSEI.

Figure 1 shows an example coded picture with 3 constituent rectangles, with each rectangle containing an individual colour component, Y, Cb, or Cr in the luma component of the coded picture. Figure 2 shows the generated target picture as YUV 4:4:4.

The proposed approach can also be used to support multiple 4:4:4 target pictures, for example representing multiple views, by using the same cr_rect_group_id[ i ] value for all 3 colour components of the same view. Additional rectangle types may also be present for the same group, i.e, depth or alpha.

Figure 1. Coded picture using 3 constituent rectangles for the Y, U, and V components of a YUV 4:4:4 picture

Figure 2. Reconstructed target picture

When a flag indicates that a CR group represents 4:4:4 content, it is required for conformance that one constituent rectangle be present in that group for each of the 3 colour component types, and that all 3 of these rectangles have the same resolution.

A 4:4:4 target picture is generated for which the samples of the 3 rectangles are allocated to the 3 colour components of the target picture.

## Multi-layer support

JVET-AI0257 proposed using a multi-layer bitstream to carry separate colour components of 4:4:4 pictures in a two-layer bitstream, with the luma component in a base layer and the two chroma components in an enhancement layer.

This contribution can utilize a proposed multi-layer extension to the CR SEI message in JVET-AJ0245 to flexibly carry colour components in constituent rectangles in multiple layers. No additional changes are required beyond the combination of the modifications for 4:4:4 support and the modifications for multi-layer support.

# Proposed specification changes

## 4:4:4 support in single layer

The changes to the CR SEI syntax and semantics are highlighted in yellow.

| constituent_rectangles( payloadSize ) { | Descriptor |
| --- | --- |
| cr_num_rects_minus1 | u(12) |
| cr_rect_id_enabled__flag | u(1) |
| cr_associated_rect_id_enabled_flag | u(1) |
| cr_rect_group_id_present_flag | u(1) |
| cr_num_groups | ue(v) |
| if( cr_rect_id_present_flag ¦ ¦ cr_associated_rect_id_enabled_flag ¦ ¦ cr_rect_group_id_present_flag ) |  |
| cr_rect_id_len | u(4) |
| cr_rect_type_enabled_flag | u(1) |
| cr_444_enabled_flag | u(1) |
| if( cr_444_enabled_flag ) |  |
| for( i = 0; i < cr_num_groups; i++ ) { |  |
| cr_group_444_flag[ i ] | u(1) |
| if( cr_group_444_flag[ i ] ) { |  |
| cr_colour_description_present_flag[ i ] | u(1) |
| if( cr_colour_description_present_flag[ i ] ) { |  |
| cr_colour_primaries[ i ] | u(8) |
| cr_transfer_characteristics[ i ] | u(8) |
| cr_matrix_coeffs[ i ] | u(8) |
| cr_full_range_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| cr_rect_type_descriptions_enabled_flag | u(1) |
| cr_subpics_partitioning_flag | u(1) |
| if( !cr_subpics_partitioning_flag ) { | u(1) |
| cr_rect_same_size_flag | u(1) |
| if( cr_rect_same_size_flag ) { |  |
| cr_num_cols_minus1 | ue(v) |
| cr_num_rows_minus1 | ue(v) |
| } else { |  |
| cr_log2_unit_size | u(4) |
| cr_rect_size_len_minus1 | u(4) |
| } |  |
| } |  |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) { |  |
| if( cr_rect type_enabled_flag ) { |  |
| cr_rect_type_present_flag[ i ] | u(1) |
| if( cr_rect_type_present_flag[ i ] ) |  |
| cr_rect_type_idc[ i ] | u(8) |
| } |  |
| if( cr_rect type_idc[ i ] != 255 ) { |  |
| if( cr_rect id_enabled_flag ) { |  |
| cr_rect_type_present_flag[ i ] | u(1) |
| if( cr_rect id_present_flag ) |  |
| cr_rect_id[ i ] | u(v) |
| } |  |
| if( cr_associated_rect id_enabled_flag ) { |  |
| cr_associated_rect_id_present_flag[ i ] | u(1) |
| if ( cr_associated_rect_id_present_flag[ i ] ) |  |
| cr_associated_rect_id[ i ] | u(v) |
| } |  |
| if ( cr_rect_group_id_present_flag ) |  |
| if( cr_num_groups > 0 ) |  |
| cr_rect_group_id[ i ] | u(v) |
| if( cr_rect type_description_enabled_flag ) |  |
| cr_rect_type_description_present_flag[ i ] | u(1) |
| } |  |
| if( !cr_subpics_partitioning_flag && !cr_rects_same_size_flag ) { |  |
| cr_rect_top_left_in_units_x[ i ] | u(v) |
| cr_rect_top_left_in_units_y[ i ] | u(v) |
| cr_rect_width_in_units_minus1[ i ] | u(v) |
| cr_rect_height_in_units minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( cr_rect_type_descriptions_enabled_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| cr_bit_equal_to_zero /* equal to 0 */ | f(1) |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) |  |
| if( rect_type_description_present_flag[ i ] ) |  |
| cr_rect_type_description[ i ] | st(v) |
| } |  |
| } |  |

The constituent rectangles SEI message enables composition of multiple rectangles within a coded picture and provides information about the rectangles, including ID, group ID, type, text description, location, and size. It may provide information to enable formation of a 4:4:4 target picture.

cr_rect_group_id_present_flag equal to 1 specifies that cr_rect_group_id[ i ] syntax elements are present. cr_rect_group_present_flag equal to 0 specifies that_cr_rect_group_id[ i ] syntax elements are not present.

cr_num_group_ specifies the maximum number of rectangle groups described in the SEI message.

cr_444_enabled_flag equal to 1 indicates that the cr_group_444_flag[ i ] syntax element is present.  cr_444_enabled_flag equal to 0 indicates that the cr_group_444_flag[ i ] syntax element is not present.

cr_group_444_flag[ i ] equal to 1 indicates that an i-th 4:4:4 target picture is formed from three rectangles of the i-th rectangle group. cr_group_444_flag[ i ] equal to 0 indicates that a 4:4:4 target picture is not formed from the rectangles of the i-th rectangle group.

cr_colour_description_present_flag[ i ] equal to 1 indicates that the cr_colour_primaries[ i ], cr_transfer_characteristics[ i ], cr_matrix_coeffs[ i ] and cr_matrix_coeffs[ i ] syntax elements are present. cr_colour_description_present_flag[ i ] equal to 0 indicates that the cr_colour_primaries[ i ], cr_transfer_characteristics[ i ], cr_matrix_coeffs[ i ] and cr_matrix_coeffs[ i ] syntax elements are not present.

cr_colour_primaries[ i ] has the same semantics as specified in clause 7.3 for the vui_colour_primaries syntax element, but applied to the i-th target picture. When not present, the value of cr_colour_primaries[ i ] is inferred to be equal to the value of vui_colour_primaries.

cr_transfer_characteristics[ i ] has the same semantics as specified in clause 7.3 for the vui_transfer_characteristics syntax element, but applied to the i-th target picture. When not present, the value of cr_transfer_characteristics[ i ] is inferred to be equal to the value of vui_transfer_characteristics.

cr_matrix_coeffs[ i ] has the same semantics as specified in clause 7.3 for the vui_matrix_coeffs syntax element, but applied to the i-th target picture. When not present, the value of cr_matrix_coeffs[ i ] is inferred to be equal to the value of vui_matrix_coeffs.

cr_full_range_flag[ i ] has the same semantics as specified in clause 7.3 for the vui_full_range_flag syntax element, but applied to the i-th target picture. When not present, the value of cr_full_range_flag[ i ] is inferred to be equal to the value of vui_full_range_flag.

cr_rect_type_present_flag[ i ] equal to 1 specifies that the cr_rect_type_idc[ i ] syntax element is present in the SEI message. cr_rect_type_present_flag[ i ] equal to 0 specifies that the cr_rect_type_idc[ i ] syntax element is not present in the SEI message.

cr_rect_type_idc[ i ] indicates the constituent picture type of the i-th rectangle from Table X. When not present and i equal to 0, the value of cr_rect_type_idc[ i ] is inferred to be equal to 0. When not present and i greater than 0, cr_rect_type_idc[ i ] is inferred to be equal to cr_rect_type_idc[ i − 1 ].

Table X – Mapping of cr_rect_type_idc[ i ] to the type of constituent rectangle

| cr_rect_type_idc[ i ] | Name | Type of constituent rectangle |
| --- | --- | --- |
| 0 | CR_TEXTURE | Texture |
| 1 | CR_ALPHA | Alpha plane |
| 2 | CR_DEPTH | Depth picture |
| 3 | CR_OBJECT_MASK | Object mask |
| 4 | CR_Y | Y or G colour component |
| 5 | CR_CB | Cb or B colour component |
| 6 | CR_CR | Cr or R colour component |
| 47 .. 127 |  | Reserved |
| 128..254 |  | Unspecified |
| 255 | CR_EMPTY | Empty |

cr_rect_group_id[ i ] indicates the group ID of the i-th rectangle. The length of the syntax element is cr_rect_id_len Ceil( Log2( cr_num_groups)  bits. When not present, the value of cr_rect_group_id[ i ] is inferred to be equal to 0.

When cr_rect_type_idc[ i ] is in the range or 4 .. 6, it is a requirement of bitstream conformance that cr_group_444_flag[ cr_rect_group_id[ i ] ] is equal to 1.

When cr_group_444_flag[ j ] equal to 1, for j in the range of 0 .. cr_num_groups − 1, the following applies:

- It is a requirement of bitstream conformance that there shall be one value y in the range of 0 .. cr_num_rects_minus1, for which cr_rect_group_id[ y ] equals j and cr_rect_type_idc[ y ] equals 4,

- It is a requirement of bitstream conformance that there shall be one value of u in the range of 0 .. cr_num_rects_minus1, for which cr_rect_group_id[ u ] equals j and cr_rect_type_idc[ u ] equals 5

- It is a requirement of bitstream conformance that there shall be one value of v in the range of 0 .. cr_num_rects_minus1, for which cr_rect_group_id[ v ] equals j and cr_rect_type_idc[ v ] equals 6

- The values of cr_rect_width_in_units_minus1[ y ], cr_rect_width_in_units_minus1[ u ],  and cr_rect_width_in_units_minus1[ v ] shall be equal.

- The values of cr_rect_height_in_units_minus1[ y ], cr_rect_height_in_units_minus1[ u ],  and cr_rect_height _in_units_minus1[ v ] shall be equal.

- A j-th target picture is generated with width equal to cr_rect_width_in_units_minus1[ y ], height equal to  cr_rect_height_in_units_minus1[ y],  SubWidthC equal to 1, SubHeightC equal to 1, ChromaFormatIdc equal to 3, and with the luma samples set equal to the luma samples of the y-th rectangle, the Cb samples set equal to the luma samples of the u-th rectangle, and the Cr samples set equal to the luma samples of the v-th rectangle.

## Combination of 4:4:4 with multi-layer support from JVET-A0245

The syntax table below combines support for 4:4:4, with modifications highlighted in yellow, with the multi-layer support proposed in JVET-AJ0245, highlighted in cyan.

| constituent_rectangles( payloadSize ) { | Descriptor |
| --- | --- |
| cr_num_rects_minus1 | u(12) |
| cr_num_layers_minus1 | ue(v) |
| if( cr_num_layers_minus1 > 0 ) |  |
| for( lIdx = 0; lIdx <= cr_num_layers_minus1; lIdx ++ ) { |  |
| cr_layer_id[ lIdx ] | u(6) |
| if (lIdx < cr_num_layers_minus1 ) |  |
| cr_num_rects_in_layer_minus1[ lIdx ] | u(12) |
| } |  |
| cr_rect_id_enabled_flag | u(1) |
| cr_associated_rect_id_enabled_flag | u(1) |
| cr_rect_group_id_present_flag | u(1) |
| cr_num_groups | ue(v) |
| if( cr_rect_id_present_flag ¦ ¦ cr_associated_rect_id_enabled_flag ¦ ¦ cr_rect_group_id_present_flag ) |  |
| cr_rect_id_len | u(4) |
| cr_rect_type_enabled_flag | u(1) |
| cr_444_enabled_flag | u(1) |
| if( cr_444_enabled_flag ) |  |
| for( i = 0; i < cr_num_groups; i++ ) { |  |
| cr_group_444_flag[ i ] | u(1) |
| if( cr_group_444_flag[ i ] ) { |  |
| cr_colour_description_present_flag[ i ] | u(1) |
| if( cr_colour_description_present_flag[ i ] ) { |  |
| cr_colour_primaries[ i ] | u(8) |
| cr_transfer_characteristics[ i ] | u(8) |
| cr_matrix_coeffs[ i ] | u(8) |
| cr_full_range_flag[ i ] | u(1) |
| } |  |
| } |  |
| cr_rect_type_descriptions_enabled_flag | u(1) |
| for ( lIdx = 0; lIdx <= cr_num_layers_minus1; lIdx++ ) { |  |
| cr_subpics_partitioning_flag[ lIdx ] | u(1) |
| if( !cr_subpics_partitioning_flag[ lIdx ] ) { | u(1) |
| cr_rect_same_size_flag[ lIdx ] | u(1) |
| if( cr_rect_same_size_flag[ lIdx ] ) { |  |
| cr_num_cols_minus1[ lIdx ] | ue(v) |
| cr_num_rows_minus1[ lIdx ] | ue(v) |
| } |  |
| } else { |  |
| cr_log2_unit_size[ lIdx ] | u(4) |
| cr_rect_size_len_minus1[ lIdx ] | u(4) |
| } |  |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) { |  |
| for( crIdx = 0; crIdx <= cr_num_rects_in_layer_minus1[ lIdx ]; crIdx++, i++ ) |  |
| if( cr_rect type_enabled_flag ) { |  |
| cr_rect_type_present_flag[ i ] | u(1) |
| if( cr_rect_type_present_flag[ i ] ) |  |
| cr_rect_type_idc[ i ] | u(8) |
| } |  |
| if( cr_rect type_idc[ i ] != 255 ) { |  |
| if( cr_rect id_enabled_flag ) { |  |
| cr_rect_type_present_flag[ i ] | u(1) |
| if( cr_rect id_present_flag ) |  |
| cr_rect_id[ i ] | u(v) |
| } |  |
| if( cr_associated_rect id_enabled_flag ) { |  |
| cr_associated_rect_id_present_flag[ i ] | u(1) |
| if ( cr_associated_rect_id_present_flag[ i ] ) |  |
| cr_associated_rect_id[ i ] | u(v) |
| } |  |
| if ( cr_rect_group_id_present_flag ) |  |
| if( cr_num_groups > 0 ) |  |
| cr_rect_group_id[ i ] | u(v) |
| if( cr_rect type_description_enabled_flag ) |  |
| cr_rect_type_description_present_flag[ i ] | u(1) |
| } |  |
| if( !cr_subpics_partitioning_flag[ lIdx ] && !cr_rects_same_size_flag[ lIdx ] ) { |  |
| cr_rect_top_left_in_units_x[ lIdx ][ i ] | u(v) |
| cr_rect_top_left_in_units_y[ lIdx ][ i ] | u(v) |
| cr_rect_width_in_units_minus1[ lIdx ][ i ] | u(v) |
| cr_rect_height_in_units minus1[ lIdx ][ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( cr_rect_type_descriptions_enabled_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| cr_bit_equal_to_zero /* equal to 0 */ | f(1) |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) |  |
| if( rect_type_description_present_flag[ i ] ) |  |
| cr_rect_type_description[ i ] | st(v) |
| } |  |
| } |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).