| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ0249-v4 |
| --- | --- |

| Title: | EE2-2.20_2.21: Neural network-based intra prediction with DIMD mode derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Urban, T. Dumas, F. Galpin, E. François (Interdigital) S. Eadie, P. Garus, P. Nikitin, M. Coban, V. Seregin, M. Karczewicz (Qualcomm) | Email: thierry.dumas@interdigital.com seadie@qti.qualcomm.com |  |
| Source: | Interdigital, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results from the combined EE2 tests of 2.20 and 2.21. These EE2 tests both proposed neural network-based intra prediction; given their similarities, the proponents have converged to a single harmonized design, as reported in this contribution. This combined test replaces the equivalent intra mode predictor in the neural network from 2.20, with DIMD derivation. The following results are reported:

AI:  -0.58%, -0.40%, -0.40% BD-rate gains for Y, U and V, respectively, with 101.6% encoder and 109.8% decoder run times.

RA:  -0.22%, -0.21%, -0.17% BD-rate gains for Y, U and V, respectively, with 99.0% encoder and 99.8% decoder run times.

# Proposed Method

The neural network-based intra prediction algorithm and models are described in JVET-AH2019 [1] and JVET-AH0156 [2].

- SADL for inference

- 16-bit integer for model weights and intermediate results, using 32-bit integer for internal computation

- 6 models depending on the block size (4x32 from JVET-AH0156 has been removed)

- sparse weights enforced at training time for complexity reduction. SADL supports a simple sparse matrix storage (compressed row method) and sparse matrix multiplication algorithm, exploiting the matrices sparsity. Sparsity constraints have been strengthened compared to JVET-AH0156.

Additionally, only the intra prediction output is retained, and the equivalent intra mode is derived using DIMD. The architectures of the different models are detailed in Figure 1 and Table 1. The model complexities are summarized in Table 2. Similarly to JVET-AI0050, an alternative IPM is used for LFNST/NSPT only for luma CBs of size larger than 64. In addition to the 1st DIMD IPM, PLANAR is used as an additional IPM candidate. A CU-level flag indicating the IPM index is signaled only for luma CBs of size larger than 64.

Figure 1: Prediction of the current w×h luma CB from its context of reference samples via the neural network fh, w . , θh,w, parametrized by θh,w, belonging to the proposed intra prediction mode. Here, w=8 and h=4. na, nl, “preprocessing”, and “postprocessing” are specified in [1]. “preprocessing” and “postprocessing” correspond to data layout changes.

Table 1: Architectures of the different models.

| architecture component | value |
| --- | --- |
| Number of hidden layers | 16x16: 3 layers. 4x4, 8x4, 16x4, 8x8, 16x8: 2 layers. |
| Number of neurons per hidden layer | 4x4, 8x4, 16x16: 1216 for the last hidden layer, 640 for the other hidden layers. 16x4, 8x8, 16x8: 1216. |
| Ratio of zero weights | 95% in the hidden layers and 50% in the output layer. |
| Activation in hidden layer | LeakyReLU |
| Bias | No |

Table 2: Model complexity for the different models.

| Model | Parameter non-zero (k) | MACs/pixel | Memory Footprint (MB) |
| --- | --- | --- | --- |
| 4x4 | 50.823 | 3176 | 0.195 |
| 8x4 | 61.886 | 1934 | 0.224 |
| 16x4 | 123.420 | 1928 | 0.450 |
| 8x8 | 132.260 | 2066 | 0.472 |
| 16x8 | 178.754 | 1396 | 0.590 |
| 16x16 | 215.376 | 841 | 0.596 |

Number of parameters for the whole intra prediction mode: 762519.

## Analysis of the latency

For the model defining the worst case in latency, i.e. the model predicting blocks of size 16x16, the pipeline for predicting a 16x16 luma CB comprises three sparse vector-matrix multiplications and a dense vector-matrix multiplication. These vector-matrix multiplications are interleaved with three LeakyReLU activations. No addition with bias exists in this prediction pipeline.

# Experiment results

The simulations of the proposed intra prediction mode are run following the JVET CTC [3] and reported in Table 3, Table 4, and Table 5.

Table 3: Proposed intra prediction mode over ECM-14.0 in All Intra configuration.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.99% | -0.84% | -0.87% | 102.7% | 108.6% | 100.7% | 100.7% |
| Class A2 | -0.40% | -0.31% | -0.30% | 102.7% | 108.1% | 102.6% | 100.9% |
| Class B | -0.45% | -0.25% | -0.25% | 102.4% | 109.8% | 101.1% | 100.6% |
| Class C | -0.47% | -0.43% | -0.38% | 99.2% | 112.7% | 101.5% | 100.9% |
| Class E | -0.74% | -0.27% | -0.30% | 101.1% | 108.7% | 99.3% | 100.8% |
| Overall | -0.58% | -0.40% | -0.40% | 101.6% | 109.8% | 101.1% | 100.8% |
| Class D | -0.55% | -0.31% | -0.77% | 99.8% | 114.1% | 100.5% | 100.9% |
| Class F | -0.28% | -0.34% | -0.31% | 100.6% | 106.2% | 97.4% | 100.7% |

Table 4: Proposed intra prediction mode over ECM-14.0 in Random Access configuration.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.33% | -0.51% | -0.52% | 98.8% | 100.0% | 100.3% | 100.8% |
| Class A2 | -0.13% | -0.14% | -0.14% | 99.3% | 99.3% | 100.3% | 99.6% |
| Class B | -0.20% | -0.06% | -0.06% | 99.1% | 99.8% | 100.2% | 100.5% |
| Class C | -0.21% | -0.21% | -0.07% | 98.8% | 100.0% | 100.4% | 100.7% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.22% | -0.21% | -0.17% | 99.0% | 99.8% | 100.3% | 100.4% |
| Class D | -0.21% | -0.27% | -0.62% | 98.5% | 100.2% | 101.0% | 100.8% |
| Class F | -0.01% | -0.41% | -0.10% | 99.0% | 99.8% | 100.2% | 100.5% |

Table 5: Proposed intra prediction mode over ECM-14.0 in Low Delay B configuration.

|  |  | Low delay B Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over ECM-14.0 |  |  |  |  |  |  |  |  |
|  | Y |  |  | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.04% |  |  | 0.28% | -0.30% | 101.2% | 100.2% | 100.3% | 100.3% |
| Class C | -0.03% |  |  | 0.57% | -0.17% | 100.4% | 100.6% | 100.5% | 100.6% |
| Class E | -0.11% |  |  | -0.03% | -0.43% | 99.9% | 100.6% | 101.5% | 100.5% |
| Overall | -0.05% |  |  | 0.30% | -0.29% | 100.6% | 100.4% | 100.7% | 100.5% |
| Class D | -0.01% |  |  | 0.02% | 0.64% | 99.7% | 99.9% | 100.6% | 100.8% |
| Class F | 0.15% |  |  | -0.21% | -0.27% | 92.0% | 99.7% | 100.6% | 100.5% |

# Conclusion

Given the trade-off between compression gains, encoding runtime, and decoding runtime, it is suggested to adopt the proposed intra prediction mode in the next version of ECM.

# Cited References

- JVET-AH2019, “Algorithm description for neural network-based video coding (NNVC-9.1)”, F. Galpin, Y. Li, D. Rusanovskyy, J. Ström, L. Wang. 34th Meeting, Rennes, FR, 17–24 April 2024.

- JVET-AH0156, “AhG12: neural network-based intra prediction”, F. Urban, T. Dumas, F. Galpin, E. François, 34th Meeting, Rennes, FR, 17–24 April 2024.

- JVET-AF2017, “Common test conditions and evaluation procedures for enhanced compression tool testing”, M. Karczewicz, Y. Ye, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).