| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ0249-v6 |
| --- | --- |

| Title: | EE2-2.20_2.21: Neural network-based intra prediction with DIMD mode derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Dumas, F. Urban, F. Galpin, E. François (Interdigital) S. Eadie, P. Garus, P. Nikitin, M. Coban, V. Seregin, M. Karczewicz (Qualcomm) | Email: thierry.dumas@interdigital.com seadie@qti.qualcomm.com |  |
| Source: | Interdigital, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-2.20 and EE2-2.21 tests, fused in a common solution. These EE2 tests propose neural network-based intra prediction. Given their similarities, the proponents have converged to a single harmonized design. The equivalent intra mode predictor (initially derived by neural network in test EE2-2.21) is obtained by DIMD (as proposed by test EE2-2.20), similarly to equivalent intra mode DIMD derivation used in Intra TMP, EIP. The network architecture was proposed in test EE2-2.21. The following results are reported.

AI:  -0.58%, -0.40%, -0.40% (Y / Cb / Cr), 101.6% EncT, 109.8% DecT.

RA:  -0.22%, -0.21%, -0.17% (Y / Cb / Cr), 99.0% EncT, 99.8% DecT.

# Proposed Method

The proposed neural network-based intra prediction algorithm and models build upon the neural network-based intra prediction algorithm and models described in JVET-AI2019 [1]. Notably,

- SADL for inference.

- 16-bit integer for model weights and intermediate results, using 32-bit integer for internal computation.

- sparse weights enforced at training time for complexity reduction. SADL supports a simple sparse matrix storage (compressed row method) and sparse matrix multiplication algorithm, exploiting the matrices sparsity.

To reduce the complexity of the neural network-based intra prediction mode both in terms of MACs/pixel and decoding runtime, the proposed intra prediction mode differs from [1] in two aspects.

- The sparsity constraints in the weight matrices have been strengthened compared to [1].

- Only the intra prediction output is retained, and the equivalent intra mode is derived by applying DIMD to the block predicted by the neural network-based intra prediction mode, as in TMP, EIP or MIP (as proposed in EE2-2.20).

To reduce the number of parameters compared to [1], 6 models are used depending on the block size, each predicting a block size in 4×4, 8×4, 16×4, 8×8, 16×8, 16×16 (as proposed in test EE2-2.21). For block sizes non natively supported by a model, similarly to MIP in VVC, the reference samples are transposed and/or down sampled (see Table 5).

The architectures of the models are detailed in Figure 2 to Figure 4. In these figures, N is the number of reference samples, and WH is the number of samples to predict (Figure 1). The number of non-zero weights for each weight matrix is displayed. A sparse vector-matrix multiplication processes chunks of 8 non-zero coefficients. The model complexities are summarized in Table 1. The LeakyReLU used is defined as a piecewise-linear function:

LeakyReLUx=x, x≥0x≫4, x<0

Similarly to JVET-AI0050, an alternative IPM is used for LFNST/NSPT for luma CBs larger than 64 luma samples. In addition to the 1st DIMD IPM, PLANAR is used as an additional IPM candidate. A CU-level flag indicating the IPM index is signaled for luma CBs larger than 64 luma samples.

Figure 1: N reference samples to predict a given W×H luma CB. If H≥8 && W≥8, t = 8; else t = 4.

Figure 2: 4x4 (N = 80) and 8x4 (N = 112) models are sequential matrix multiplications and LeakyReLUs (piecewise-linear functions).

Figure 3: 16x4 (N = 176), 8x8 (N = 320), and 16x8 (N = 448) models are sequential matrix multiplications and LeakyReLUs (piecewise-linear functions).

Figure 4: 16x16 model is sequential matrix multiplications and LeakyReLUs (piecewise-linear functions).

Table 1: Model complexity for each model.

| Model | Parameter non-zero (k) | MACs/pixel | Memory Footprint (MB) |
| --- | --- | --- | --- |
| 4x4 | 50.823 | 3176 | 0.195 |
| 8x4 | 61.886 | 1934 | 0.224 |
| 16x4 | 123.420 | 1928 | 0.450 |
| 8x8 | 132.260 | 2066 | 0.472 |
| 16x8 | 178.754 | 1396 | 0.590 |
| 16x16 | 215.376 | 841 | 0.596 |

Number of parameters for the whole intra prediction mode: 762519.

Note that, for block size 16x16, one additional sparse matrix multiplication is used compared to the cases in Figure 2 and Figure 3. However, this still corresponds to the lowest complexity case in MACs/pixel, as shown by Table 1.

Based on Table 1, compared to JVET-AI0201, in this fused method, the worst-case MACs/pixel drops from 4837 to 3176 and the number of parameters decreases from 1.33M to 0.76M. Coding efficiency is unchanged.

# Experiment results

Proposed intra prediction mode has been run following the JVET CTC [3]. Performances are reported in Table 2, Table 3, and Table 4.

Table 2: Proposed intra prediction mode over ECM-14.0 in All Intra configuration.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.99% | -0.84% | -0.87% | 102.7% | 108.6% | 100.7% | 100.7% |
| Class A2 | -0.40% | -0.31% | -0.30% | 102.7% | 108.1% | 102.6% | 100.9% |
| Class B | -0.45% | -0.25% | -0.25% | 102.4% | 109.8% | 101.1% | 100.6% |
| Class C | -0.47% | -0.43% | -0.38% | 99.2% | 112.7% | 101.5% | 100.9% |
| Class E | -0.74% | -0.27% | -0.30% | 101.1% | 108.7% | 99.3% | 100.8% |
| Overall | -0.58% | -0.40% | -0.40% | 101.6% | 109.8% | 101.1% | 100.8% |
| Class D | -0.55% | -0.31% | -0.77% | 99.8% | 114.1% | 100.5% | 100.9% |
| Class F | -0.28% | -0.34% | -0.31% | 100.6% | 106.2% | 97.4% | 100.7% |

Table 3: Proposed intra prediction mode over ECM-14.0 in Random Access configuration.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.33% | -0.51% | -0.52% | 98.8% | 100.0% | 100.3% | 100.8% |
| Class A2 | -0.13% | -0.14% | -0.14% | 99.3% | 99.3% | 100.3% | 99.6% |
| Class B | -0.20% | -0.06% | -0.06% | 99.1% | 99.8% | 100.2% | 100.5% |
| Class C | -0.21% | -0.21% | -0.07% | 98.8% | 100.0% | 100.4% | 100.7% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.22% | -0.21% | -0.17% | 99.0% | 99.8% | 100.3% | 100.4% |
| Class D | -0.21% | -0.27% | -0.62% | 98.5% | 100.2% | 101.0% | 100.8% |
| Class F | -0.01% | -0.41% | -0.10% | 99.0% | 99.8% | 100.2% | 100.5% |

Table 4: Proposed intra prediction mode over ECM-14.0 in Low Delay B configuration.

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | 0.28% | -0.30% | 101.2% | 100.2% | 100.3% | 100.3% |
| Class C | -0.03% | 0.57% | -0.17% | 100.4% | 100.6% | 100.5% | 100.6% |
| Class E | -0.11% | -0.03% | -0.43% | 99.9% | 100.6% | 101.5% | 100.5% |
| Overall | -0.05% | 0.30% | -0.29% | 100.6% | 100.4% | 100.7% | 100.5% |
| Class D | -0.01% | 0.02% | 0.64% | 99.7% | 99.9% | 100.6% | 100.8% |
| Class F | 0.15% | -0.21% | -0.27% | 92.0% | 99.7% | 100.6% | 100.5% |

# Additional experiments

## Contribution of each model to the compression gains

Table 5 presents per-model tool-on and tool-off results in AI configuration for CTC.

Table 5: per-model tool on/tool off results.

| removed model predicting block size W×H | 4×4 | 8×4 | 16×4 | 8×8 | 16×8 | 16×16 |
| --- | --- | --- | --- | --- | --- | --- |
| block sizes no longer supported | 4×4 | 8×4, 4×8 | 16×4, 4×16, 32×4, 4×32 | 8×8 | 16×8, 8×16, 32×8, 8×32 | 16×16, 32×16, 16×32, 32×32, 64×64 |
| Tool on Y BD-rate | -0.54% | -0.50% | -0.47% | -0.51% | -0.41% | -0.37% |
| Change in Y BD-rate from removing model | +0.04% | +0.08% | +0.11% | +0.07% | +0.17% | +0.21% |

## Selection rate of the proposed intra prediction mode

The usage frequency of the NN-based intra prediction mode for each block size is shown in Figure 5.

Figure 5: NN-intra and non-NN intra prediction usage frequencies in luma for each block size where NN-intra may apply (AI CTC; ISP is ignored).

Figure 5 reveals that the frequency of selection of the proposed intra prediction mode varies very little over all block sizes.

# SADL Integration

Small Ad-hoc Deep Learning (SADL) library is a pure C++, header-only library licensed under the BSD-3-Clause with ~10k lines of code that implements common neural-network operations (e.g. matrix multiplication, convolution, piecewise-linear). Since it is a header-only library, it is integrated into ECM’s CMake build system just by including the header directory; no additional libraries nor software dependencies must be installed nor configured. While SADL provides both floating-point and int16 fixed-point implementations, this proposal exclusively uses int16 fixed-point computation.

# Conclusion

NN intra prediction is proposed for luma blocks in ECM. Luma coding gain is -0.58% for AI and -0.22% for RA. Runtime increase over ECM is claimed to be reasonable.

It is suggested to adopt proposed intra prediction mode in next version of ECM.

# Cited References

- JVET-AI2019, “Description of algorithms version 8 and software version 10 in neural network-based video coding (NNVC)”, F. Galpin, Y. Li, D. Rusanovskyy, J. Ström, L. Wang. 35th Meeting, Sapporo, JP, 10–19 July 2024.

- JVET-AF2017, “Common test conditions and evaluation procedures for enhanced compression tool testing”, M. Karczewicz, Y. Ye, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).