| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0255-v4 |
| --- | --- |

| Title: | AHG12: Multiple transform set selection for intra LFNST/NSPT with subsampled DIMD for transform set selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban, Marta Karczewicz, Pavel Nikitin, Patrick Garus, Bappaditya Ray, Vadim Seregin (Qualcomm) Fan Wang, Yue Yu, Haoping Yu, Dong Wang (OPPO) Charles Bonnineau, Karam Naser, Saurabh Puri, Fabrice Le Léannec (InterDigital) Lei Zhao, Kai Zhang, Li Zhang (Bytedance) | Tel: Email: | mcoban@qti.qualcomm.com martak@qti.qualcomm.com wangfan6@oppo.com firstname.lastname@interdigital.com zhaolei.0324@bytedance.com |
| Source: | Qualcomm Inc., OPPO, Interdigital, Bytedance Inc. |  |  |

_____________________________

# Abstract

Multiple LFNST/NSPT transform sets are used for DIMD, TIMD, SGPM, MIP, EIP, and IntraTMP modes.   In this contribution DIMD method for derivation of intra mode for transform set selection operates on subsampled neighboring or prediction block samples. The number of transforms in secondary set is configurable according to the block size.

On top of ECM-14.0, simulation results are reported as follows:

AI {-0.14% Y, 0.08% U, 0.03% V, 100.9% EncT, 100.2% DecT}.

# Proposal

Multiple transform set selection (MTSS) for intra LFNST/NSPT was proposed in JVET-AH0056. Secondary IPM for selection of LFNST/NSPT transform kernels for MIP, EIP, SGPM and IntraTMP modes is derived from second highest HoG on the prediction block or neighboring blocks. In order to reduce complexity, prediction or neighboring blocks are subsampled for derivation of intra mode for transform selection. In addition, restrictions on the block size as well as number of transform candidates in the second set are imposed.

The derivation methods for transform sets are summarized below [1]:

|  | IPM to derive the 1st transform set | IPM to derive the 2nd transform set |
| --- | --- | --- |
| DIMD, OBIC, TIMD | The 1st IPM used in current mode | The 2nd IPM used in current mode |
| SGPM, MIP, EIP, ITMP | The 1st IPM derived by HoG of prediction | The 2nd IPM derived by HoG of prediction |

The 1st and 2nd candidate should have different transform sets or different transpose type. If the 2nd candidate is not yet decided by the above process, it is derived by the HoG of neighboring reconstructed pixels.

MTSS can be applied to CUs with number of pixels equal or larger than 128. For CUs less than 256 pixels only first 2 secondary set candidates can be used, for larger CUs all 3 candidates can be used.

# Experimental results

The performance of proposed scheme was evaluated using the common test conditions defined in [2].

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.21% | 0.05% | -0.20% | 101.5% | 99.6% | 99.5% |
| Class A2 | -0.11% | 0.06% | -0.04% | 100.1% | 99.4% | 100.2% |
| Class B | -0.12% | -0.04% | 0.07% | 100.9% | 100.3% | 99.9% |
| Class C | -0.11% | 0.17% | 0.11% | 100.2% | 100.6% | 100.7% |
| Class E | -0.17% | 0.18% | 0.17% | 101.9% | 100.7% | 100.3% |
| Overall | -0.14% | 0.08% | 0.03% | 100.9% | 100.2% | 100.1% |
| Class D | -0.07% | 0.09% | 0.02% | 99.1% | 100.4% | 100.6% |
| Class F | -0.08% | 0.17% | 0.16% | 97.9% | 98.9% | 100.4% |
| Class TGM | 0.04% | 0.02% | -0.03% | 101.4% | 100.8% | 100.3% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | -0.06% | -0.02% | 0.04% | 99.4% | 99.4% | 100.1% |
| Class C | -0.03% | -0.08% | 0.08% | 100.4% | 100.3% | 100.1% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.02% | -0.28% | -0.16% | 99.1% | 99.0% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# References

- F. Wang et. al, “EE2-4.4a: Multiple transform set selection for intra LFNST/NSPT without complexity reduction”, JVET-AI0163, 35th Meeting, July 2024.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, 25th Meeting, by teleconference, 12–21 January 2022.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

OPPO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).