| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0256-v1 |
| --- | --- |

| Title: | Recent Developments in BVC: Optimizing the H.266/VVC Encoder |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hongbin Liu1, Yuwen He2, Siping Tao2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | +1-858-999-7093 lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents recent developments of the optimized H.266/VVC encoder, BVC, developed by Bytedance’s Advanced Video Group. BVC has been further optimized for various use cases, including live streaming, offline transcoding, real-time communication, and image coding. In its updated version, BVC v2 introduces several new features, improvements, bugfixes and cleanups, enhancing both coding performance and stability, enabling its successful adoption in real-world applications. When compared to x265 (v4.0), HM-18.0 and AOM-AV1 (v3.10.0), the corresponding overall PSNR average BD-rate gain and speedup factors of BVC v2 are summarized as follows (except for those results explicitly identified as for HD+UHD classes of the CTC, all other results are for CTC all classes):

- Live streaming use case

  - Compared to HM RA: 17.31% BD-rate saving, 760x speedup; 20.91% BD-rate saving, 847x speedup (for HD+UHD)

  - Compared to x265 medium: 51.78% BD-rate saving, 1.1x speedup; 53.45% BD-rate saving, 1.0x speedup (for HD+UHD)

  - Compared to AOM-AV1 good mode (CPU-used=6): 45.34% BD-rate saving, 3.8x speedup; 44.07% BD-rate saving, 3.8x speedup (for HD+UHD)

- Offline transcoding use case

  - Compared to HM RA: 42.19% BD-rate saving, 36.7x speedup; 43.61% BD-rate saving, 43.5x speedup (for HD+UHD)

  - Compared to x265 placebo: 50.05% BD-rate saving, 1.5x speedup; 51.23% BD-rate saving, 1.4x speedup (for HD+UHD)

  - Compared to AOM-AV1 (CPU-used=3): 24.95% BD-rate saving, 1.6x speedup; 26.85% BD-rate saving, 1.7x speedup (for HD+UHD)

- Real-time communication use case

  - Compared to HM LDP: 0.75% BD-rate saving, 194x speedup

  - Compared to x265 superfast: 47.05% BD-rate saving, 1.2x speedup

  - Compared to AOM-AV1 real-time mode (CPU-used=6): 22.57% BD-rate saving, 1.3x speedup

- Image coding use case

  - Compared to HM AI: 22.75% BD-rate saving, 3.5x speedup

  - Compared to x265 placebo: 25.49% BD-rate saving, 2.6x speedup

  - Compared to AOM-AV1 (CPU-used=3): 14.04% BD-rate saving, 1.7x speedup

Introduction

The state-of-the-art video compression standard H.266/VVC [1] was finalized in July 2020. It has been demonstrated that 50% bitrate saving could be achieved compared to its predecessor, H.265/HEVC. Some VVC codec products have been developed due to its high coding efficiency [2]. To promote widespread market adoption of H.266/VVC for various use cases such as live streaming, offline transcoding, real-time communication and image coding, Bytedance developed a software VVC codec, namely BVC [3][4][5]. Some highlights of recent progresses of the BVC v2 encoder are listed as follows:

- New features and improvements

  - More presets targeting at different complexities and use cases

  - AI-based optimization algorithms

  - Rate-distortion-perception design for visual quality improvement

  - Tile based coding

  - Complexity-adaptive real-time coding

  - Decoding-complexity-aware encoding optimization

  - Further improvements of existing tools and introduction of new non-normative coding tools, e.g., RPR, ALF etc.

  - Integration and optimization of missing coding tools in the previous BVC version

  - SIMD/parallel coding optimization

  - Encoder optimization for the ARM platform

- Cleanups and bug fixes

Simulation Results

The latest reference software x265 (v4.0) [6], HM-18.0 [6] and AOM-AV1 (v3.10.0) [8] were used as benchmarks, and simulations were conducted following the common test conditions (CTC) [9], with exceptions listed in the table below to account for real-world use cases. The term 'AV1' in Table 1 represents the AOM-AV1 (v3.10.0). It is however noted that the BVC encoder is not targeting at optimizing coding performance for the common test sequences defined by the video groups.

Table 1: Differences in test conditions compared to CTC

|  | Live streaming | Offline Transcoding | RTC | Image Coding |
| --- | --- | --- | --- | --- |
| Configuration | RA | RA | LDP | AI |
| Threads | BVC/x265/AV1: 16; HM: 1 |  | 1 | BVC/x265/AV1: 16; HM: 1 |
| Bitdepth | 8 |  |  |  |
| Intra period | CTC | 256 | CTC | 1 |
| Preset | BVC: Live-medium/slow x265: medium AV1: CPU-used=6 | BVC: OT-medium/slow x265: placebo AV1: CPU-used=3 | BVC: RTC-veryfast/fast x265: superfast AV1: CPU-used=6 | BVC: Image-fast/medium/slow x265: placebo AV1: CPU-used=3 |
| Rate control | BVC vs HM: CQP [22, 27, 32, 37] | HM: CQP [22, 27, 32, 37] | CQP: BVC/HM: [22, 27, 32, 37] x265: [24, 28, 33, 37] AV1: [26, 35, 44, 54] | HM: CQP [22, 27, 32, 37] |
|  | BVC vs x265&AV1: CBR (Mbits/s) UHD: {20, 15, 10, 5} 1080p: {10, 7, 4, 2} class C: {5, 2, 1, 0.5} | BVC: CRF [22, 27, 32, 37] X265: CRF [24, 29, 34, 39] AV1: CRF [32, 42, 54, 63] |  | BVC: CRF [22, 27, 32, 37] X265: CRF [18, 23, 28, 33] AV1: CRF [22, 32, 42, 52] |
| Other key parameters | x265: --tune psnr |  |  |  |
|  | AV1: -q good --passes=1 |  | AV1: -q --rt --passes=1 --lag-in-frames=0 | AV1: -q good --passes=1 |

In the simulation tests, three commonly used evaluation metrics are applied: PSNR, SSIM and VMAF. When calculating the overall average PSNR/SSIM BD-rate savings, BD-rate savings of the Y, U and V component are calculated with Equation (1) and (2), respectively. PSNR_YUV is a weighted average with weighting factors 6, 1 and 1 for the PSNR of Y, U and V component, respectively. Similarly, SSIM_YUV is a weighted average with SSIM of the Y, U and V components.

PSNR_YUV=186*PSNR_Y+PSNR_U+PSNR_V           (1)

SSIM_YUV=186*SSIM_Y+SSIM_U+SSIM_V           (2)

Live streaming use case

The BVC encoder's performance for the live streaming use case was evaluated by comparing to HM RA, x265 in CBR, and AOM-AV1 in CBR. The results are listed in Table 2 to Table 4, respectively. BVC encoder in the live steaming use case achieves 17.31%/21.16%/20.16% (PSNR YUV/SSIM YUV/VMAF) BD-rate saving with 760x speedup compared to HM-18.0 CQP with RA configuration, and 51.78%/48.70%/48.08% and 45.34%/46.85%/38.16% BD-rate saving with 1.0x and 3.8x speedup compared to x265 and AOM-AV1 in CBR mode, respectively.

Table 2: BVC Live-slow preset compared to HM RA in CQP

|  | PSNR BD-Rate |  |  |  | SSIM BD-Rate |  |  |  | VMAF | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |  |  |
| Class A1 | -20.03% | -35.52% | -39.70% | -24.56% | -22.77% | -50.11% | -49.52% | -27.07% | -23.95% | 892 |
| Class A2 | -18.36% | -31.03% | -28.23% | -21.37% | -22.38% | -38.61% | -37.18% | -25.03% | -27.74% | 756 |
| Class B | -12.34% | -41.96% | -40.55% | -18.45% | -16.90% | -49.10% | -49.40% | -21.85% | -21.38% | 884 |
| Class C | -3.53% | -23.80% | -22.44% | -7.39% | -9.71% | -29.41% | -29.91% | -12.95% | -10.11% | 593 |
| Overall | -12.73% | -33.64% | -33.08% | -17.31% | -17.25% | -41.95% | -41.78% | -21.16% | -20.16% | 760 |
| UHD+HD Average | -16.08% | -37.22% | -36.96% | -20.91% | -20.00% | -46.51% | -46.10% | -24.14% | -23.81% | 847 |

Table 3: BVC Live-medium preset compared to x265 medium in CBR

|  | PSNR BD-Rate |  |  |  | SSIM BD-Rate |  |  |  | VMAF | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |  |  |
| Class A1 | -51.14% | -57.96% | -63.21% | -53.52% | -46.36% | -63.55% | -65.54% | -49.57% | -44.06% | 1.2 |
| Class A2 | -56.40% | -62.84% | -58.39% | -57.17% | -50.08% | -60.54% | -54.69% | -51.56% | -49.73% | 1.0 |
| Class B | -49.35% | -57.90% | -58.97% | -51.18% | -48.07% | -58.07% | -59.84% | -49.37% | -51.42% | 1.0 |
| Class C | -45.11% | -53.34% | -56.13% | -47.17% | -43.29% | -53.75% | -57.49% | -45.06% | -45.66% | 1.4 |
| Overall | -49.99% | -57.69% | -58.95% | -51.78% | -46.86% | -58.51% | -59.32% | -48.70% | -48.08% | 1.1 |
| UHD+HD Average | -51.76% | -59.26% | -59.97% | -53.45% | -48.15% | -60.24% | -59.99% | -50.02% | -48.95% | 1.0 |

Table 4: BVC Live-medium preset compared to AOM-AV1 in CBR

|  | PSNR BD-Rate |  |  |  | SSIM BD-Rate |  |  |  | VMAF | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |  |  |
| Class A1 | -32.23% | -42.26% | -44.79% | -35.59% | -34.21% | -49.07% | -49.09% | -36.57% | -25.17% | 4.3 |
| Class A2 | -40.19% | -58.63% | -55.09% | -43.67% | -39.67% | -59.87% | -56.25% | -43.95% | -35.16% | 3.7 |
| Class B | -47.15% | -58.00% | -57.50% | -49.40% | -48.55% | -59.26% | -58.67% | -49.94% | -42.65% | 3.6 |
| Class C | -45.84% | -58.87% | -59.25% | -48.82% | -50.66% | -62.69% | -63.70% | -52.88% | -44.55% | 3.8 |
| Overall | -42.42% | -55.21% | -54.95% | -45.34% | -44.47% | -58.26% | -57.61% | -46.85% | -38.16% | 3.8 |
| UHD+HD Average | -41.18% | -53.88% | -53.38% | -44.07% | -42.22% | -56.65% | -55.40% | -44.66% | -35.84% | 3.8 |

Offline transcoding use case

The BVC encoder's performance at the OT-medium/slow presets for the offline transcoding use case were evaluated. The results compared to HM, x265 and AOM-AV1 in RA configuration are presented Table 5 to Table 7, respectively. When compared to HM, BVC with the OT-slow preset achieves 42.19%/50.01%/45.27% (PSNR YUV/SSIM YUV/VMAF) BD-rate saving and 36.7x speedup. When compared to x265, BVC achieves 50.05%/58.81%/51.74% BD-rate saving and 1.5x speedup. When compared to AOM-AV1, BVC achieves 24.95%/27.88%/33.42% BD-rate saving and 1.6x speedup.

Table 5: BVC OT-slow preset in CRF compared to HM RA in CQP

|  | PSNR BD-Rate |  |  |  | SSIM BD-Rate |  |  |  | VMAF | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |  |  |
| Class A1 | -38.14% | -49.43% | -54.89% | -41.42% | -46.49% | -64.13% | -65.57% | -49.12% | -40.20% | 54.5 |
| Class A2 | -40.52% | -51.72% | -51.58% | -43.40% | -47.07% | -62.17% | -63.28% | -50.13% | -48.52% | 40.4 |
| Class B | -39.28% | -65.75% | -63.79% | -45.05% | -49.29% | -73.82% | -72.20% | -53.11% | -48.50% | 38.8 |
| Class C | -34.65% | -53.35% | -51.47% | -38.30% | -44.16% | -59.87% | -60.29% | -46.71% | -42.61% | 18.1 |
| Overall | -38.07% | -56.37% | -56.28% | -42.19% | -46.92% | -65.83% | -65.91% | -50.01% | -45.27% | 36.7 |
| UHD+HD Average | -39.31% | -57.47% | -58.03% | -43.61% | -47.92% | -68.00% | -67.96% | -51.21% | -46.24% | 43.5 |

Table 6: BVC OT-medium preset compared to x265 (placebo preset) RA in CRF

|  | PSNR BD-Rate |  |  |  | SSIM BD-Rate |  |  |  | VMAF | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |  |  |
| Class A1 | -44.23% | -58.65% | -65.10% | -49.13% | -54.43% | -71.42% | -73.75% | -57.47% | -46.74% | 1.6 |
| Class A2 | -45.49% | -61.79% | -63.02% | -49.66% | -54.15% | -69.26% | -71.17% | -57.91% | -52.91% | 1.3 |
| Class B | -47.69% | -72.76% | -71.30% | -53.43% | -59.09% | -79.47% | -78.19% | -62.05% | -56.44% | 1.4 |
| Class C | -42.05% | -63.41% | -62.59% | -46.81% | -54.05% | -68.50% | -69.05% | -56.43% | -48.72% | 1.9 |
| Overall | -45.05% | -65.25% | -66.08% | -50.05% | -55.83% | -72.89% | -73.46% | -58.81% | -51.74% | 1.5 |
| UHD+HD Average | -46.15% | -65.92% | -67.35% | -51.23% | -56.47% | -74.49% | -75.06% | -59.67% | -52.83% | 1.4 |

Table 7: BVC OT-medium preset compared to AOM-AV1 (CPU-used=3) in CRF

|  | PSNR BD-Rate |  |  |  | SSIM BD-Rate |  |  |  | VMAF | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |  |  |
| Class A1 | -19.12% | -30.82% | -31.93% | -23.72% | -22.15% | -32.62% | -32.74% | -25.62% | -28.42% | 1.4 |
| Class A2 | -26.53% | -35.64% | -40.14% | -29.15% | -30.42% | -40.43% | -47.32% | -34.45% | -37.63% | 1.3 |
| Class B | -24.19% | -39.05% | -37.96% | -27.36% | -28.70% | -45.11% | -41.85% | -30.72% | -36.94% | 2.1 |
| Class C | -17.22% | -27.53% | -29.43% | -19.71% | -19.39% | -29.52% | -31.87% | -21.09% | -29.61% | 1.5 |
| Overall | -21.79% | -33.65% | -34.91% | -24.95% | -25.25% | -37.52% | -38.46% | -27.88% | -33.42% | 1.6 |
| UHD+HD Average | -23.45% | -35.87% | -36.91% | -26.85% | -27.38% | -40.43% | -40.86% | -30.35% | -34.80% | 1.7 |

Real-time communication use case

Although we only began optimizing BVC for the RTC use case this year, we have already observed the significant potential of VVC for RTC applications. The BVC encoder's performance for the RTC use case were evaluated by comparing to HM LDP, x265 and AOM-AV1 in CQP. As shown in Table 8 to Table 10, BVC achieves 0.75%/0.44%/10.63% (PSNR YUV/SSIM YUV/VMAF ) BD-rate saving with 194x speedup compared to HM-18.0 LDP, and 47.05%/50.38%/40.24% and 22.57%/21.59%/30.82% BD-rate saving compared to x265 and AOM-AV1 in CQP mode, respectively.

Table 8: BVC RTC-fast preset compared to HM LDP in CQP

|  | PSNR BD-Rate |  |  |  | SSIM BD-Rate |  |  |  | VMAF | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |  |  |
| Class B | -0.66% | -1.37% | -0.80% | -0.38% | -0.79% | -2.95% | -3.82% | -0.91% | -11.04% | 203 |
| Class C | 1.82% | 2.60% | 2.48% | 2.21% | 2.05% | 4.00% | 2.60% | 2.49% | -4.62% | 137 |
| Class E | -1.73% | -17.37% | -18.21% | -5.30% | -1.01% | -18.44% | -18.97% | -3.55% | -17.95% | 372 |
| Overall | -0.10% | -4.04% | -4.06% | -0.75% | 0.10% | -4.51% | -5.47% | -0.44% | -10.63% | 194 |

Table 9: BVC RTC-veryfast preset compared to x265 superfast LDP in CQP

|  | PSNR BD-Rate |  |  |  | SSIM BD-Rate |  |  |  | VMAF | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |  |  |
| Class B | -43.55% | -52.99% | -54.80% | -45.67% | -48.89% | -55.27% | -57.89% | -49.92% | -37.22% | 1.5 |
| Class C | -41.24% | -52.14% | -53.23% | -43.62% | -44.93% | -55.25% | -57.57% | -46.90% | -36.30% | 1.0 |
| Class E | -52.18% | -58.30% | -60.80% | -53.91% | -54.99% | -58.43% | -61.91% | -55.78% | -50.53% | 1.1 |
| Overall | -44.94% | -54.03% | -55.78% | -47.05% | -49.10% | -56.05% | -58.79% | -50.38% | -40.24% | 1.2 |

Table 10: BVC RTC-veryfast preset compared to AOM-AV1 in CQP

|  | PSNR BD-Rate |  |  |  | SSIM BD-Rate |  |  |  | VMAF | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |  |  |
| Class B | -26.07% | -21.87% | -23.42% | -25.59% | -25.17% | -20.19% | -21.31% | -24.62% | -33.63% | 1.0 |
| Class C | -20.67% | -23.66% | -26.08% | -21.54% | -18.38% | -23.72% | -25.12% | -19.28% | -25.79% | 1.6 |
| Class E | -19.11% | -14.35% | -21.11% | -18.90% | -20.78% | -7.61% | -15.32% | -19.63% | -32.85% | 1.6 |
| Overall | -22.53% | -20.58% | -23.73% | -22.57% | -21.81% | -18.22% | -21.08% | -21.59% | -30.82% | 1.3 |

Image coding use case

The BVC encoder's performance at the fast/medium/slow presets in the image coding use case were evaluated. The results compared to HM, x265 and AOM-AV1 in AI configuration are presented Table 11 to Table 13, respectively. When compared to HM, BVC achieves 22.75%/24.60%/27.80% (PSNR YUV/SSIM YUV/VMAF) BD-rate saving and 3.5x speedup. When compared to x265, BVC achieves 25.49%/27.02%/27.84% BD-rate saving, and the encoding speed is 2.6x speedup. When compared to AOM-AV1, BVC achieves 14.04%/15.04%/22.30% BD-rate saving and 1.7x speedup.

Table 11: BVC Image-slow preset in CRF compared to HM AI in CQP

|  | PSNR BD-Rate |  |  |  | SSIM BD-Rate |  |  |  | VMAF | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |  |  |
| Class A1 | -24.05% | -36.33% | -35.27% | -27.25% | -24.69% | -53.21% | -45.19% | -28.24% | -27.11% | 6.9 |
| Class A2 | -22.66% | -27.52% | -25.80% | -23.55% | -23.83% | -35.53% | -35.21% | -25.27% | -30.24% | 3.2 |
| Class B | -19.01% | -32.69% | -34.98% | -21.65% | -19.99% | -40.93% | -45.52% | -23.14% | -27.13% | 3.1 |
| Class C | -18.98% | -23.19% | -26.16% | -20.15% | -21.51% | -29.56% | -34.31% | -23.17% | -27.30% | 1.9 |
| Overall | -20.74% | -29.85% | -30.85% | -22.75% | -22.10% | -39.27% | -40.40% | -24.60% | -27.80% | 3.5 |
| UHD+HD Average | -21.38% | -32.27% | -32.55% | -23.70% | -22.32% | -42.80% | -42.62% | -25.11% | -27.98% | 4.1 |

Table 12: BVC Image-fast preset compared to x265 (placebo preset) AI in CRF

|  | PSNR BD-Rate |  |  |  | SSIM BD-Rate |  |  |  | VMAF | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |  |  |
| Class A1 | -22.01% | -41.67% | -41.31% | -27.36% | -22.71% | -58.37% | -51.09% | -27.91% | -23.90% | 3.7 |
| Class A2 | -18.67% | -28.02% | -27.74% | -20.94% | -19.78% | -37.60% | -39.17% | -22.88% | -26.03% | 2.9 |
| Class B | -20.72% | -44.47% | -38.83% | -24.31% | -21.29% | -55.70% | -48.40% | -24.55% | -25.85% | 2.5 |
| Class C | -26.85% | -38.18% | -39.28% | -28.99% | -30.75% | -44.64% | -46.20% | -32.55% | -34.64% | 2.1 |
| Overall | -22.20% | -38.94% | -37.23% | -25.49% | -23.79% | -49.66% | -46.51% | -27.02% | -27.84% | 2.6 |
| UHD+HD Average | -20.51% | -39.22% | -36.48% | -24.22% | -21.27% | -51.49% | -46.62% | -25.01% | -25.37% | 2.9 |

Table 13: BVC Image-medium preset compared to AOM-AV1 (cpu-used=3) in CRF

|  | PSNR BD-Rate |  |  |  | SSIM BD-Rate |  |  |  | VMAF | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |  |  |
| Class A1 | -18.55% | -7.51% | -6.12% | -15.70% | -18.40% | -10.12% | -8.60% | -17.33% | -24.44% | 1.8 |
| Class A2 | -18.75% | -1.23% | -0.42% | -15.45% | -18.67% | 0.88% | 0.23% | -14.90% | -20.68% | 1.1 |
| Class B | -15.62% | 1.57% | -3.99% | -13.26% | -16.12% | 3.71% | -2.62% | -14.20% | -22.57% | 2.1 |
| Class C | -14.91% | -0.96% | -2.38% | -12.71% | -16.62% | -0.25% | -2.59% | -14.47% | -21.57% | 1.7 |
| Overall | -16.64% | -1.48% | -3.27% | -14.04% | -17.22% | -0.68% | -3.24% | -15.04% | -22.30% | 1.7 |
| UHD+HD Average | -17.27% | -1.67% | -3.60% | -14.52% | -17.44% | -0.83% | -3.47% | -15.24% | -22.57% | 1.7 |

Conclusion

The BVC encoder, an optimized VVC software solution, offers a range of features and presets and has been applied across diverse use cases and products. We believe there is still significant potential for BVC to achieve even better coding performance across all use cases.

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10,” JVET-T2001, Oct. 2020.

- G. Sullivan, “Deployment status of the VVC standard,” JVET-AI0021, JVET, July 2024.

- J. Cui, Y. Fan, Y. He, X. Jiang, H. Liu, H. Shi, H. Yang, H. Yang, H. Yin, J. Zhang, L. Zhang, “An optimized VVC encoder implementation,” JVET-V0127, Apr. 2021.

- Y. He, L. Li, Y. Li, H. Yin, J. Zhang, L. Zhang, Y. Zhang, “Performance of a VVC software decoder – BVC,” JVET-V0128, Apr. 2021.

- X. Gao, Y. He, X. Jiang, Y. Li, J. Wang, Y. Wu, L. Zhang, W. Zhang, W. Zhu, “Update on optimized VVC encoder implementation BVC for live-streaming,” JVET-AD0361, Apr. 2023.

- https://github.com/videolan/x265

- https://vcgit.hhi.fraunhofer.de/jvet/HM/-/tags/HM-18.0

- https://aomedia.googlesource.com/aom/

- K. Sühring, K. Sharman, “Common Test Conditions for HM Video Coding Experiments,” JVET-U1100, Dec. 2020.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).