| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0257-v1 |
| --- | --- |

| Title: | EE2-4.2: Improved implicit MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Charles Bonnineau Saurabh Puri Karam Naser Fabrice Le Léannec | Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 Test 4.2 on improved implicit MTS, initially presented in contribution JVET-AI0223 [1]. In this proposal implicit MTS uses a lookup-table (LUT) to infer the primary transform pair of the current block based on its intra mode and TU size.

Two EE2 tests are presented. In test 4.2a, the proposed implicit MTS method is only applied to ISP blocks following CTC configuration settings. In test 4.2b, the proposed implicit MTS method is also applied to other intra modes using non-CTC configuration settings (i.e., MTS=0, MTSImplicit=1).

The following results are obtained for 4.2a compared to ECM-14.0:

AI {-0.02% Y, -0.03% U, 0.00% V, xxx.x% EncT, xxx.x% DecT}

RA {x.xx% Y, x.xx% U, x.xx% V, xxx.x% EncT, xxx.x% DecT}

LDB {x.xx% Y, x.xx% U, x.xx% V, xxx.x% EncT, xxx.x% DecT}

The following results are obtained for 4.2b compared to ECM-14.0 with MTS=0 and MTSImplicit=1:

AI {-0.15% Y, 0.06% U, 0.09% V, xxx.x% EncT, xxx.x% DecT}

RA {x.xx% Y, x.xx% U, x.xx% V, xxx.x% EncT, xxx.x% DecT}

LDB {x.xx% Y, x.xx% U, x.xx% V, xxx.x% EncT, xxx.x% DecT}

# Introduction

Implicit MTS has been introduced in VVC as an alternative tool to explicit MTS. This tool reduces the encoder complexity by inferring the optimal transform type from decoder-side information instead of performing an intensive rate-distortion search in the encoder.  However, implicit MTS remained unchanged in the enhanced compression model (ECM), which introduce new primary transforms to explicit MTS.

It is asserted that the implicit MTS method can benefit from the introduction of the new primary transforms introduced in the ECM.

# Proposed method

In this proposal, the primary transform pair is directly inferred from the intra prediction mode and the TU size of the current block (sizeIdx) using a lookup-table (LUT). The LUT contains transform pairs based on the following separable primary transforms:

{DCT2, DCT5, DCT8, DST1, DST4, DST7}

For ISP blocks, the sizeIdx used as input to the LUT is based on the position of the current ISP subpartition within the ISP block.

Regarding test 4.2b, the proposed implicit MTS method is applied to angular modes only. The default implicit MTS method (i.e., based on the shape of the TU) is kept for TIMD and DIMD modes. DCT2 is used for MIP, EIP, SGPM and IntraTMP modes.

# Experimental results

The proposed method is implemented on top of the ECM-14.0 [2]. For test 4.2a, the simulations were performed following the CTCs [4] for the all-intra (AI), random access (RA) and low delay B (LDB) configurations. For test 4.2b, the following configuration parameters are used: --MTS=0, --MTSImplicit=1.

Table 1: Experimental results of Test 4.2a over ECM-14.0 (AI)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.06% | -0.04% |  |  | 100.0% | 100.5% |
| Class A2 | -0.01% | -0.11% | 0.00% |  |  | 100.0% | 100.2% |
| Class B | -0.01% | -0.01% | 0.08% |  |  | 101.6% | 100.0% |
| Class C | -0.02% | -0.07% | -0.13% |  |  | 100.1% | 100.0% |
| Class E | -0.03% | -0.03% | 0.07% |  |  | 100.0% | 100.0% |
| Overall | -0.02% | -0.03% | 0.00% |  |  | 100.4% | 100.1% |
| Class D | -0.02% | 0.10% | -0.09% |  |  | 100.0% | 99.9% |
| Class F | -0.03% | -0.03% | 0.01% |  |  | 98.2% | 100.1% |

Table 2: Experimental results of Test 4.2a over ECM-14.0 (RA)

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

Table 3: Experimental results of Test 4.2a over ECM-14.0 (LDB)

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

Table 4: Experimental results of Test 4.2b over ECM-14.0 with MTS=0 and MTSImplicit=1 (AI)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 with --MTS=0 --MTSImplicit=1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.15% | 0.15% | -0.10% |  |  | 100.0% | 100.4% |
| Class A2 | -0.09% | -0.05% | 0.04% |  |  | 100.0% | 100.6% |
| Class B | -0.22% | 0.08% | 0.16% |  |  | 102.0% | 100.2% |
| Class C | -0.10% | 0.05% | 0.06% |  |  | 99.2% | 100.2% |
| Class E | -0.13% | 0.02% | 0.24% |  |  | 100.1% | 100.3% |
| Overall | -0.15% | 0.06% | 0.09% |  |  | 100.4% | 100.3% |
| Class D | -0.12% | -0.11% | -0.10% |  |  | 100.2% | 99.9% |
| Class F | -0.08% | 0.11% | -0.14% |  |  | 100.9% | 100.2% |

Table 5: Experimental results of Test 4.2b over ECM-14.0 with MTS=0 and MTSImplicit=1 (RA)

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 with --MTS=0 --MTSImplicit=1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

Table 6: Experimental results of Test 4.2b over ECM-14.0 with MTS=0 and MTSImplicit=1 (LDB)

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 with --MTS=0 --MTSImplicit=1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

# Conclusion

This contribution proposes an intra mode and TU size dependent implicit MTS method. Considering the coding efficiency improvement, we recommend integrating the tool in next version of the ECM.

# References

- C. Bonnineau, et. al., “AHG12: Improved implicit MTS” JVET-AI0223, July 2024

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AH2017, October 2023

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).