| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0258-v1 |
| --- | --- |

| Title: | AHG9: Refinement and extension of IFM SEI message |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | P. de Lagrange D. Doyen E. François F. Urban C. Salmon-Legagneur | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |  |

_____________________________

# Abstract

In the context of the image format metadata SEI message considered in JVET-AI2023, this contribution first proposes a formulation of semantics for Exif metadata (when ifm_type_id equals zero) that avoids normative reference to CIPA and JEITA or a specific version of an Exif document, leveraging the fact that Exif attributes are stored as a TIFF data structure that must contain specific information. This contribution further proposes to extend the IFM SEI message to support metadata as TIFF data structures not tied to Exif with a new value of ifm_type_id.

# Formulation of semantics for Exif metadata

Without changing the functionality, this contribution proposes to resolve the editorial notes present in JVET-AI2032, related to the formulation of semantics when the data payload is EXIF metadata (ifm_type_id equal to 0), by avoiding normative references to documents published by CIPA and JEITA, that are not (yet) organizations approved by ITU SG16 for normative referencing.

Also, it proposes a more specific description of the data payload while avoiding reference to a specific EXIF document, that would cause this SEI message to be tied to a specific EXIF version.

## More specific description of data payload

The current description of the data payload is “Exif metadata as specified by (Ed Note: awaiting conclusion of ITU A.5 and A.4 processes)”, and “Exif metadata is specified by any of the Exchangeable image file (Exif) format for digital still cameras standards developed jointly by the Camera & Imaging Products Association (CIPA) and the Japan Electronics and Information Technology Industries Association (JEITA), for the associated video source pictures prior to encoding, e.g., for camera-captured content. To date, there are multiple versions of Exif deployed, each with its own indicator to signal the version number in use.”.

The editorial note that follows calls for a more specific definition, stating that first CIPA/JEITA can not be normatively referenced currently, and that a more specific reference is needed instead of “the concept of “any of the” standards produced by those organizations.”

Indeed, the Exif “standard” defines the organization and contents of data storage (e.g. SD card) in a digital camera: folder names and structure, image and sound formats, etc.

The specification of “Exif metadata” (or “attributes”) referred to in the IFM SEI message is actually found in section 4.6 of Exif v3.0 for example (“Tags” sub-section of “Exif Image File Specification” section), and more specifically sections 4.6.3 (“Exif-specific IFD”) and 4.6.6 (“Exif IFD attribute information”).

Fortunately, there is no need to be that specific, because Exif specifies that camera attribute information (or “Exif metadata” as understood here) is stored in a regular TIFF rev. 6.0 data structure, that contains standard tags, plus an Exif-defined set of extra tags (Exif private tags) inside a nested structure (the “Exif  IFD”, with tag id #0x8769). Exif also defines two other nested tag structures (or “IFD”): the GPS Info IFD (with tag id #0x8825), and interoperability IFD (tag #0xA005).

This TIFF structure is an integral part of the image file format when the image is in TIFF format, and is embedded in an APP segment when it is in JPEG format.

As illustrated in Figure 1 (image found on dev.exiv2.org), a TIFF data structure is made of a header, directories, and data. Data is addressed through offsets in the different tags in each directory.

Figure 1 – TIFF data structure

An example of a TIFF structure containing Exif metadata is given in Figure 2 (also found on dev.exiv2.org). In this example, many tags are standard TIFF ones. In Exif v3.0, section 4.6.9 defines which tags are mandatory (or forbidden) depending on image format (TIFF or JPEG).

STRUCTURE OF TIFF FILE (MM): ReaganExif.tif

address |    tag                           |      type |    count |    offset | value

10 | 0x0100 ImageWidth                |     SHORT |        1 |           | 200

22 | 0x0101 ImageLength               |     SHORT |        1 |           | 130

34 | 0x0102 BitsPerSample             |     SHORT |        4 |       242 | 8 8 8 8

46 | 0x0103 Compression               |     SHORT |        1 |           | 1

58 | 0x0106 PhotometricInterpretation |     SHORT |        1 |           | 2

70 | 0x010e ImageDescription          |     ASCII |      403 |       250 | 040621-N-6536T-062.USS Ronald Re ...

82 | 0x010f Make                      |     ASCII |       18 |       653 | NIKON CORPORATION

94 | 0x0110 Model                     |     ASCII |       10 |       671 | NIKON D1X

106 | 0x0112 Orientation               |     SHORT |        1 |           | 1

118 | 0x0115 SamplesPerPixel           |     SHORT |        1 |           | 4

130 | 0x011a XResolution               |  RATIONAL |        1 |       681 | 3000000/10000

142 | 0x011b YResolution               |  RATIONAL |        1 |       689 | 3000000/10000

154 | 0x011c PlanarConfiguration       |     SHORT |        1 |           | 1

166 | 0x0128 ResolutionUnit            |     SHORT |        1 |           | 2

178 | 0x0131 Software                  |     ASCII |       40 |       697 | Adobe Photoshop Elements 12.0 Ma ...

190 | 0x0132 DateTime                  |     ASCII |       20 |       737 | 2016:09:13 11:58:16

202 | 0x013b Artist                    |     ASCII |       34 |       757 | Photographer..s Mate 3rd Class ( ...

214 | 0x8769 ExifTag                   |      LONG |        1 |       792 | 792

226 | 0x8825 GPSTag                    |      LONG |        1 |      1400 | 1400

1422 | 0x0103 Compression               |     SHORT |        1 |           | 6

1434 | 0x011a XResolution               |  RATIONAL |        1 |      1498 | 72/1

1446 | 0x011b YResolution               |  RATIONAL |        1 |      1506 | 72/1

1458 | 0x0128 ResolutionUnit            |     SHORT |        1 |           | 2

1470 | 0x0201 JPEGInterchangeFormat     |      LONG |        1 |      1514 | 1514

1482 | 0x0202 JPEGInterchangeFormatLeng |      LONG |        1 |      4196 | 4196

END ReaganExif.tif

Figure 2 – Example EXIF data

Instead of normatively referring to an Exif document, it is proposed to state that, when ifm_type_id equals to 0, the metadata payload is a TIFF rev. 6.0 data structure, that shall contain a tag with id 0x8769 (ExifTag). A note can further explain that the TIFF data structure is expected to be compliant with attribute information defined in an Exif standard (but this compliance is out of the scope of VSEI).

# Extension to support a TIFF data structure

This contribution further proposes to enable support of metadata as a standard TIFF rev. 6.0 data structure, when ifm_type_id equals 6, without enforcing the presence of ExifTag (id 0x8769), since standard TIFF tags can already contain useful information, as illustrated in Figure 2.

# References

- S. McCarthy, J. Boyce, J. Chen, S. Deshpande, M. Hannuksela, Hendry, Y.-K. Wang, “Technologies under consideration for future extensions of VSEI (version 5)”, JVET-AI2032, Sapporo, JP, July 2024

- TIFF specification rev. 6.0: https://www.itu.int/itudoc/itu-t/com16/tiff-fx/docs/tiff6.pdf

- Exiv2 wiki, “The Metadata in TIFF files”		
https://dev.exiv2.org/projects/exiv2/wiki/The_Metadata_in_TIFF_files

- Exiv2 wiki, “The Metadata in JPEG files”	
https://dev.exiv2.org/projects/exiv2/wiki/The_Metadata_in_JPEG_files

- Exiv2, “Standard Exif Tags” - https://exiv2.org/tags.html

- CIPA, “Exchangeable image file format for digital still cameras: Exif Version 3.0”	
https://cipa.jp/std/documents/download_e.html?DC-008-Translation-2023-E

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).