| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0259-v1 |
| --- | --- |

| Title: | EE2-related: On implicit MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Patrick Garus Bappaditya Ray Pavel Nikitin | Email: | martak@qti.qualcomm.com pgarus@qti.qualcomm.com bray@qti.qualcomm.com pnikitin@qti.qualcomm.com |
| Source: | Qualcomm Technologies Inc. |  |  |

_____________________________

# Abstract

In this contribution, an improved implicit MTS scheme is proposed by considering in addition to block shape, intra mode type and direction as in EE2-4.2, also the difference between two dominant directions of the predicted block.

The simulation results, based on ECM-14.0 [1] with implicit MTS configuration (MTS =0, MTSImplicit =1), are reported as follows:

AI: { -0.26% Y, -0.05% U, -0.06% V, 100.3% EncT, 101.2% DecT }

# Introduction

In VVC/ECM, the implicit transform selection uses two distinct transform kernels: DCT2 and DST7, and the choices are size dependent. DST7 is used when the size is in the range of [4, 16], otherwise DCT2 is used. In JVET-AI0223, implicit MTS is improved by using block shape and intra directional mode dependency and is currently being investigated in EE2 (EE2-4.2).

# Proposal

This contribution proposes improved implicit MTS by considering in addition to block shape, intra mode type (whether it is regular intra-prediction, DIMD, TIMD, SGPM, Intra-TMP, MIP or EIP) and intra direction as in EE-4.2, also the difference between two dominant directions of the coding block. Two dominant directions are estimated in a similar fashion as the multiple transform set selection for intra-LFNST/NSPT method studied in EE2-4.4a/b (JVET-AI0163). The existing DCT/DST kernels in ECM are being used for this method, thus no new transform kernels are added in ECM.

# Test Result

The proposed method is implemented on top of ECM-14.0 [1] with implicit MTS enabled (MTS = 0, MTSImplicit = 1). Simulations for anchor were performed by following the common test conditions [2]. The test results are as follows.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.25% | 0.01% | -0.24% | 100.5% | 100.6% | 100.2% | 99.0% |
| Class A2 | -0.24% | -0.23% | -0.09% | 100.0% | 100.7% | 100.4% | 99.2% |
| Class B | -0.31% | 0.04% | 0.01% | 100.0% | 101.6% | 100.5% | 99.1% |
| Class C | -0.20% | 0.01% | -0.11% | 100.3% | 101.1% | 101.2% | 100.0% |
| Class E | -0.28% | -0.17% | 0.08% | 101.3% | 101.6% | 99.1% | 100.0% |
| Overall | -0.26% | -0.05% | -0.06% | 100.3% | 101.2% | 100.3% | 99.5% |
| Class D | -0.22% | -0.14% | -0.08% | 100.8% | 100.9% | 100.8% | 100.0% |

# Reference

[1] M. Coban, and et al., “Algorithm description of Enhanced Compression Model 14 (ECM 14),” JVET-AI2025, July 2024.

[2] M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, July 2024.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).