| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0261-r2 |
| --- | --- |

| Title: | Non-EE2: TMVP Candidate Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Hong Limin Wang Krit Panusopone | Email: | Seungwook.Hong@nokia.com Limin.2.Wang@nokia.com Krit.Panusopone@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents an enhanced Temporal Motion Vector Predictor (TMVP) method on top of ECM-14.0. In the current ECM design, TMVP candidates are selected from either position C0 or position C1, but only one position is included in the candidate list. This contribution proposes that both positions C0 and C1 of a pair be checked, and motion information from both positions be included in the candidate list, provided there are still available slots for TMVP candidates. This contribution also proposes to increase the number of TMVP Merge candidates with the proposed TMVP candidate selection. Simulation results of these two changes are reported as follows:

Test 1 (including motion from both C0 and C1 positions):

LB: -0.07%, 0.02%, -0.29%, 101.0%, 99.6%.

RA: 0.01%, -0.02%, -0.02%, 100.8%, 101.1%

Test 2 (including motion from both C0 and C1 positions and increasing the number of TMVP Merge candidates):

LB: -0.07%, -0.03%, -0.50%, 100.6%, xx.x%.

RA: -0.01%, -0.04%, 0.02%, 100.1%, xx.x%

# Introduction

In ECM, TMVP candidates are selected from pairs of distinct locations, with each pair consisting of two positions C0, C1. However, only one of these positions is included in the candidate list, depending on whether valid motion information is available. C1 is only considered when motion information at C0 is unavailable or invalid, while C0 is selected if it contains valid motion information.

# Proposed Method

This contribution proposes two improvements to the current TMVP candidate selection. The first improvement checks both positions C0 and C1 for each pair. If both positions contain valid motion information, and if there are available slots in the candidate list, the motion vectors from both positions are included. This change enables better utilization of available motion information, potentially improving the accuracy of motion prediction. The proposed method ensures that both positions are checked without any bias towards position C0, which is currently favored in ECM. For the second improvement, the proposed method increases the number of TMVP Merge candidates as described in Table 1.

Table 1. The size of MaxNumberMergeCand, maxNumMergeCandStage2 in TMVPMergeCand

| tmMergeFlag | Test 1 | Test 2 |  |
| --- | --- | --- | --- |
| True | {3, 6} | {6, 12} |
| False | {8, 12} | {16, 24} |

# Experimental Results

The proposed method has been implemented on top of the ECM-14.0 software. Two tests were conducted based on the common test conditions specified in the JVET document [1]. Test 1 evaluates the coding performance of including motion from both the C0 and C1 positions, while Test 2 evaluates the effect of increasing the number of TMVP Merge candidates selected by the method in Test 1. The test results are summarized in Tables 2 and 3.

Table 2. Simulation results of Test 1

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.08% | -0.18% | 101.1% | 99.9% | 99.9% | 99.8% |
| Class C | -0.13% | -0.16% | -0.13% | 100.8% | 99.4% | 100.0% | 99.9% |
| Class E | -0.10% | 0.14% | -0.71% | 101.1% | 99.2% | 99.9% | 99.8% |
| Overall | -0.07% | 0.02% | -0.29% | 101.0% | 99.6% | 100.0% | 99.9% |
| Class D | -0.18% | 0.65% | 0.78% | 101.9% | 99.1% | 99.4% | 100.0% |
| Class F | -0.48% | -0.93% | -0.50% | 111.2% | 100.6% | 100.4% | 100.2% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.03% | -0.08% | -0.09% | 100.4% | 100.0% | 100.0% | 100.2% |
| Class A2 | 0.03% | -0.03% | -0.03% | 100.4% | 99.6% | 99.9% | 99.9% |
| Class B | 0.00% | -0.04% | -0.01% | 100.5% | 100.1% | 100.0% | 99.7% |
| Class C | -0.01% | 0.05% | 0.03% | 101.9% | 104.4% | 98.4% | 99.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.02% | 100.8% | 101.1% | 99.6% | 99.9% |
| Class D | -0.08% | -0.04% | 0.05% | 100.4% | 112.4% | 99.3% | 99.6% |
| Class F | 0.01% | 0.01% | 0.05% | 101.5% | 108.4% | 97.5% | 99.0% |

Table 3. Simulation results of Test 2 (QP 22 and QP 27 results for classes A1 and A2 (RA) and for class B and Arena of Valor (LDB) are copied from Test 1)

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | -0.05% | -0.40% | 100.4% | #NUM! | 99.9% | #NUM! |
| Class C | -0.10% | 0.02% | -0.33% | 100.8% | #NUM! | 99.1% | #NUM! |
| Class E | -0.07% | -0.05% | -0.91% | 100.7% | #NUM! | 100.5% | #NUM! |
| Overall | -0.07% | -0.03% | -0.50% | 100.6% | #NUM! | 99.8% | #NUM! |
| Class D | -0.14% | 0.11% | 0.47% | 101.4% | #NUM! | 99.7% | #NUM! |
| Class F | -0.30% | -0.49% | -0.10% | 107.3% | #NUM! | 99.3% | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.06% | -0.09% | 100.2% | #NUM! | 100.0% | #NUM! |
| Class A2 | 0.01% | 0.00% | -0.06% | 100.4% | #NUM! | 99.8% | #NUM! |
| Class B | -0.02% | -0.04% | 0.05% | 99.8% | #NUM! | 100.0% | #NUM! |
| Class C | -0.01% | -0.04% | 0.13% | 100.1% | #NUM! | 98.5% | #NUM! |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.04% | 0.02% | 100.1% | #NUM! | 99.5% | #NUM! |
| Class D | -0.01% | -0.04% | -0.02% | 98.8% | #NUM! | 99.4% | #NUM! |
| Class F | 0.00% | -0.08% | -0.05% | 99.7% | #NUM! | 97.8% | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

# Conclusions

The simulation results show -0.07%, 0.03%, -0.50% gain on Y/Cb/Cr with 10.1%/xx.x% encoding/decoding time in LB condition and -0.01%, -0.04%, 0.02% gain on Y/Cb/Cr with 100.1%/xx.x% encoding/decoding time in RA condition The proposed method demonstrates improved prediction performance by leveraging both TMVP positions and increasing the number of TMVP Merge candidates. It is recommended to include the proposed method in the next round of EE.

# References

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).