| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0262-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AJ0249 (EE2-2.20-2.21: Neural network based intra prediction with DIMD mode derivation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports identical PSNR and bitrates as those reported in JVET-AJ0249 on the combination of EE2-2.20 and EE2-2.21. The decoder time has also been verified to with several runs of decoding on same machine in parallel using the updated software October 23 (removing unnecessary computations, bit exact with software from October 11). The software has been inspected and is asserted to match the contribution.

# Introduction

The combination in JVET-AJ0249 is based on the intra prediction based neural network from NNVC-10.0 in EE2-2.21 with the intra mode derivation using decoder side intra mode derivation (DIMD) as was done in EE2-2.20 (also in ECM for IntraTMP, EIP, and MIP).

The proposed method is used for block sizes 4x4 up to 64x64. The number of models in this neural network is 6 (excluding the model 4x32 from NNVC-10.0, the models have been retrained to use more sparsity compared to the intra prediction neural network in NNVC-10.0) as shown below taken from JVET-AJ0249:

| Model | Parameter count (k) | MAC/pixel | Memory Footprint (MB) |
| --- | --- | --- | --- |
| 4x4 | 50.823 | 3176 | 0.195 |
| 8x4 | 61.886 | 1934 | 0.224 |
| 16x4 | 123.420 | 1928 | 0.450 |
| 8x8 | 132.260 | 2066 | 0.472 |
| 16x8 | 178.754 | 1396 | 0.590 |
| 16x16 | 215.376 | 841 | 0.596 |

In comparison to improved MIP in ECM the proposed NN intra uses in general a larger spatial neighbourhood (improved MIP used up to 2 rows/columns above current block). The input to the NN intra are L shaped rows above and columns to the left of the current block (at most the spatial coverage in rows are equal to height divided by 2 and columns are equal to width divided by 2, for a 64x64 block, the 32 rows/columns are then subsampled to 8 rows/columns before fed to the NN). The number of MAC/pixel is higher, up to 3176 MAC/pixel, compared to improved MIP which requires ~132MAC/pixel. The memory usage is lower than for improved MIP which requires about 3.7MB memory. In some cases when a neighboring mode is NN intra the candidate intra mode it set to planar mode supposedly to manage runtime.

# Results

The results have been compared to ECM-14.0 [1] based on the CTC for the explorations in [2]. Encoder time is not accurate. Decoder time is accurate and is computed in parallel on same machine several times without MD5 and writing reconstruction files using the updated software from October 23 but a software that is bit exact with the initial software from October 11.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,99% | -0,84% | -0,87% | 89,5% | 104,9% | 101,4% | 100,9% |
| Class A2 | -0,40% | -0,31% | -0,30% | 84,9% | 104,7% | 98,2% | 100,9% |
| Class B | -0,45% | -0,25% | -0,25% | 83,1% | 106,1% | 101,3% | 100,8% |
| Class C | -0,47% | -0,43% | -0,38% | 89,2% | 111,1% | 101,0% | 101,0% |
| Class E | -0,74% | -0,27% | -0,30% | 84,9% | 107,1% | 100,4% | 101,0% |
| Overall | -0,58% | -0,40% | -0,40% | 86,1% | 106,9% | 100,6% | 100,9% |
| Class D | -0,55% | -0,31% | -0,77% | 100,1% | 107,6% | 100,7% | 101,0% |
| Class F | -0,28% | -0,34% | -0,31% | 83,8% | 104,7% | 101,6% | 100,9% |
| Class TGM | 0,00% | -0,05% | -0,05% | 77,2% | 99,9% | 101,5% | 100,5% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,33% | -0,51% | -0,52% | 87,7% | 100,1% | 100,3% | 100,8% |
| Class A2 | -0,13% | -0,14% | -0,14% | 88,5% | 99,9% | 100,2% | 100,5% |
| Class B | -0,20% | -0,06% | -0,07% | 82,7% | 100,6% | 100,2% | 100,3% |
| Class C | -0,21% | -0,21% | -0,07% | 86,4% | 101,0% | 100,4% | 100,8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,22% | -0,21% | -0,18% | 85,8% | 100,5% | 100,3% | 100,6% |
| Class D | -0,21% | -0,27% | -0,62% | 92,5% | 101,2% | 100,7% | 100,8% |
| Class F | -0,01% | -0,41% | -0,10% | 90,4% | 100,3% | 100,2% | 100,4% |
| Class TGM | -0,06% | -0,07% | 0,05% | 84,5% | 100,2% | 100,2% | 100,0% |

# References

- F. Galpin, Y. Li, D. Rusanovskyy, J. Ström, L. Wang., “Algorithm description for neural network-based video coding (NNVC-10)”, JVET-AJ2019, 35th Meeting, Sapporo, JP, 12–19 July 2024.

- F. Urban, T. Dumas, F. Galpin, E. François (Interdigital), S. Eadie, P. Garus, P. Nikitin, M. Coban, V. Seregin, M. Karczewicz (Qualcomm), “AhG12: neural network-based intra prediction”, JVET-AJ0249, 36th Meeting, Antalya, TR, 1–8 November 2024.

- M. Karczewicz, Y. Ye , “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).