| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0263-v1 |
| --- | --- |

| Title: | AHG9: New Auxiliary Picture Type and SEI for Image Segmentation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis M. Tourapis, Dimitri Podborski, Seethal Paluri, Jungsun Kim | Email: | {atourapis, dpodborski, s_paluri, jungsun_kim} |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

HEVC currently supports two auxiliary image types that carry alpha and depth data. It also defines two SEI messages that specify how this data should be processed (alpha channel information and depth representation information). Currently, alpha use is only limited for carrying transparency information. However, a use case to utilize alpha for the indication of masking and selection information, e.g. for segmentation and content authoring purposes is also identified.

This contribution presents four options to address this use case: (i) introducing a new auxiliary picture type and a new SEI message, (ii) defining a new mode for alpha_channel_use_idc, (iii) defining a new alpha_channel_use_idc and a new SEI message, or (iv) defining a new SEI message. Assuming consensus on the utility of this use case for alpha, despite the common JVET practice of not updating or extending existing SEI messages, the proponents recommend adopting either option (i) or (iii).

# Introduction

The term alpha channel technically refers to the fourth channel in an RGBA image file that is commonly used to carry transparency data. In one application, the transparency data, when present, can be used to determine how pixels from multiple images that need to be rendered together can be blended before rendering. This can be used in video editing and production applications among others. For example, an application can determine the foreground and background [1][2] in a scene and mark in the alpha channel the corresponding locations with appropriately selected values (e.g. assuming an 8 bit alpha, 0 for the background and 255 for the foreground). During rendering or authoring, the pixels marked as either background or foreground in the alpha channel can easily be replaced with other data from other images. Several image and video coding formats and specifications [4], such as HEVC [5] and VVC, support alpha channels. HEVC and VVC support alpha as auxiliary video data and utilize SEI messages to infer their characteristics and usage. This auxiliary video data layer only contains values in the first channel, i.e. the luma, of the video signal, which represent the different levels of opacity in the scene.

A less common use of alpha, which currently is not supported in these specifications, is alpha as a tool for region masking, annotation, selection, and segmentation. An example use is mentioned in [3]. In this application, the alpha layer (or layers) could be used to identify non overlapping objects in a frame. In an example shown in Figure 1, six objects/segments were identified highlighted with a different value in the alpha layer image. An optional id, or even other information such as a label, could also be provided that is associated with each object/segment. For example, an ID equal to 5 could be associated with a clock. Overlapping segments could potentially be identified by using multiple alpha channels.

Figure 1 Example of segmenting and mapping to values in the alpha frame to IDs.

In the above example, if a user wanted to select the Safari Browser window (object with ID equal to 4), the decoder could select the pixels in the texture image that corresponded to the samples in the alpha plane that had the value associated with ID equal to 4. All other pixels would be ignored or zeroed out. If, instead, another object with another ID was needed, the decoder would select the corresponding texture pixels and ignore all others. Multiple objects could also be selected if desired. In an essence this can also be seen as a “dynamic” alpha mask with the user being able to control which object or objects should be opaque and which areas should be fully transparent. Further operations such as cropping could also benefit from this segmentation technique.

The current HEVC [5] specification supports the alpha_channel_info() SEI message for the indication of the properties of an alpha channel. This could potentially be extended to support additional alpha channel types using a new value for the syntax element alpha_channel_use_idc, and could even be extended by adding more syntax elements at the tail end of this SEI message. However, the proponents are aware that it has been a long standing practice in JVET not to allow the extension of existing SEI messages to not introduce backward compatibility issues when parsing such updated SEI message. Assuming that that option is not possible, this document proposes several other solutions that should be acceptable to the group for integration in the HEVC specification and potentially also in the V.SEI specification.

# Proposal

To address the proposed new alpha use case, four alternative approaches are presented below. Each approach is designed to provide a distinct solution, with details and specific syntax provided in the sections referenced.

- Approach 1: Define a new AuxId and introduce a dedicated picture segmentation info SEI, i.e. picture_segmentation_info(), message. This solution makes it very clear to decoders that this involves a new auxiliary image use case and does not impact any existing metadata. See section 3.1 for more details.

- Approach 2: Reuse the existing alpha auxiliary picture type, introducing only a new alpha_channel_use_idc value in the existing alpha channel information SEI message. No further information is provided for each segment and the value in the alpha picture is also considered as directly corresponding to an ID for any other form of consideration or processing. More information is available in section 3.2.

- Approach 3: Similar to approach 2, this method reuses the alpha auxiliary type but introduces a new picture segmentation information SEI message. This enables mapping between numerical values present in the auxiliary picture and segment IDs, adding flexibility. Refer to Section 3.3 for further explanation.

- Approach 4: Reuse the existing alpha auxiliary picture type while using the same SEI message as in Approach 1 to specify its usage while ignoring any information in an alpha channel information SEI message. For detailed information, see Section 3.4.

An implementation of the segment selection approach will be provided as a new tool in HDRTools, named as AlphaPicker.

# Proposed Syntax

## Definition of New AuxId and New SEI Message

Modify Table F.2 as follows:

Table F.2 – Mapping of AuxId to the type of auxiliary pictures

| AuxId | Name of AuxId | Type of auxiliary pictures | SEI message describing interpretation of auxiliary pictures |
| --- | --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane | Alpha channel information |
| 2 | AUX_DEPTH | Depth picture | Depth representation information |
| 3 | AUX_SEGMENT | Segmentation plane | Picture segmentation information |
| 4..127 |  | Reserved |  |
| 128..159 |  | Unspecified |  |
| 160..255 |  | Reserved |  |

AuxId[ lId ] shall be in the range of 0 to 2 3, inclusive, or 128 to 159, inclusive, for bitstreams conforming to this version of this Specification. Although the value of AuxId[ lId ] shall be in the range of 0 to 2 3, inclusive, or 128 to 159, inclusive, in this version of this Specification, decoders shall allow values of AuxId[ lId ] in the range of 0 to 255, inclusive.

It is a requirement of bitstream conformance that when AuxId[ lId ] is equal to AUX_ALPHA, or AUX_DEPTH, or AUX_SEGMENT either of the following applies:

–  chroma_format_idc is equal to 0 in the active SPS for the layer with nuh_layer_id equal to lId.

–  The value of all decoded chroma samples is equal to 1 << ( BitDepthC − 1 ) in all pictures that have nuh_layer_id equal to lId and for which this VPS RBSP is the active VPS RBSP.

Add the following:

Table F.14.2.12 Picture Segmentation information SEI message syntax

| picture_segmentation_info( payloadSize ) { | Descriptor |
| --- | --- |
| segment_cancel_flag | u(1) |
| if ( !segment_cancel_flag ) { |  |
| segment_channel_bit_depth_minus8 | u(3) |
| segment_min_value | u(v) |
| segment_max_value | u(v) |
| MinValue = segment_min_value |  |
| MidValue = ( segment_max_value + MinValue ) / 2 |  |
| MaxValue = Max( segment_max_value, MinValue ) |  |
| segment_channel_clip_flag | u(1) |
| if ( segment_channel_clip_flag ) |  |
| segment_channel_clip_type_flag | u(1) |
| segment_label_idc | u(2) |
| if ( segment_label_idc == SEGMENT_ID_FULL ) { |  |
| for ( i = MinValue; i <= MaxValue; i++ ) { |  |
| segment_type_id[ i ] | u(16) |
| } |  |
| } |  |
| else if ( segment_label_idc == SEGMENT_ID_SPARSE ) { |  |
| segment_values_cnt | u(v) |
| for ( i = 0; i < segment_values_cnt; i++ ) { |  |
| segment_value[ i ] | u(v) |
| segment_type_id[ segment_value[ i ] ] | u(16) |
| } |  |
| } |  |
| // Other types of information could also be added, such as a text label |  |
| } |  |
| } |  |

F.14.3.12 Picture Segmentation information SEI message semantics

The picture segmentation information SEI message provides information that could be used for the purposes of picture segmentation, annotation, and segment extraction using auxiliary pictures of type AUX_SEGMENT and one or more associated primary pictures.

For an auxiliary picture with nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] equal to AUX_SEGMENT, an associated primary picture, if any, is a picture in the same access unit having AuxId[ nuhLayerIdB ] equal to 0 such that ScalabilityId[ LayerIdxInVps[ nuhLayerIdA ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ nuhLayerIdB ] ][ j ] for all values of j in the range of 0 to 3, inclusive, and 4 to 15, inclusive.

When an access unit contains an auxiliary picture picA with nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] equal to AUX_SEGMENT, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–  The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

–  A CLVS containing the auxiliary picture picA ends.

– A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the picture segmentation information SEI message applies.

segment_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous picture segmentation info SEI message in output order that applies to the current layer. segment_cancel_flag equal to 0 indicates that picture segmentation information follows.

Let currPic be the picture that the picture segmentation information SEI message is associated with. The semantics of picture segmentation information channel SEI message persist for the current layer in output order until one or more of the following conditions are true:

–  A new CLVS of the current layer begins.

–  The bitstream ends.

–  A picture picB with nuh_layer_id equal to targetLayerId in an access unit containing an picture segmentation information SEI message with nuh_layer_id equal to targetLayerId is output having PicOrderCnt(picB) greater than PicOrderCnt( currPic ), where PicOrderCnt( picB ) and PicOrderCnt( currPic ) are the PicOrderCntVal values of picB and currPic, respectively, immediately after the invocation of the decoding process for picture order count for picB.

segment_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the auxiliary picture. segment_channel_bit_depth_minus8 shall be in the range 0 to 7 inclusive. segment_channel_bit_depth_minus8 shall be equal to bit_depth_luma_minus8 of the associated primary picture.

segment_min_value specifies the minimum sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered for the purposes of clipping.

segment_max_value specifies the maximum sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered for the purposes of clipping.

When present, segment_max_value shall be greater than or equal to segment_min_value.

segment_channel_clip_flag equal to 0 indicates that no clipping operation is applied to obtain the interpretation sample values of the decoded auxiliary picture. segment_channel_clip_flag equal to 1 indicates that the interpretation sample values of the decoded auxiliary picture are altered according to the clipping process described by the segment_channel_clip_type_flag syntax element. When not present, the value of segment_channel_clip_flag is inferred to be equal to 0.

segment_channel_clip_type_flag equal to 0 indicates that, for purposes of segment blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than ( segment_max_value – segment_min_value ) / 2 is set equal to segment_max_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less or equal than ( segment_max_value – segment_min_value ) / 2 is set equal to segment_min_value to obtain the interpretation sample value for the auxiliary picture luma sample. segment_channel_clip_type_flag equal to 1 indicates that, for purposes of segment blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than segment_max_value is set equal to segment_max_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less than or equal to segment_min_value is set equal to segment_min_value to obtain the interpretation sample value for the auxiliary picture luma sample.

segment_label_idc indicates the type of label used to identify the segment. The value of segment_label_idc shall be in the range of 0 to 3, inclusive.

Table F.14.3.13 Segment label indicated by segment_label_idc

| segment_label_idc | Segment Label |
| --- | --- |
| 0 | SEGMENT_ID_FULL |
| 1 | SEGMENT_ID_SPARSE |
| 2-3 | Reserved |

segment_values_cnt specifies the number of possible values that will be identified with an explicit ID in subsequent syntax.

segment_value[ i ] indicates the value for which an explicit ID is to be identified. segment_value[ i ], when present, shall be in the range of 0 to 216 -1, inclusive.

segment_type_id[ i ] indicates the id of the i-th segment. segment_type_id[ i ] shall be in the range of 0 to 216 -1, inclusive.

## New alpha_channel_use_idc Value

Modify the semantics of alpha_channel_use_idc in section F.14.3.8 as follows:

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. alpha_channel_use_idc equal to 3 indicates that the decoded sample values of the auxiliary coded picture can be utilized for the purposes of picture segmentation, annotation, and segment extraction from one or more associated primary pictures. Values greater than 3 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2.

## New alpha_channel_use_idc Value and New SEI Message

Modify the semantics of alpha_channel_use_idc in section F.14.3.8 as follows:

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. alpha_channel_use_idc equal to 3 indicates that the decoded sample values of the auxiliary coded picture can be utilized for the purposes of picture segmentation, annotation, and segment extraction from one or more associated primary pictures. Values greater than 3 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2.

| picture_segmentation_info( payloadSize ) { | Descriptor |
| --- | --- |
| segment_cancel_flag | u(1) |
| if ( !segment_cancel_flag ) { |  |
| segment_channel_bit_depth_minus8 | u(3) |
| segment_label_idc | u(2) |
| if ( segment_label_idc == SEGMENT_ID_FULL ) { |  |
| for ( i = MinValue; i <= MaxValue; i++ ) { |  |
| segment_type_id[ i ] | u(16) |
| } |  |
| } |  |
| else if ( segment_label_idc == SEGMENT_ID_SPARSE ) { |  |
| segment_values_cnt | u(v) |
| for ( i = 0; i < segment_values_cnt; i++ ) { |  |
| segment_value[ i ] | u(v) |
| segment_type_id[ segment_value[ i ] ] | u(16) |
| } |  |
| } |  |
| // Other types of information could also be added, such as a text label |  |
| } |  |
| } |  |

F.14.3.12 Picture Segmentation information SEI message semantics

The picture segmentation information SEI message provides information that could be used for the purposes of picture segmentation, annotation, and segment extraction using auxiliary pictures of type AUX_ALPHA, associated with an alpha channel information SEI message that indicates that alpha_channel_use_idc equal to 3, and one or more associated primary pictures.

segment_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous picture segmentation info SEI message in output order that applies to the current layer. segment_cancel_flag equal to 0 indicates that picture segmentation information follows Let currPic be the picture that the picture segmentation information SEI message is associated with. The semantics of picture segmentation information channel SEI message persist for the current layer in output order until one or more of the following conditions are true:

–  A new CLVS of the current layer begins.

–  The bitstream ends.

–  A picture picB with nuh_layer_id equal to targetLayerId in an access unit containing an picture segmentation information SEI message with nuh_layer_id equal to targetLayerId is output having PicOrderCnt(picB) greater than PicOrderCnt( currPic ), where PicOrderCnt( picB ) and PicOrderCnt( currPic ) are the PicOrderCntVal values of picB and currPic, respectively, immediately after the invocation of the decoding process for picture order count for picB.

segment_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the auxiliary picture. segment_channel_bit_depth_minus8 shall be in the range 0 to 7 inclusive. segment_channel_bit_depth_minus8 shall be equal to bit_depth_luma_minus8 of the associated primary picture.

segment_label_idc indicates the type of label used to identify the segment. The value of segment_label_idc shall be in the range of 0 to 3, inclusive.

Table F.14.3.13 Segment label indicated by segment_label_idc

| segment_label_idc | Segment Label |
| --- | --- |
| 0 | SEGMENT_ID_FULL |
| 1 | SEGMENT_ID_SPARSE |
| 2-3 | Reserved |

segment_values_cnt specifies the number of possible values that will be identified with an explicit ID in subsequent syntax.

segment_value[ i ] indicates the value for which an explicit ID is to be identified. segment_value[ i ], when present, shall be in the range of 0 to 216 -1, inclusive.

segment_type_id[ i ] indicates the id of the i-th segment. segment_type_id[ i ] shall be in the range of 0 to 216 -1, inclusive.

## New SEI Message

Add the following:

Table F.14.2.12 Picture Segmentation information SEI message syntax

| picture_segmentation_info( payloadSize ) { | Descriptor |
| --- | --- |
| segment_cancel_flag | u(1) |
| if ( !segment_cancel_flag ) { |  |
| segment_channel_bit_depth_minus8 | u(3) |
| segment_min_value | u(v) |
| segment_max_value | u(v) |
| MinValue = segment_min_value |  |
| MidValue = ( segment_max_value + MinValue ) / 2 |  |
| MaxValue = Max( segment_max_value, MinValue ) |  |
| segment_channel_clip_flag | u(1) |
| if ( segment_channel_clip_flag ) |  |
| segment_channel_clip_type_flag | u(1) |
| segment_label_idc | u(2) |
| if ( segment_label_idc == SEGMENT_ID_FULL ) { |  |
| for ( i = MinValue; i <= MaxValue; i++ ) { |  |
| segment_type_id[ i ] | u(16) |
| } |  |
| } |  |
| else if ( segment_label_idc == SEGMENT_ID_SPARSE ) { |  |
| segment_values_cnt | u(v) |
| for ( i = 0; i < segment_values_cnt; i++ ) { |  |
| segment_value[ i ] | u(v) |
| segment_type_id[ segment_value[ i ] ] | u(16) |
| } |  |
| } |  |
| // Other types of information could also be added, such as a text label |  |
| } |  |
| } |  |

F.14.3.12 Picture Segmentation information SEI message semantics

The picture segmentation information SEI message provides information that could be used for the purposes of picture segmentation, annotation, and segment extraction using auxiliary pictures of type AUX_ALPHA and one or more associated primary pictures.

segment_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous picture segmentation info SEI message in output order that applies to the current layer. segment_cancel_flag equal to 0 indicates that picture segmentation information follows

Let currPic be the picture that the picture segmentation information SEI message is associated with. The semantics of picture segmentation information channel SEI message persist for the current layer in output order until one or more of the following conditions are true:

–  A new CLVS of the current layer begins.

–  The bitstream ends.

–  A picture picB with nuh_layer_id equal to targetLayerId in an access unit containing an picture segmentation information SEI message with nuh_layer_id equal to targetLayerId is output having PicOrderCnt(picB) greater than PicOrderCnt( currPic ), where PicOrderCnt( picB ) and PicOrderCnt( currPic ) are the PicOrderCntVal values of picB and currPic, respectively, immediately after the invocation of the decoding process for picture order count for picB.

segment_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the auxiliary picture. segment_channel_bit_depth_minus8 shall be in the range 0 to 7 inclusive. segment_channel_bit_depth_minus8 shall be equal to bit_depth_luma_minus8 of the associated primary picture.

segment_min_value specifies the minimum sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered for the purposes of clipping.

segment_max_value specifies the maximum sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered for the purposes of clipping.

When present, segment_max_value shall be greater than or equal to segment_min_value.

segment_channel_clip_flag equal to 0 indicates that no clipping operation is applied to obtain the interpretation sample values of the decoded auxiliary picture. segment_channel_clip_flag equal to 1 indicates that the interpretation sample values of the decoded auxiliary picture are altered according to the clipping process described by the segment_channel_clip_type_flag syntax element. When not present, the value of segment_channel_clip_flag is inferred to be equal to 0.

segment_channel_clip_type_flag equal to 0 indicates that, for purposes of segment blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than ( segment_max_value – segment_min_value ) / 2 is set equal to segment_max_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less or equal than ( segment_max_value – segment_min_value ) / 2 is set equal to segment_min_value to obtain the interpretation sample value for the auxiliary picture luma sample. segment_channel_clip_type_flag equal to 1 indicates that, for purposes of segment blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than segment_max_value is set equal to segment_max_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less than or equal to segment_min_value is set equal to segment_min_value to obtain the interpretation sample value for the auxiliary picture luma sample.

segment_label_idc indicates the type of label used to identify the segment. The value of segment_label_idc shall be in the range of 0 to 3, inclusive.

Table F.14.3.13 Segment label indicated by segment_label_idc

| segment_label_idc | Segment Label |
| --- | --- |
| 0 | SEGMENT_ID_FULL |
| 1 | SEGMENT_ID_SPARSE |
| 2-3 | Reserved |

segment_values_cnt specifies the number of possible values that will be identified with an explicit ID in subsequent syntax.

segment_value[ i ] indicates the value for which an explicit ID is to be identified. segment_value[ i ], when present, shall be in the range of 0 to 216 -1, inclusive.

segment_type_id[ i ] indicates the id of the i-th segment. segment_type_id[ i ] shall be in the range of 0 to 216 -1, inclusive.

When both alpha_channel_info() and picture_segmentation_info() are present for a picture, the value of alpha_channel_use_idc should be equal to 2 and the information in the picture segmentation information SEI message should take precedence over the information in the alpha channel information SEI message.

# Conclusion

This contribution presents four options to annotate alpha channel values for the purpose of the indication of alpha based segments. The proponents have a preference towards options (i) and (iii) since these do not collude with the existing usage of alpha while providing added functionality, such as enabling ID tagging of each segment.

# References

- Alpha matting in computer vision, https://www.linkedin.com/pulse/alpha-matting-computervision-hemanth-chakravarthy-mudduluru-r2hze/

- Matte (filmmaking), https://en.wikipedia.org/wiki/Matte_(filmmaking)

- Marion Gerlinger, “The Alpha Channel”, Linearity blog, https://www.linearity.io/blog/alpha-channel/, 13 June 2022

- DigitalRebellion, https://www.digitalrebellion.com/blog/posts/list_of_video_formats_supporting_alpha_channels

- ISO/IEC 23008-2 and ITU-T H.265  Edition 7, published November 2019.

# Patent rights declaration(s)

Apple Inc,. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).