| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0274 |
| --- | --- |

| Title: | EE2 Test 3.12: Combined test of Test 3.6 and Test 3.11 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang, Kai Zhang, Na Zhang, Zhipin Deng, Wenbin Yin, Lei Zhao, Mehdi Salehifar, Li Zhang (Bytedance) Ke Jia, Jie Chen, Xinwei Li, Ru-Ling Liao, Yan Ye (Alibaba) | Tel: Email: | {wangyang.cs, zhangkai.video, lizhang.idm}@bytedance.com {feijia.jk, jiechen.cj, sid.lxw, ruling.lrl, yan.ye}@alibaba-inc.com |
| Source: | Bytedance Inc., Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-3.12, which are the combined test of EE2-3.6 on GPM-affine harmonized with GPM-TM and EE2-3.11 on template matching (TM) and merge with motion vector difference (MMVD) extensions to regression-based GPM. Three tests are conducted in EE2-3.12, and the simulation results on top of ECM-14.0 are reported as below:

Test 3.12a (Test 3.6 + Test 3.11a)

RA: {-0.10%, -0.19%, -0.19%; 101.6%, 100.0%};

LDB: {-0.22%, -0.19%, -0.16%; 101.9%, 99.9%}.

Test 3.12a* (Test 3.6 + Test 3.11a + encoder optimization)

RA: {-0.08%, -0.19%, -0.18%; 100.9%, 100.2%};

LDB: {-0.22%, -0.19%, -0.16%; 101.9%, 99.9%}.

Test 3.12b (Test 3.6 + Test 3.11b)

RA: {-0.11%, -0.18%, -0.15%; 101.9%, 100.0%};

LDB: {-0.22%, 0.17%, -0.16%; 105.0%, 99.9%}.

Introduction

In EE2-3.6 [1], GPM-affine is proposed to be harmonized with GPM-TM (GPM-affine-TM). An affine candidate could be refined by template matching (TM) using the same method as affine mode. A GPM sub-partition can be predicted by GPM-affine-TM or GPM-TM.

EE2-3.11 [2] studies template matching (TM) and merge with motion vector difference (MMVD) extensions to regression-based GPM. In Test 3.11a, only TM extension is applied. When regression-based GPM is used for the current block, a TM flag is signaled to indicate whether the motion vector is to be refined by TM. If the TM flag is true, the motion vectors of both partitions are refined. The template used in the TM process consists of above and left reconstructed samples (AL shape). The TM search process is the same as that of regular GPM-TM. In Test 3.11b, both TM and MMVD extensions are applied. The TM extension is same as Test 3.11a. When TM flag is true, the motion vectors of both partitions are refined by TM. When TM flag is false, an MMVD flag is signaled to indicate whether the motion vectors of partitions are refined by the MVD. If the MMVD flag is true, an MVD index is signaled to indicate the MVD. In order to reduce the encoder complexity of MMVD extension, the number of MVD candidates is reduced to 40.

Three combined tests are conducted in EE2-3.12, of which test 3.12a is the combination of test 3.6 and test 3.11a, test 3.12b is the combiniation of test 3.6 and 3.11b. Additionally, to further reduce the encoder complexity, test 3.12a* is provided on top of 3.12a by adding an encoder fast algorithm. The normative part of test 3.12a* and test 3.12a is the same. The three tests are listed as follow.

- Test 3.12a: Test 3.6 + Test 3.11a

- Test 3.12a*: Test 3.6 + Test 3.11a + encoder optimization

- Test 3.12b: Test 3.6 + Test 3.11b

Simulation results

The three tests were implemented on top of ECM-14.0 [3]. Simulations were conducted following the common test conditions of EE2 [4]. The simulation results compared to ECM-14.0 are shown in the following tables.

Table 1: Test results of Test 3.12a

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.16% | -0.30% | 101.5% | 100.2% |  |  |  |  |  |
| Class A2 | -0.09% | 0.01% | -0.18% | 101.4% | 99.9% |  |  |  |  |  |
| Class B | -0.12% | -0.33% | -0.16% | 101.3% | 100.1% | -0.11% | -0.02% | -0.12% | 101.3% | 99.8% |
| Class C | -0.11% | -0.20% | -0.17% | 102.2% | 99.9% | -0.27% | -0.17% | -0.19% | 101.8% | 101.1% |
| Class E |  |  |  |  |  | -0.34% | -0.49% | -0.18% | 103.0% | 98.6% |
| Overall | -0.10% | -0.19% | -0.19% | 101.6% | 100.0% | -0.22% | -0.19% | -0.16% | 101.9% | 99.9% |
| Class D | -0.06% | 0.05% | -0.15% | 102.7% | 99.9% | -0.11% | -0.02% | -0.12% | 101.3% | 99.8% |
| Class F | -0.06% | -0.03% | -0.03% | 101.3% | 101.0% | -0.27% | -0.17% | -0.19% | 101.8% | 101.1% |

Table 2: Test results of Test 3.12a*

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.52% | -0.36% | 101.3% | 100.5% |  |  |  |  |  |
| Class A2 | -0.06% | 0.08% | -0.23% | 101.2% | 99.9% |  |  |  |  |  |
| Class B | -0.11% | -0.15% | -0.12% | 100.9% | 100.4% | -0.11% | -0.02% | -0.12% | 101.3% | 99.8% |
| Class C | -0.09% | -0.18% | -0.09% | 100.4% | 99.9% | -0.27% | -0.17% | -0.19% | 101.8% | 101.1% |
| Class E |  |  |  |  |  | -0.34% | -0.49% | -0.18% | 103.0% | 98.6% |
| Overall | -0.08% | -0.19% | -0.18% | 100.9% | 100.2% | -0.22% | -0.19% | -0.16% | 101.9% | 99.9% |
| Class D | -0.07% | -0.06% | -0.07% | 101.9% | 99.7% | -0.11% | -0.02% | -0.12% | 101.3% | 99.8% |
| Class F | -0.04% | 0.01% | -0.06% | 101.0% | 100.3% | -0.27% | -0.17% | -0.19% | 101.8% | 101.1% |

Table 3: Test results of Test 3.12b

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.26% | -0.25% | 101.8% | 99.9% |  |  |  |  |  |
| Class A2 | -0.08% | -0.01% | -0.24% | 102.1% | 99.7% |  |  |  |  |  |
| Class B | -0.15% | -0.24% | -0.15% | 102.4% | 100.3% | -0.11% | 0.11% | -0.27% | 103.5% | 99.7% |
| Class C | -0.11% | -0.17% | -0.02% | 101.1% | 100.1% | -0.29% | -0.02% | -0.14% | 105.6% | 99.7% |
| Class E |  |  |  |  |  | -0.32% | 0.51% | 0.00% | 106.7% | 100.3% |
| Overall | -0.11% | -0.18% | -0.15% | 101.9% | 100.0% | -0.22% | 0.17% | -0.16% | 105.0% | 99.9% |
| Class D | -0.11% | 0.10% | -0.20% | 102.4% | 100.2% | -0.16% | 0.78% | -0.53% | 102.9% | 97.9% |
| Class F | -0.04% | -0.06% | 0.00% | 101.0% | 100.9% |  |  |  |  |  |

Conclusions

In this contribution, the test results of EE2-3.12a, EE2-3.12a* and EE2-3.12b are reported. It is recommended to adopt one of the combinations into ECM.

Reference

- Y. Wang, K. Zhang, N. Zhang, Z. Deng, W. Yin, L. Zhao, M. Salehifar, and L. Zhang, “EE2-3.6: GPM-affine with TM,” Document of Joint Video Experts Team, JVET-AJ0095, Kemer, TR, 1–8 November 2024.

- K. Jia, J. Chen, X. Li, R.-L. Liao, and Y. Ye, “EE2-3.11: Regression-based GPM extension,” Document of Joint Video Experts Team, JVET-AJ0115, Kemer, TR, 1–8 November 2024.

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0.

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AI2017, Sapporo, JP, 12–19 July 2024.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).