| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0276-v1 |
| --- | --- |

| Title: | Non-EE2: Improvement to ALF scaling factor |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Nan Hu, Hongtao Wang, Yiting Shao, Vadim Seregin 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | martak@qti.qualcomm.com nanh@qti.qualcomm.com hongtaow@qti.qualcomm.com yitishao@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM-14.0, scaling factors are signalled and applied to the correction residuals of ALF signaled filters. A scaling factor is signalled for each ALF class. In this proposal, for each ALF class, two scaling factors are signalled. One is applied to boundary samples and the other one is applied to non-boundary samples. On top of ECM-14.0 following common test conditions, simulation results of the proposed method are reported below:

AI: %, %, %, % EncT, % DecT.

RA: %, %, %, % EncT, % DecT.

LB: %, %, %, % EncT, % DecT.

# Introduction

In ECM-14.0, for each ALF class of signalled filters, a scaling factor is signalled and applied to the correction residuals.

# Proposed method

In this proposal, boundary information is derived from the deblocking filter. If a 2x2 block is located at a CU or TU boundary, then the samples of the block are classified as boundary samples. Otherwise, the samples of the block are classified as non-boundary samples. Then, two scaling factors are signalled for each ALF class of signalled filters in the same way as in ECM-14.0. One scaling factor is for the boundary samples and the other one is applied to non-boundary samples. The same candidates scaling factor values are used as in ECM-14.0.

# Simulation results

The proposed method is implemented on top of ECM-14.0. The simulations were performed under common test conditions defined in JVET-AI2017. The simulation results are reported in the table below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% | 99.7% | 99.8% | 100.3% | 100.4% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 99.9% | 100.0% | 100.3% | 100.2% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.11% | -0.02% | 0.06% | 99.7% | 99.4% | 100.2% | 100.5% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.06% | 0.20% | 0.00% | 100.5% | 100.2% | 100.1% | 100.2% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.07% | 0.76% | 0.67% | 100.1% | 100.2% | 100.2% | 99.2% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).