| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0277 |
| --- | --- |

| Title: | EE2 Test 2.22: A combination test of Test 2.19c and Test 3.10a |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang, Kai Zhang, Wenbin Yin, Zhipin Deng, Na Zhang, Lei Zhao, Mehdi Salehifar, Li Zhang (Bytedance) Lai Zhang, Zhihuang Xie, Yue Yu, Haoping Yu, Dong Wang (OPPO) | Tel: Email: | {wangyang.cs, zhangkai.video, lizhang.idm}@bytedance.com {zhanglai, xiezhihuang}@oppo.com |
| Source: | Bytedance Inc., OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results for EE2 Test 2.22, which is a combination test of EE2 Test 2.19c on multi-model for EIP and block-vector guided EIP, and EE2 Test 3.10a on SATD-based reordering for intra coding. On top of ECM-14.0, the simulation results are reported as below:

Test 2.22 (Test 2.19c + Test 3.10a):

AI: {-0.12%, -0.11%, -0.10%; 101.0%%, 100.8%};

RA: {-0.05%, -0.10%, -0.15%; 99.9%, 99.8%}.

Introduction

EE2 Test 2.19c [1] tests a combination of Test 2.19a and Test 2.19b. In Test 2.19a, it is proposed that using the multi-model extrapolation filter-based intra prediction (EIP) method derives two extrapolation filters and applies them to an EIP-coded CU. In Test 2.19b, it is proposed that a block vector is used to determine a reference area instead of directly using the adjacent spatial reference area for calculating the EIP filter parameters.

In EE2 Test 3.10a [2], SATD-based reordering is used for intra and intra block copy (IBC) coding tools, such as reordering in template-based multiple reference line intra prediction (TMRL) and EIP, and adaptive reordering of merge candidates (ARMC) for IBC.

Test 2.22 is a combination of Test 2.19c and Test 3.10a.

Simulation results

EE2 Test 2.22 was implemented on top of ECM-14.0 [3]. Simulations were performed following the common test conditions of EE2 [4]. The simulation results compared to ECM-14.0 are shown in the following table.

Table 1: Test results of Test 2.22

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.20% | -0.05% | 101.2% | 101.3% | -0.03% | -0.17% | -0.20% | 100.1% | 99.2% |
| Class A2 | -0.12% | -0.17% | -0.17% | 100.9% | 100.5% | -0.08% | -0.03% | -0.07% | 100.4% | 100.0% |
| Class B | -0.09% | -0.07% | -0.13% | 101.2% | 99.8% | -0.05% | -0.02% | -0.23% | 98.7% | 100.0% |
| Class C | -0.06% | -0.15% | -0.15% | 100.7% | 102.1% | -0.05% | -0.19% | -0.08% | 100.8% | 99.9% |
| Class E | -0.20% | -0.01% | 0.04% | 101.1% | 100.8% |  |  |  |  |  |
| Overall | -0.12% | -0.11% | -0.10% | 101.0% | 100.8% | -0.05% | -0.10% | -0.15% | 99.9% | 99.8% |
| Class D | -0.02% | 0.16% | 0.04% | 101.6% | 101.2% | 0.01% | -0.19% | -0.66% | 101.3% | 100.0% |
| Class F | -0.25% | -0.36% | -0.21% | 101.4% | 101.8% | -0.06% | -0.29% | -0.17% | 100.0% | 100.0% |

Conclusions

In this contribution, the test results of EE2 Test 2.22 are reported. It is recommended to adopt Test 2.22 into ECM.

Reference

- L. Zhang, Z. Xie, Y. Yu, H. Yu, and D. Wang, “EE2-2.19c: a combination of EE2.19a and EE2.19b,” Document of Joint Video Experts Team, JVET-AJ0084, Kemer, TR, 1–8 November 2024.

- Y. Wang, K. Zhang, W. Yin, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, and L. Zhang, “EE2-3.10: SATD-based reordering,” Document of Joint Video Experts Team, JVET-AJ0096, Kemer, TR, 1–8 November 2024.

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0.

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AI2017, Sapporo, JP, 12–19 July 2024.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).