| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0289-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0081 (EE2-2.18: Chroma TMRL) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xinwei Li | Tel: Email: | sid.lxw@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of JVET-AJ0081 (EE2-2.18), where TMRL (template-based multiple reference line intra prediction) is extended to chroma as a new chroma prediction mode. It is confirmed that the implementation matches the description of JVET-AJ0081 and the partial results match those provided by the proponent.

Introduction

In JVET-AJ0081 (EE2-2.18), a chroma TMRL mode is proposed. The cross-checker conducted simulations using the provided software on top of ECM-14.0 according to CTC [1] for enhanced compression tool testing with all intra and random access configurations. It is confirmed that the implementation matches the description of JVET-AJ0081 and the partial results match those provided by the proponent.

Table 1: Coding performance of EE2-2.18

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.18% | -0.01% | 101.0% | 100.6% | 99.8% | 100.0% |
| Class A2 | -0.01% | -0.23% | -0.23% | 101.1% | 99.5% | 100.0% | 100.1% |
| Class B | 0.00% | -0.21% | -0.27% | 100.5% | 100.1% | 99.7% | 100.0% |
| Class C | 0.02% | -0.30% | -0.40% | 101.1% | 100.8% | 100.6% | 100.0% |
| Class E | 0.00% | -0.65% | -0.40% | 100.6% | 100.7% | 102.3% | 100.0% |
| Overall | 0.00% | -0.30% | -0.27% | 100.8% | 100.3% | 100.4% | 100.0% |
| Class D | 0.02% | 0.04% | -0.45% | 101.1% | 100.5% | 100.1% | 100.0% |
| Class F | -0.03% | -0.07% | -0.07% | 100.8% | 100.5% | 100.0% | 100.1% |
| Class TGM | 0.00% | -0.10% | -0.12% | 101.3% | 100.3% | 100.1% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | 0.00% | -0.30% | -0.01% | 100.4% | 100.5% | 99.8% | 99.8% |
| Class C | 0.00% | -0.16% | -0.15% | 100.0% | 100.3% | 100.0% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | 0.05% | -0.34% | -0.31% | 100.2% | 100.8% | 99.9% | 100.0% |
| Class F | 0.10% | -0.08% | -0.20% | 100.1% | 101.7% | 99.9% | 100.0% |
| Class TGM | -0.07% | 0.00% | -0.05% | 100.3% | 101.6% | 100.2% | 100.1% |

References

- H. Huang, Y. Yu, H. Yu and D. Wang, “EE2-2.18: Chroma TMRL,” JVET-AJ0081, Oct. 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, Aug. 2024.

Patent rights declaration(s)

Alibaba group does not have any current or pending patent rights relating to the technology described in this contribution.