| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0294-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0078 (EE2-related: Extended overlapped block blending for MV/BV based prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xinwei Li | Tel: Email: | sid.lxw@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of JVET-AJ0078, which an extended overlapped block blending (OBB) for motion vector (MV) or block vector (BV) based prediction such as inter prediction, IBC, and IntraTMP. It is confirmed that the implementation matches the description of JVET-AJ0078 and the partial results match those provided by the proponent.

Introduction

In JVET-AJ0078, OBB is introduced to IBC or IntraTMP applied current blocks regardless of prediction mode in neighboring blocks. Moreover, JVET-AJ0078 additionally allows OBB for inter predicted current blocks having neighboring blocks with BV. The cross-checker conducted simulations using the provided software on top of ECM-14.0 according to CTC [1] for enhanced compression tool testing with all intra and random access configurations. It is confirmed that the implementation matches the description of JVET-AJ0078 and the partial results match those provided by the proponent.

Table 1: Coding performance of JVET-AJ0078

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.02% | -0.05% | 101.0% | 100.2% | 99.4% | 100.2% |
| Class A2 | -0.06% | -0.17% | -0.12% | 100.9% | 100.8% | 100.0% | 100.1% |
| Class B | -0.04% | -0.14% | -0.05% | 101.4% | 100.3% | 100.0% | 100.1% |
| Class C | -0.03% | -0.09% | -0.08% | 101.0% | 101.5% | 99.0% | 100.2% |
| Class E | -0.11% | -0.03% | 0.08% | 100.9% | 100.3% | 101.9% | 100.2% |
| Overall | -0.05% | -0.09% | -0.05% | 101.1% | 100.6% | 100.0% | 100.2% |
| Class D | 0.00% | 0.02% | -0.03% | 101.2% | 99.7% | 100.2% | 100.2% |
| Class F | -0.01% | -0.05% | -0.09% | 100.9% | 101.8% | 99.3% | 100.2% |
| Class TGM | 0.01% | -0.02% | -0.01% | 100.9% | 100.8% | 100.0% | 100.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0.07% | -0.30% | -0.01% | 99.8% | 100.1% | 99.6% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.06% | 0.02% | 0.04% | 99.6% | 99.9% | 100.0% | 99.9% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

References

- Y. Kidani, H. Kato, K. Kawamura (KDDI), “EE2-related: Extended overlapped block blending for MV/BV based prediction,” JVET-AJ0078, Oct. 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, Aug. 2024.

Patent rights declaration(s)

Alibaba group does not have any current or pending patent rights relating to the technology described in this contribution.