| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0300-v2 |
| --- | --- |

| Title: | AHG15: Reading the Auxiliary Information for gaming sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | S. Thiebaud, S. Puri, T. Poirier, R. Utida, F. Galpin | Email: sylvain.thiebaud@interdigital.com Saurabh.puri@interdigital.com Tangi.poirier@interdigital.com Renan.utida@interdigital.com Franck.galpin@interdigital.com |  |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

Auxiliary Information associated with gaming sequences of Classes G1 and G3 provide two kinds of information: auxiliary meta data (e.g. camera parameters) and auxiliary video data (e.g. a depth map or a motion map). Camera parameters define the characteristics and the position of the virtual camera of the game engine. Depth and motion maps provide the depth and the motion per pixel in floating point, for each frame, with the same resolution as the gaming content.

Following the JVET contribution JVET-AI0054, it has been decided to establish an AHG software branch in VTM to read this auxiliary information.

This contribution presents the way the camera parameters, depth maps and motion maps are available per frame, at the encoder side, in the dedicated branch of the VTM repository.

- Introduction and context

JVET contributions JVET-AH0179 [1] and JVET-AH0147 [2] proposed to provide some auxiliary information provided by the gaming engine, such as camera parameters, depth maps and motion vectors for gaming sequences of class G1 and G3. As described in these contributions, camera parameters define the characteristics and the position of the virtual camera of the game engine. They must be provided per frame, synchronized with the video sequence.

The format defining these camera parameters has been specified in JVET-AI2027 [3] “Draft Common Test Conditions (CTC) for gaming applications”. These camera parameters are provided in an ascii file, each line providing the parameters associated with a video frame, as a JSON object.

JVET contribution JVET-AI0054 [4] proposed a sample of C++ code to read these parameters, to avoid adding a new dependency to an external JSON library. Following this contribution, it has been decided to establish an AHG software branch in VTM to implement this software dedicated to gaming content. The piece of code reading the camera parameters has been integrated in this branch. The way these parameters are now available per frame in VTM is described in Subsection 2.1.

JVET-AI2027 [3] also specifies the format of the depth and motions maps provided as auxiliary video information. These maps are packed in a single 32 bits float EXR file, per frame. This contribution provides information on the software added to the VTM AHG gaming branch at JVET AHG Gaming content compression / VVCSoftware_VTM · GitLab to read the auxiliary video data per frame and make it available on the encoder side. A new encoder parameter is added in the configuration file as described below in sub-section 2.2.

# Accessing the Auxiliary Information in VTM

## Accessing the Camera Parameters as meta data

Currently four elements are mandatory in the camera parameters file:

- The Projection matrix “projectionMatrix”,

- the World to Camera Matrix “worldToCameraMatrix”,

- the Near clipping plane “nearClipPlane”

- the Far clipping plane “farClipPlane”.

Since the parameters must be synchronized with the gaming content, the element “frame” is also mandatory to indicate the corresponding frame index of these parameters.

JVET-AI2027 [3] describes these parameters and introduces their meaning in the game engine.

A new parameter “CameraParamFile” has been introduced in the encoder configuration file.

Example:

CameraParamFile : C:/GamingContent/DarkTree-HD-Float32/cameraParam.txt

It specifies the location of the camera parameters file associated with the gaming content on the test system. This new parameter is optional and can be omitted. When specified, the four mandatory camera parameters are available in VTM as new fields in the picture structure (JVET_AI0054 defined).

struct Picture : public UnitArea

{

…

#if JVET_AI0054

float m_projMatrixCoeff[16];

float m_worldMatrixCoeff[16];

float m_near, m_far;

#endif

…

}

Camera parameters as presented in JVET-AI0054 [4] available in the Picture structure

## Accessing the Depth and Motion as auxiliary video data

As indicated above, the depth and motion information are provided in 32 bits EXR files, one file per frame.

When these 3 maps are provided as a unique EXR, the depth is coded in the red channel, the horizontal component of the motion vector in the green channel and the vertical component in the blue channel.

JVET-AI2027 [3] specifies the naming convention of these files as “{SeqName}_motion_depth_frame{X}.exr”, where {SeqName} represents the name of the sequence and {X} the frame number to which the auxiliary data corresponds.

A new parameter “ExrDMVFile” has been introduced in the encoder configuration file .

Exemple:

ExrDMVFile : C:/GamingContent/DarkTree-HD-Float32/Darktree_motion_depth_frame0.exr

It specifies the location of the EXR depth and motion file associated with the gaming content on the test system. This new parameter is optional and can be omitted. When specified, the three maps corresponding to the depth and the horizontal and vertical components of the motion are available in VTM as new fields in the picture structure (DMV_FROM_EXR defined, as well as JVET_AI0054).

struct Picture : public UnitArea

{

…

#if DMV_FROM_EXR

image::ImageRGB<float> 	m_dMVframe;

image::Image<float>& 	m_depth = m_dMVframe.R;

image::Image<float>& 	m_mvx = m_dMVframe.G;

image::Image<float>& 	m_mvy = m_dMVframe.B;

#endif

…

}

Depth and motion maps available in the Picture structure

Note that the depth and motion available here in the Picture structure represent the raw information provided by the game engine.

For instance, m_depth does not represent the real depth of the game scene, but the non-linear depth as used by the game engine. The relationship between this information and the real depth depends on the way the game engine represents the depth, as documented in JVET-AI2027 [3].

In the same way, the two motion components m_mvx and m_mvy can be interpreted differently depending on the game engine. The relationship between these motion components and a motion vector used in VTM is also documented in JVET-AI2027 [3].

# Conclusions

This contribution is to inform that the software to read the auxiliary video data in addition to the auxiliary meta data has been added to the AHG15 software branch and to describe how to access the auxiliary video data on the encoder side using a new configuration parameter.

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.

# References.

[1]	JVET-AH0179, AHG15: On auxiliary information for the Unity Gaming sequences proposed by InterDigital, S. Thiebaud, S. Puri, T. Poirier, F. Galpin. 34th Meeting, Rennes FR, 17–24 April 2024.

[2] 	JVET-AH0147, AHG15: Description of the auxiliary information for the gaming content sequences proposed by Huawei, W. Chen, Z. Lin, Yuxin Zhao, K. Cai, J. Sauer, E. Alshina, Yin Zhao, 34th Meeting, Rennes FR, 17–24 April 2024.

[3] 	JVET-AI2027, Draft Common Test Conditions (CTC) for gaming applications, J. Sauer, Huawei, R. Chernyak, Tencent, S. Puri, InterDigital, S. Thiebaud, InterDigital. 35th Meeting, Sapporo, JP, 12–19 July 2024.

[4] JVET-AI0054, AHG15: Reading the camera parameters for gaming sequences of class G1 proposed by InterDigital, S. Thiebaud, S. Puri, T. Poirier, F. Galpin, 35th Meeting, Sapporo, JP, 12–19 July 2024.