| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0305-v1 |
| --- | --- |

| Title: | AHG4/AHG12: Test Sequences Featuring User-Generated Contents |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Yang Wang1, Wenbin Ying1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | zhipin.deng@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents five vertical videos sequences (videos in portrait mode) featuring user-generated contents. The coding results of ECM-14.0 on these proposed sequences over VTM-11.0ecm14.0 for {Y, U, V} are reported as below:

|  | AI |  |  | RA |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Seq #1 | -22.62% | -17.31% | -15.97% | -33.03% | -27.01% | -27.00% |  |  |  |
| Seq #2 | -14.87% | -7.37% | -8.61% | -29.40% | -22.49% | -22.38% |  |  |  |
| Seq #3 | -14.40% | -7.11% | -7.17% | -28.29% | -21.17% | -21.10% | -28.38% | -26.41% | -26.29% |
| Seq #4 | -19.58% | -16.93% | -17.64% | -31.23% | -29.67% | -29.89% |  |  |  |
| Seq #5 | -15.37% | -6.99% | -6.59% | -28.40% | -20.60% | -20.80% |  |  |  |
| Avg. | -16.0% | -8.5% | -8.5% | -30.0% | -22.1% | -22.1% |  |  |  |

# Introduction

Vast amounts of User-Generated Content (UGC) videos are processed daily on popular platforms like YouTube, Instagram, TikTok, and Kwai. Research shows that UGC is 8.7 times more effective than influencer content and 6.6 times more influential than branded content, highlighting its growing popularity over Professionally Generated Content (PGC) [1]. Effective compression of UGC videos is essential for reducing bandwidth, storage requirements, and delivery costs, while also enhancing the user experience. Meanwhile, advancements in UGC compression techniques enable platforms to preserve high video quality despite significant compression ratio — crucial for maintaining engagement in highly visual, short-form content. Therefore, we propose the inclusion of UGC videos in common test conditions to encourage broader expertise and further innovation in UGC compression.

Compared to PGC videos, UGC videos have some distinct characteristics, such as:

- Orientation: UGC videos are often captured vertically (portrait mode), while PGC videos are usually filmed horizontally (landscape mode).

- Equipment Quality: UGC videos are usually captured with a variety of unprofessional equipment, which may generate video content with undesirable artifacts such as jitter, noise, or blur.

- Content Diversity: UGC videos are captured by billions of unprofessional users in their daily life. UGC videos involve extremely rich content from diversified users all over the world, who may produce undesirable effects such as jitter, improper lighting (too bright or too dark), abrupt scenes change or out-of-focus region.

# Proposed Sequences

In this contribution, we present five vertical UGC video sequences (videos in portrait mode) as test sequences for ECM. Table 1 summarizes the information of test sequences. All these sequences are 720×1280@30Hz with 8-bit. Table 2 depicts the thumbnails of the proposed UGC sequences.

Table 1. Proposed sequences information

| Test sequence | Resolution | fps | Bit-depth | Frames |
| --- | --- | --- | --- | --- |
| Seq #1: Escalator | 720×1280 | 30 | 8 bits | 300 |
| Seq #2: SwimmingPool | 720×1280 | 30 | 8 bits | 300 |
| Seq #3: BusStation | 720×1280 | 30 | 8 bits | 300 |
| Seq #4: Park | 720×1280 | 30 | 8 bits | 300 |
| Seq #5: Store | 720×1280 | 30 | 8 bits | 300 |

Table 2. Thumbnail of the proposed UGC sequences

| Escalator | SwimmingPool | BusStation | Park | Store |
| --- | --- | --- | --- | --- |

# Simulation Results

The proposed sequences are coded with VTM-11.0ecm14.0 and ECM-14.0 [2] under CTC [3] .

The simulation results of ECM-14.0 compared with VTM-11.0ecm14.0 on the five sequences are summarized in Table 3.  Figure 1 illustrates the RD-curves of the five sequences under RA configuration. More detailed results could be found in the attached excel sheets.

Table 3: Coding performance of ECM14.0 compared with VTM-11.0ecm14.0 on UGC sequences

|  | AI |  |  | RA |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Seq #1 | -22.62% | -17.31% | -15.97% | -33.03% | -27.01% | -27.00% |  |  |  |
| Seq #2 | -14.87% | -7.37% | -8.61% | -29.40% | -22.49% | -22.38% |  |  |  |
| Seq #3 | -14.40% | -7.11% | -7.17% | -28.29% | -21.17% | -21.10% | -28.38% | -26.41% | -26.29% |
| Seq #4 | -19.58% | -16.93% | -17.64% | -31.23% | -29.67% | -29.89% |  |  |  |
| Seq #5 | -15.37% | -6.99% | -6.59% | -28.40% | -20.60% | -20.80% |  |  |  |
| Avg. | -16.0% | -8.5% | -8.5% | -30.0% | -22.1% | -22.1% |  |  |  |

Figure 1. RD-curves of the five UGC sequences under RA configuration.

# Conclusions

In this contribution, five UGC vertical video sequences are proposed. It is recommended to add an additional sequence class “class UGC” into the CTC of ECM, and include the proposed five sequences in the class UGC.

# Reference

- https://www.socialmediatoday.com/news/survey-reveals-that-ugc-can-drive-improved-trust-and-loyalty-for-ecommerce/606801/

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AI2017, JP, July 2024.

# Patent Rights Declaration(s)

Bytedance Inc. does not have any current or pending patent rights relating to the technology described in this contribution.